IEEE P802.11
Wireless LANs

| LB189 D2.0 comment resolution (PHY – Guard interval modes) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-09-04 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Tevfik Yucek | Qualcomm | 3105 Kifer Road, Santa Clara, CA |  | tyucek@qca.qualcomm.com |

Abstract

This document contains proposed resolution of some of the comments categorized as guard interval modes in LB189 of P802.11af D2.0. Proposed resolutions are based on 802.11af draft text D2.0.

This submission provides resolution to comments 2, 222, 581, 589, 590, 375,.

- Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGaf Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGaf Draft (i.e. they are instructions to the 802.11 editor on how to merge the text with the baseline documents).

TGaf Editor:  Editing instructions preceded by “TGaf Editor” are instructions to the TGaf editor to modify existing material in the TGaf draft.   As a result of adopting the changes, the TGaf editor will execute the instructions rather than copy them to the TGaf Draft.

Submission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# Guard interval modes related comments

| CID | Page | Clause | Comment | Proposed Change |
| --- | --- | --- | --- | --- |
| 2 | 229 | 23.1.1 | If we want to be consistant with 11ac, then only normal preamble is mandatory and SGI is optional; if this sub-bullet does intend to restrict that SGI is mandatory, then a more accurate description is "Normal and short guard interval " | either change to "Normal guard interval", or change to "Normal and short guard interval", depending on the true intention |
| 581 | 229 | 23.1.1 | Why long guard interval is not mandory? (In table 23-3, it is shown that GI = T_DFT/4, which is the same definition given in 802.11ac, table 22-5 for long guard interval) | Typo in the text? Short guard interval need to be considered as a mandatory. |
| 589 | 233 | 23.3.3 | There is inconsistence: if the L-STF field follows the precedure in 22.3.4.2, then (f) Insert GI and apply windowing: prepend a GI(LONG_GI) and apply window as described in 22.3.7. But page 239 line 5 shows that short GI is mandory for TVHT. | If LONG GI is mandory, then there is no modification required here. |
| 590 | 233 | 23.4.3 | similar comment to previous one: There is inconsistence: if the L-LTF field follows the precedure in 22.3.4.3, then (f) Insert GI and apply windowing: prepend a GI(LONG_GI) and apply window as described in 22.3.7. But page 239 line 5 shows that short GI is mandory for TVHT. | If LONG GI is mandory, then there is no modification required here. |
| 375 | 229 | 23.1.1 | Long GI can be very useful in long delay spread situations. | Make support of Long GI mandatory. |
| 222 | 229 | 23.1.1 |  | Add mandatory support of Long GI for increased range. |

Propose: Accept CID 2, 222, 581, 589, 590, and 375.

The intention of PHY submission was to make short GI a mandatory feature in addition to normal guard interval. Hence, in TGaf capable devices shall support both guard interval modes (short and normal). In draft 2.0, making short guard interval mandatory is emphasized as this was a change from 11ac PHY, which caused the confusion.

SGI is made mandatory since the bits that indicate support for SGI in VHT capabilities are used to indicate support for different TVHT modes (see Section 8.4.2.160.2a) in TVHT.

# Editing instructions

23.1.1 Introduction to the TVHT PHY

….

A TVHT STA shall support:

—TVHT_MODE_1 (One frequency segment)

—Single spatial stream MCSs 0 to 7 (transmit and receive)

—Binary convolutional coding

—Normal and short guard interval (transmit and receive)

…