IEEE P802.11
Wireless LANs

| CC9 cluase 9.17b comment resolution |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-07-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Yongho Seok | LG Electronics |  | +82-31-450-1947 | yongho.seok@lge.com |

Abstract

This submission proposes comment resolutions of the following CIDs from TGah Draft 0.1 Comment Collection 9.

- 414, 965, 964, 269, 907, 77, 120, 833, 78

Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGah Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGah Draft (i.e. they are instructions to the 802.11 editor on how to merge the text with the baseline documents).

TGah Editor: Editing instructions preceded by “TGah Editor” are instructions to the TGah editor to modify existing material in the TGah draft.  As a result of adopting the changes, the TGah editor will execute the instructions rather than copy them to the TGah Draft.

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 414 | 126.00 | 9.17b | Typo | Add "." at the end of the sentence (after 9 bits) | Accepted – TGah editor to make the changes proposed by the commenter. |
| 965 | 127.00 | 9.17b | In R.4.6.C [SFD r14], item 10. Use the NDP CTS frame with the Address Indicator field set to 0 and the RA field set to Partial BSSID for sector training. [13/0302r1] a. The NDP CTSs with the Address Indicator set to 0 and RA set to Partial BSSID will be transmitted through the Sectorized Beams (with assending Sector ID starting with Sector ID=0) during the sector training. Suggest not to assign to STA with AID that conflicts with PBSSID bits | Insert the text "An AP should not assign an AID to a S1G STA that results in the PARTIAL_AID value equals to its partial BSSID, if sectorization type I is supported" | Rejected – Lines 54-58 of Page 127 is already saying that AP should not assing an AID to a S1G STA that results in the PARTIAL_AID value equals to its partial BSSID. |
| 964 | 127.00 | 9.17b | Typo | Add ")" behind the text of "BSSID[40:43]" | Accepted – TGah editor to make the changes proposed by the commenter. |
| 269 | 128.00 | 9.17b | Shown example value "165" may be wrong. If this is right, the Table 9-19b explanation is hard to understand. Hence, detail procedure to calculate PAID should be written in this section | Maybe, correct value is not "165" but "161". | Rejected – Please see the below explanation. (It was referred from #4623 of IEEE 802.11ac Draft 3.0 comment resolution excel sheet.) The last two octets become 53-52 = 01010011 - 01010010 (hex) -> bit reverse: 11001010 : 01001010. Then BSSID[39:47] tells us to take the last 9 bits here: 001001010, which is (taking lsb at left) 164. So, (164 mod 511) + 1 is 165. |
| 907 | 127.00 | 9.17b | S1G AP cannot know all nearby overlapping BSS's BSSID. | Modify in the equation shown in line 58 from "Overlapping BSSID" to "Identified Overlapping BSSID". | Rejected – Comment is true. S1G AP can not detect all OBSSs. But, the equation also does not say that the partial AID shall not be conflicted with all Overlapping BSSIDs. Because the commenter's proposal ("identified OBSS") is more ambiguous, there is no reason to change current equation. |
| 77 | 127.00 | 9.17b | (9.8b) a parantesis is missing | put a ")" in the correct place | Revised – See the proposed resolution of CID 964. |
| 120 | 127.00 | 9.17b | How is the Partial_AID computed when a Relay forwards packets to a STA or to an AP? | Update Table 9-19b to reflect also the actions of a Relay | Rejected – A Relay is an entity that logically consists of an AP and a STA, as illustrated in clause 4.11a. So, when a Relay transmits a frame to either an AP or a STA , the partial AID is calculated according the role of the Relay. |
| 78 | 127.00 | 9.17b | () mod 2^9 should be replaced by () mod (2^9-1)+1 | as in the comment | Revised – TGah editor to make changes shown in 11-13-0774r0 under the heading for CID 833, 78 |

CID 78

Discussion:

Regarding CID 78, as shown in the IEEE 802.11ac draft 5.0, an AP should not assign an AID to a STA that results in the PARTIAL_AID value, as computed using Equation (9-8b), being equal to 0. The same rule shall be applied in a partial AID equation of S1G BSS.

Propose:

Revised for 78, per discussion and editing instructions in 11-13/0774r0.

TGah editor: Modify the sub-clause 9.17b as the following:

9.17b Group ID and partial AID in S1G PPDUs

…

An S1G AP should not assign an AID to a S1G STA that results in the PARTIAL_AID value, as computed using Equation (9-8b), being equal to either

0 or (dec(BSSID[39:47])mod(29-1))+1 or (dec(Overlapping BSSID[39:47])mod(29-1))+1.