IEEE P802.11
Wireless LANs

| Some LB 199 Proposed Comment Resolutions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-01-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dorothy Stanley | Aruba Networks | 1322 Crossman ave, Sunnyvale, CA | +1 408 227 4500 | dstanley@arubanetworks.com |

Abstract

Proposed resolutions to the following CIDs are included in this document:

2433, 2483 – resolution approved

2414 and 2005, 2405, 2406, 2427, 2002, 2226, 2246, 2392– resolutions approved 2013-12-20

2199 – assigned to Carlos Cordiero

2410– assigned to Mark Hamilton

2065 – to be done

2407 –  assigned to Brian Hart

2426 – Assign to Dan Harkins

2467, 2470, 2472, 2474, 2475, 2476, 2477, 2478, 2484, 2485

2466 – Assign to Carlos Cordiero

2471, 2481 – Needs submission, assign to Matthew Fischer

New in r4

Cid 2485

| 2485 | 2240.39 | B.4.3 |  |  | The subclause 13.1(Mesh STA dependencies) specifies that dot11MeshActivated shall be set to false when dot11DMGOptionImplemented is true. It means a DMG STA cannot be a mesh STA. Though, in PICS proforma, CF21 (Mesh station) does not exclude DMS STA. | Change the status column of CF21 to "(Not CF25):O.1". |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

The cited text is below:

Proposed resolution: Accepted

CID 2484

| 2484 | 2240.25 | B.4.3 |  |  | The condition of CF19 (WNM supported) includes CF15 (3.65-3.70 GHz band in United States) & DSE5 & DSE6 & DSE7 & DSE8 & DSE9, which inhibit WNM other than US 3.6GHz band. | Remove (CF15 & DSE5 & DSE6 & DSE7 & DSE8 & DSE9) from Status. |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

The cited text is below:

The condition in the status column is

“(CF8 & CF11 & CF13 & CF15 & DSE5 & DSE6 & DSE7 & DSE8 & DSE9 ):O”

CF8: Multidomain operation capability implemented

CF11: Operating classes capability implemented

CF13: Radio Measurement supported

CF15: 3.65–3.70 GHz band in United States

DSE5: STA association procedure Transmission of Association Request frame with Supported Operating Classes element by a STA Transmission of Association Response frame with Supported Operating Classes element by an AP

DSE6: Reassociation

DSE7: Probe Request with Supported Operating Classes Element

DSE8: Probe Response with Supported Operating Classes Element

DSE9: Extended channel switch procedure Transmission of extended channel switch announcement and channel switch procedure by an AP Transmission of extended channel switch announcement and channel switch procedure by a STA Reception of extended channel switch announcement and channel switch procedure by a STA

The status symbol is optional - When “(all of the conditions are true) then optional” allows use of WNM in non 3.6 bands.

Proposed resolution: Rejected

Then current text:

When “(all of the conditions are true) then optional” does allow use of WNM in non 3.6 bands.

CID 2478

| 2478 | 463.25 | 6.1.7.4.2 |  |  | Better wording is possible | Change "network" to "ESS" in several places in the description box in the table. Similar changes to other SAPs of the same family of SAPs. |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

The cited text is below:

The commenter’s suggested change is

From:

“An identifier for the network, composed of the string value of the SSID element used to identify the network”

to

“An identifier for the ESS network, composed of the string value of the SSID element used to identify the ESS network”

Similar changes at 464.31 and 465.46.

Proposed resolution: Accepted

At 463.25, change as shown below:

“An identifier for the network, composed of the string value of the SSID element used to identify the network”

to

“An identifier for the ESS network, composed of the string value of the SSID element used to identify the ESS network”

Similar changes at 464.31 and 465.46.

CID 2477

| 2477 | 463.21 | 6.1.7.4.2 |  |  | Better wording is possible - connected seems more appropriate since that is the state name. | Change "The MAC address of the non-AP STA that is reporting that an ESS has become available" to "The MAC address of the non-AP STA that is reporting that it has established a connection with an ESS." Look at other related SAPs as well. |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

The cited text is below:

And similarly:

The commenter proposes to change from

"The MAC address of the non-AP STA that is reporting that an ESS has become available"

To

"The MAC address of the non-AP STA that is reporting that it has established a connection with an ESS."

Proposed resolution: Revised

At 463.21

"The MAC address of the non-AP STA that is reporting that an ESS has become available"

To

"The MAC address of the non-AP STA that is reporting that it has established a connection with an ESS."

At 464.13 change from

“This event is triggered to indicate that an ESS is no longer available for sending frames.”

To

“This event is triggered to indicate that the connection to an ESS is no longer available for sending frames.”

At 464.28, change from

“The MAC address of the non-AP STA that is reporting that an ESS is no longer available.”

To

“The MAC address of the non-AP STA that is reporting that it no longer has a connection with an ESS.”

At 465.24, change from

“This event is triggered to indicate the expectation that anESS will no longer be available for sending frames

in the near future.”

To

“This event is triggered to indicate the expectation that the connection to an ESS will no longer be available for sending frames in the near future.”

At 465.42, change from

“The MAC address of the non-AP STA that is reporting that an ESS is expected to go down.”

To

The MAC address of the non-AP STA that is reporting that the connection with an ESS is expected to no longer be present.

CID 2476

| 2476 | 80.51 | 4.4.3 |  |  | Shouldn't association, etc, have a qualifier for "not mesh service"? See the DSS examples. | As suggested. |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

The cited text is below:

The DSS examples are:

Discussion:

The cited text is in the PBSS control point section. The “not a mesh facility” qualifier isn’t needed, since the text states “Non-PCP STAs do not provide PCPS.”

Proposed resolution: Rejected

The cited text is in the PBSS control point section. The “not a mesh facility” qualifier isn’t needed, since the text states “Non-PCP STAs do not provide PCPS.”

CID 2475

| 2475 | 60.29 | 4.3.8 |  |  | The language used here makes me feel like a PCP is a BSS and therefore, that all statements about BSS within the standard are applicable to PBSS. Is this really true in such a general sense? Here is the sentence that creates the inference: "A STA within a DMG BSS is a QoS STA; hence a DMG BSS is a QoS BSS" | Verify that all statements about BSS within the standard are applicable to PBSS and correct the language in places where this is not true. |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

The cited sentence is:

“A STA within a DMG BSS is a QoS STA; hence a DMG BSS is a QoS BSS.”

The point of the text seems to be that each DMG BSS is by definition required to be a QoS BSS.

Not sure we need a “hence” statement to make the point and at the same time introduce the implication that all statements that apply to a BSS also apply to a PBSS.

Suggest a couple of options:

Change the cited text as shown below:

“A STA within a DMG BSS is a QoS STA; hence a DMG BSS is a QoS BSS.”

Alternatively, delete the cited sentence, as the sentence at P59L59 makes the same point:

“The QoS enhancements are available to QoS STAs associated with a QoS access point or PCP

in a QoS BSS.”

Discussion: pick one.

Proposed resolution: Revised

Delete the cited sentence, as the sentence at P59L59 makes the same point:

“The QoS enhancements are available to QoS STAs associated with a QoS access point or PCP

in a QoS BSS.”

CID 2474

| 2474 | 57.31 | 4.3.5.4 |  |  | Why is there no possible connection between a PCP and a CCSR? A PCP has access to a wireless medium and therefore, could communicate with a CCSR that was located within a STA on the wireless medium as indicated is allowed in P57L6 - furthermore, it should be possible for PCPs to communicate with each other in order to allow a PCP to PCP DS to be constructed. | Explain. |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

The cited text is

“There are no S-PCPs in an ECPAC since a PCP has no mechanism to communicate with the CCSR.”

It appears (P56L62 and P57L1-9) that by definition there are no S-PCPs in a CCSS:

“Centralized PCP/AP clustering is where there can be multiple S-APs in the BSA of any one S-AP

and all S-APs are coordinated via a single centralized coordination service set (CCSS).”

And

“A CCSS comprises a centralized coordination service root (CCSR) and a set of one or more synchronization APs that are stationary with respect to their local environment while operating and are connected to the CCSR via, for

instance, one of the following”

Additionally,

An ECPAC (P57L20-21) has only S-APs for synchronization:

“An extended centralized PCP/AP cluster (ECPAC) comprises a single CCSS and the set of centralized PCP/

AP clusters so that each S-AP of a centralized PCP/AP cluster is within the CCSS.”

An S-PCP is the Synchronization BSS control point that provides synchronization to a PCP/AP cluster.

And Annex Y, though informative, says

“Via unspecified means over the DS, a centralized coordination service root (CCSR) performs the following functions:

I don’t understand the option at P57L6 for S-AP connection to the CCSR “The wireless medium to an associated STA that contains the CCSR”. The CCSR in such a case is not accessed via the DS.

Suggest making text consistent for S-AP only.

Proposed resolution: Revised

Note to commenter: S-PCPs are used only with Decentralized clustering, see P56L61 and the CCSR is defined only using S-APs, see P56L62.

Delete the cited sentence at P57L30:

“There are no S-PCPs in an ECPAC since a PCP has no mechanism to communicate with the CCSR.”

And delete the sentence fragment at P57L6

“— The wireless medium to an associated STA that contains the CCSR”

CID 2472

| 2472 | 1368.21 | 10.1.4.5 |  |  | If an IBSS contains a mix of STA capabilities, e.g. non-HT and HT, then a non-HT STA cannot adopt the HT Op IE as shown in 6.3.3.2 nor can it transmit the IE in its beacons. So it does not transmit this IE in beacons then when an HT STA sees the beacon from the non-HT STA and the non-HT STA happens to have an older TSF, then does the HT STA stop transmitting the HT Op IE in its beacons? I.e. does the HT STA "adopt" the "lack of the presence of the HT Op IE"? There is a general question here of whether an IBSS falls to the LCD of capabilities of its members or not, and should there be some desription explicitly stating that a STA either DOES or DOES NOT fall to the LCD? | Include an explicit description of the behavior of a STA in an IBSS that has greater capability than other members - that is continues to send indication of those capabilities, e.g. HT Operation IE, even when the last beacon with the highest TSF has appeared with e.g. NO HT OP IE |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

What is the behaviour of legacy devices?

In 10.1.4.5 (Synchronizing with a BSS), the 5th and 6th paragraphs deal with IBSS. Could add text:

In addition to adopting the synchronization parameters as described in the first paragraph of this subclause, a STA joining an IBSS shall adopt each of the parameters found in the BSSDescription of the MLMEJOIN.request primitive according to the rule found for that parameter in the “IBSS adoption” column of the matching row of the BSSDescription table found in 6.3.3.3.2 (Semantics of the service primitive). Parameters adopted by a STA when joining an IBSS shall not be changed by the STA except when adopting parameters following the reception of a Beacon frame with a later timestamp as described in 10.1.5 (Adjusting STA timers).When an IBSS contains STAs with a mix of STA capabilities, e.g. non-HT and HT, a STA in the IBSS continues to send indication of its capabilities in beacon frames that it transmits, e.g. an HT Operation IE, even when the last beacon with the highest TSF has appeared without an HT Operation element.

In addition to the table entries in 6.3.3.3.2 (Semantics of the service primitive), if

dot11MultiDomainCapabilityActivated is true, a STA that is joining an IBSS and receives a Beacon or

Probe Response frame containing a Country element shall adopt the applicable parameters included in that Country element, and the dot11RegDomainsSupportedEntry shall be set to Other.

Discuss: Ensure the proposed additional text does not conflict with the Country element requirements.

CID 2471

| 2471 | 1155.55 | 9.19 |  |  | Several elements and frames use the operating class. Additions were made to the operating class for 11n and 11ac. These additions were for wider BW modes - older STA do not understand those operating classes, but the STA operating in the new operating classes generally include backwards compatible operating modes. e.g. 11ac includes 80 mhz BW operation and there is an operating class to support this legacy 11a STA do not understand the new operating class, but they can probably still associate as 20 mhz only STA, and therefore, would benefit by knowing that the 20 mhz operating class is supported by the target BSS/AP Therefore, whenever operating class is transmitted, it should be transmitted with the highest possible mutual understood operating class between the sender and the receiver, limited by the highest supported operating class of the third party being described |  |  | MAC |  | Needs submission |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

The comment is marked as “Needs submission”.

Assign to Matthew Fischer (the commenter).  No problem with the text is identified and suggested remedy is provided.

CID 2481

| 2481 | 1142.14 | 9.7.6.5 |  |  | ACK (and other control response frames) are supposed to be sent at rates as specified in this subclause. Generally speaking, this subclause prescribes ACK transmission at a Basic Rate, which is, for eliciting frames that are transmitted at higher rate or MCS values, typically a rate that is lower than the rate or MCS of the eliciting frame by at least two increments. When the eliciting frame is at one of the lower rates/MCS, then the ACK will often be at the same rate/MCS. Normally, this is ok - reliability of the ACK transmission is good or better than the eliciting frame because the eliciting frame usually has significantly more bits than the ACK. But if the eliciting STA has a higher TX power, then the rate for the ACK can be too high and is very likely to fail. It would be nice if the eliciting STA can signal to the responder that a lower rate/MCS will be needed. This ensures a receiveable ACK and it allows the eliciting STA to correctly compute the ACK duration for DUR field use. | Allow a control response frame to be sent at a rate/MCS lower than otherwise allowed when there is a power difference or for other reasons. Create a mechanism that allows the eliciting STA to indicate the preferred response rate/MCS. E.g. signal in the eliciting frame, a value of step down in rate/MCS from the normally computed value for the response frame. |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

The comment is marked as “Needs submission”. Assign to commenter.

CID 2467

| 2467 | 1625.59 | 10.33.2.2 |  |  | The language here is not well worded: "The initiator and responder shall move to the Initial state when the STT moves from 1 to 0 (other than set to 0)." | Propose to change text to: "The initiator shall transition to the Initial state when its STT transitions from 1 to 0, but not when the STT is set to 0. The responder shall transition to the Initial state when its STT transitions from 1 to 0, but not when the STT is set to 0. |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

The cited text is below:

Commenter proposes to change the cited text

From

"The initiator and responder shall move to the Initial state when the STT moves from 1 to 0 (other than set to 0)."

To

"The initiator shall transition to the Initial state when its STT transitions from 1 to 0, but not when the STT is set to 0. The responder shall transition to the Initial state when its STT transitions from 1 to 0, but not when the STT is set to 0.”

While the 2 sentences could perhaps be combined into a single sentence, the change improves the state transition description.

Proposed resolution: Accepted

CID 2470

| 2470 | 1368.48 | 10.1.4.5 |  |  | Need more clarity on what is to be adopted by a STA joining an IBSS. The language here implies, for example, that a STA that is unaware of say, HT parameters, must adopt them: "In addition to adopting the synchronization parameters as described in the first paragraph of this subclause, a STA joining an IBSS shall adopt each of the parameters found in the BSSDescription of the MLMEJOIN.request primitive according to the rule found for that parameter in the "IBSS adoption" column of the matching row of the BSSDescription table found in 6.3.3.3.2 (Semantics of the service primitive)." | Change to "In addition to adopting the synchronization parameters as described in the first paragraph of this subclause, a STA joining an IBSS shall adopt each of the parameters found in the BSSDescription of the MLMEJOIN.request primitive according to the rule found for that parameter in the "IBSS adoption" column of the matching row of the BSSDescription table found in 6.3.3.3.2 (Semantics of the service primitive) when those parameters exist at the joining STA." |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

The cited text in in 10.1.4.5 “Synchronizing with a BSS”:

The commenter proposes to change from:

"In addition to adopting the synchronization parameters as described in the first paragraph of this subclause, a STA joining an IBSS shall adopt each of the parameters found in the BSSDescription of the MLMEJOIN.request primitive according to the rule found for that parameter in the "IBSS adoption" column of the matching row of the BSSDescription table found in 6.3.3.3.2 (Semantics of the service primitive)."

To

"In addition to adopting the synchronization parameters as described in the first paragraph of this subclause, a STA joining an IBSS shall adopt each of the parameters found in the BSSDescription of the MLMEJOIN.request primitive according to the rule found for that parameter in the "IBSS adoption" column of the matching row of the BSSDescription table found in 6.3.3.3.2 (Semantics of the service primitive) when those parameters exist at the joining STA."

Agree with the commenter that the parameters may not exist. However prefer not to introduce the term “joining STA”. Is the following clear enough?

"In addition to adopting the synchronization parameters as described in the first paragraph of this subclause, a STA joining an IBSS shall adopt each of the parameters found in the BSSDescription of the MLMEJOIN.request primitive according to the rule found for that parameter in the "IBSS adoption" column of the matching row of the BSSDescription table found in 6.3.3.3.2 (Semantics of the service primitive) when those parameters exist at the joining STA."

Proposed resolution: Revised

Insert the phrase “when parameters exist at the STA” at the end of the ciited sentence, as shown below:

"In addition to adopting the synchronization parameters as described in the first paragraph of this subclause, a STA joining an IBSS shall adopt each of the parameters found in the BSSDescription of the MLMEJOIN.request primitive according to the rule found for that parameter in the "IBSS adoption" column of the matching row of the BSSDescription table found in 6.3.3.3.2 (Semantics of the service primitive) when those parameters exist at the joining STA."

CID 2466

| 2466 | 1624.35 | 10.33.2.2 |  |  | What does this text mean? "all the streams within the Switching Stream element that have the LLT Type field set to 1 shall be switched using the Stream-based Link Loss Countdown," - it sounds like it is saying that the session transfer will take place based on the LLC timer, but nowhere does it really say this - nowhere does it say, make the transition when the counter reaches zero, and furthermore, the counter counts down as long as no frame is received - what if a frame is received? Then the counter will be reloaded and so, if frames keep arriving for this stream, the stream will never transition! Is the intention that the source of the frames of the stream will stop sending the frames and this will cause the timer to reach 0? If so, this is very implicit and should be stated more clearly - and it is completely unclear which entity in the system will perform the gating to stop letting frames from this stream pass out to the network. I suspect that the entity that would do this is actually not a part of the 802.11 system, but some entity above 802.11 - in which case, it would still be nice to have some statement in here pointing to that fact. | Clarify just exactly how this counter is used to make a transition despite the possible reloads. |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

The cited text describes Fast Session Transfer (11ad) and is below:

Discussion:

Propose to assign to Carlos Cordiero

CID 2433

| 2433 | 2720.57 | C |  |  | There appears to be a discrepancy for the AIFSN EDCA parameter specified in Table 8-118 (Page 731) and that specified in the MIB (dot11EDCATableAIFSN, Page 2720). | Add reference to the default values used when dot11OCBActivated is true. |  | GEN |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

The comment is on a discrepancy between Table 8-118 AIFSN EDCA parameter and the dot11EDCATableAIFSN MIB variable when dot11OCBActivated (Outside the context of a BSS) is true.

The Table 8-118 definition is below:

And the MIB variable definition is below:

The dot11EDCATableIndex values are:

So, Table 8-118 says that when dot11OCBActivated (Outside the context of a BSS) is true, the default AIFSN values are 9/6/3/2 for BK/BE/VI/VO.

While the MIB variable says that the default AIFSN values are 7/3/2/2 for BK/BE/VI/VO.

Proposed Resolution: Revised

At 2720.57, change as indicated below:

dot11EDCATableAIFSN OBJECT-TYPE

SYNTAX Unsigned32 (2..15)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by the MAC upon receiving an EDCA Parameter Set in a Beacon

frame.

Changes take effect as soon as practical in the implementation.

This attribute specifies the number of slots, after a SIFS, that the STA,

for a particular AC, senses the medium idle either before transmitting or

executing a backoff.  See Tables 8-117 and 8-118. The default value for this attribute is

7, if dot11EDCATableIndex is 1,

3, if dot11EDCATableIndex is 2

2, otherwise."

::= { dot11EDCAEntry 4 }

Apply same changes to MIB variables at 2720.3 and 2720.24.

CID 2199

| 2199 | 1411.32 | 10.3.4.1 |  |  | Section 10.3.4.1 states that DMG STAs do not support authentication and deauthentication. This appears to be an optimisation that should only apply to cases where the Open Authentication algorithm is used, otherwise 11ad STAs cannot make use of other authentication algorithms such as SAE, Fast BSS Transition and those in 11ai. Even when Open Authentication is in use I'm not sure how multi-band operation is affected by this restriction. | Restrict this optimisation to cases where the Open Authentication algorithm is in use. This will require also changes in other parts of the draft, such as Figure 10-12 which shows authentication and association states. |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

The cited text is below:

The commenter notes that because DMG STAs don’t support.11 authentication frames, they can’t use protocols that are carried in .11 authenticaiton frames. That’s true. If we changed to allow exchange of authentication frames, use would be optional so as to not make existing implementations non-compliant. Are there interoperability issues – if a DMG STA sends an authentication request frame to another DMG sta that doesn’t support authentication frames, won’t connect. Need advertisement mechanism?

Need discussion. What was the rationale for dropping use of authentication frames in 11ad?

Assigned to Carlos Cordiero

Proposed resolution: TBD

Possible: Rejected

“The comment fails to identify changes in sufficient detail so that the specific wording of the changes that will satisfy the commenter can be determined.”

CID 2410

| 2410 | 2745.02 | C.3 |  |  | dot11HRCCAModeImplemented only appears in Annex C.3 I can find no reference to it outside of Annex C.3. Suggest it is deleted. | Delete dot11HRCCAModeImplemented at P2745L2, P2753L57, P2754L8-27, P2819L37 |  | GEN |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

The cited MIB variable is below (at P2754L8-27):

Assigned to Mark Hamilton

Proposed resolution: TBD

CID 2483

| 2483 | 2401.00 | C.3 |  |  | dot11BeaconInterval' is referred In 10.2.3.3 (Initialization of power management within an IBSS), but it is not defined C.3. | Define the dot11BeaconInterval in MIB. |  | GEN |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

The cited MIB variable name is incorrect – change.

Proposed resolution: Revised

Change from “dot11BeaconInterval” to “dot11BeaconPeriod”

CID 2065

| 2065 | 1200.14 | 9.22.7.2.2 |  |  | The insertion (9.22.7.2.2) is possibly in the wrong place. It is talking about Block Ack, not HT-Immediate block ack. Further, adding it necessitated turning the orginal contents of 9.22.7.2 into an introduction, when in fact it is the main meat of this subclause. | Find a better home for this subclause. Or if it in the right place, rename 9.22.7.2.1 to "General". |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

Agree with the commenter that the 11ad insertion is in the wrong place.

Looking for a better place…….

Proposed resolution: TBD

CID 2414, 2005

| 2414 |  | 29.56 |  | 3.2 |  |  |  |  |  | CID 1674 from LB193 was marked as ACCEPTED but the change was not implemented in D2.0 |  | Delete the definiteion for EAPOL-Start frame or add an appropriate definition |  |  | GEN |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2005 | 29.44 |  | 3.2 |  |  |  |  |  | The "See EAPOL-Key frame" is bogus, because the two terms are no longer synonyms. |  | Add a definition, or delete cited entry |  |

Discussion:

CID 1674 is below:

| 1674 | 27.55 | 3.2 |  | A | EAPOL-Key and EAPOL-Start are not the same. See 802.1X-2010 clause 11.3.2 | Delete Syn: EAPOL-Start frame. |
| --- | --- | --- | --- | --- | --- | --- |

And its resolution is:

| ACCEPTED (GEN: 2013-06-26 17:25:33Z) | EDITOR | 201307 approved | Resolved |  | I | EDITOR: 2013-07-25 14:53:56Z |
| --- | --- | --- | --- | --- | --- | --- |

The relevant text is below:

Proposed resolution: Revised

Change the definition to:

EAPOL-Start frame: A Data MPDU that carries an 802.1X EAPOL-Start PDU.

CID 2405

| 2405 | 1543.32 | 10.24.6 |  |  | The sentence "The STA does not support the fine timing measurement procedure." does not seem to have any meaning and can be removed. |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

The cited text and context is below:

Similar text is

And:

And 1542, lines 1-7:

There are multiple wordings that describe the “is supported” case. Additionally in some cases the “does not support” case is described (Timing measurement and Fine timing measurement” and in some cases the “does not support case” is not described in the text.

Agree that the cited text is not required.

Proposed resolution: Revised

Delete the text “"The STA does not support the [fine] timing measurement procedure." at 1542L1-2

and 1543 L30-32

CID 2406

| 2406 | 1543.22 | 10.24.6 |  |  | add explanatory text on what is the difference between timing measurement and the fine timing measurement. They both seem to provide the same thing, clock sync and flight time measurement. | as suggested |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

There is discussion on the 2 capabilities in clause 4.

Is additional text needed?

Proposed Resolution: Rejected

There is already text in 4.3.14.18 and 4.3.14.19 that describes each feature.

CID 2407

| 2407 | 1544.04 | 10.24.6 |  |  | Text says: "A STA that supports the fine timing measurement procedure may transmit a Fine Timing Measurement Request frame to a peer STA". Yet, the figure 10-31 shows the request frame as being mandatory (not dotted line). | Align figures 10-30 and 10-31. In both figures the Request Frame is optional (right?), but in 10-30 it is dotted line, while in 10-31 is not. Also in figure 10-30 there is a star next to the Request, what does that mean? Make the Request Frame in both figures either dotted line or continuous line. The star should also be present in both figures or in neither. |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

Assign to Brian Hart

CID 2426

| 2426 | 1879.14 | 13.5.7 |  |  | In the equation deriving MTK (1879.7), localLinkID and peerLinkID are used as well as nonces and MAC addresses. In the following paragraph starting from 1879.14, how "min" and "max" operations are treated and which octet is treated as MSB are described for nonces and MAC addresses. However, it is not clear how localLinkID and peerLinkID shall be treated in deriving MTK. | Please clarify how localLinkID and peerLinkID shall be treated (min, max operation and which bit is MSB). |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

Assign to Dan Harkins

CID 2427

| 2427 | 1895.08 | 13.10.8.6 |  |  | Sentence in 1895.7 reads: "In order to improve path stability (and further reduce overhead), a mesh STA may use the same originator HWMP SN for a certain time interval. In this case, the originator HWMP SN shall be incremented only after at least dot11MeshHWMPnetDiameterTraversalTime has elapsed since the previous increment." The normative behavior "the originator HWMP SN shall be incremented only after..." is conditional where the condition is given as "a mesh STA may use the same...". It should be more reasonable to say "In this case, the originator WMP SN is incremented only after...". | Replace "In this case, the originator HWMP SN shall be incremented only after ..." with "In this case, the originator HWMP SN is incremented only after ...". |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

The cited text is below:

The commenter proposes to change from

In order to improve path stability (and further reduce overhead), a mesh STA may use the same originator

HWMP SN for a certain time interval. In this case, the originator HWMP SN is incremented only after

at least dot11MeshHWMPnetDiameterTraversalTime has elapsed since the previous increment.

The preceeding “may” defines the requirement. Must the time interval be defined as in the current shall?

Propose to retain the current language. The “shall” defined the required elapsed time if the “may” condition is present.

Proposed Resolution: Revised

The “shall” defined the required elapsed time if the “may” condition is present.Reword to clarify the constraint on the time interval. Change the cited text as indicated below:

“ In this case, the originator HWMP SN shall not be incremented until

at least dot11MeshHWMPnetDiameterTraversalTime has elapsed since the previous increment. “

CID 2002, 2226, 2246, 2392

| 2002 | 28.18 | 3.2 |  |  | "The contiguous period of time" - do we need this qualification | Either remove "contiguous" here, or add it to all references to periods of time that are not otherwise qualified. |
| --- | --- | --- | --- | --- | --- | --- |
| 2226 | 28.18 | 3.2 |  |  | "contiguous" means either "sharing a border" or "near another entity". What border is shared or other entity near a "contiguous interval"? | Replace "contiguous" with "continuous". |
| 2246 | 55.55 | 4.3.5.2 |  |  | "contiguous" means either "sharing a border" or "near another entity". What border is shared or other entity near overlapping BSSs? | Replace "contiguous" with "continuous". |
| 2392 | 1944.08 | 13.14.9.1 |  |  | "contiguous" means either "sharing a border" or "near another entity". But what is a single time period contiguous with? | Replace "contiguous" with "continuous". |

Discussion:

The comment is similar to that in CIDs 2338, 2339, 2359, 2336 which have already been resolved.

Cited text is below:

At 28.18

At 55.55

At 1944.08

Proposed resolution: Revised

Delete “contiguous” at 28.18, and 1944.08.

At 55.55, change to “This is commonly used to provide complete coverage within a

physical volume.”

References: