IEEE P802.11
Wireless LANs

| TGaq – PADP(Pre-Association Discovery Protocol) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-07-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| RYU Cheol | ETRI | 161 Gajeong-dong, Yuseong-gu, Daejeon, Korea | +82-42-860-6604 | ryuch@etri.re.kr |

Abstract

This document is the text for proposed changes of the Pre-Association Discovery Protocol (PADP) D.01

8.4.2.122a Pre-association Discovery Capabilities

Change the Figure 8-401aq and text as the following.

|  | Element ID | Length | Update Counter | Service Type mask | PADP ID | ULP List count (optional) | ULP ID#1 (optional) | … | ULP ID#n (optional) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  | 1 |

Figure 8-401aq – PADP Capabilities element format

The Element ID field is set to the value given in Table 8-54 [this table requires a new value adding to it].

The value of the Length field is 2 plus the number of optional ULP ID elements.

The Update Counter is the not-decreasing number of the revision of the service information in AP.

The Service Type mask is a 1 octet bit mask, indicating the Service Types which are supported by an AP.  The format of the mask is shown in Figure 8-402aq.

After Figure 8-402aq – Service 3 Type mask format,  Change the text and Table 8-401aq and the following as the below

The PADP identifier (ID) field indicates the PADP supported by an AP. The PADP ID is one of the following

Table 8-401aq – PADP Identifiers

| PADP | ULP ID |
| --- | --- |
| PAD request/response using a Service Identifier Hash | 0 |
| PAD request/response using Service Identifier Number | 1 |
| Encapsulated ULP | 2 |
| Reserved | 3-220 |
| Vendor Specific | 221 |
| Reserved | 222-255 |

The PADP identifier (ID) field is a bit mask for the PADPs supported by an AP. The format of the mask is shown in Figure 8-<TBD>

|  | PAD request/response using a Service Identifier Hash | PAD request/response using Service Identifier Number | Encapsulated ULP | PAD broadcasting | Reserved | Vendor Specific | Reserved |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bit: | 0 | 1 | 2 | 3 | 4-5 | 6 | 7 |

Figure 8-<TBD> - PADP Identifier Mask

8.4.6.1 General

Change as the following including Figure 8-403aq

PADP-elements are defined to have a common format consisting of a 2-octet Info ID field, a 2-octet Length field, a 1-octet PADP Indentifier mask, and a variable-length element-specific Information field. The PADP-element format is shown in Figure 8-403aq:

|  | Info ID |  | Length |  | PADP Indentifier mask |  | PADP Protocol- specific Information |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: |  | 2 |  | 2 |  | 1 |  | variable |  |

Figure 8-403aq – PADP-element format

Each PADP-element in 8.4.6 is assigned a value of 276.

The Length field is a 2-octet field that indicates the number of octets in the Information field and is encoded following the conventions given in 8.2.2 (Conventions).

PADP Indentifier mask is a 1-octet field that is a bit mask for the PADPs supported by an AP and it is shown in Figure Figure 8-<TDB> PADP Identifier mask.

The PADP Protocol-Specific Information field is a variable length field that contains a specific PADP element definition.

8.4.6.2 PADP Request/Response using a Service Identifier Hash

The PADP Request/Response using a Service Identifier Hash (SIH) is a transaction-oriented protocol for exchanging Service Discovery information, which identifies services by using a hash as explained in Annex AQ1.  The hash is used both as a identifier within a service query request and also as a response identification of services that are available within the AP.

The PADP Protocol-Specfic Information fields are defined as follows:

8.4.6.2.1 Request PADP-element

The Request PADP-element is used to request service information between STAs using the PADP Request/Response protocol. The Request PADP-element is included in a GAS Query Request.

|  | Token | SIH | Service Type Mask |
| --- | --- | --- | --- |
| Octets: | 1 | 6 | 1 |

Figure 8-404aq – Request PADP-element format

The token identifies a specific instance of the PADP request.

The Service Identifier Hash (SIH) field is an 6 octet version of a Unique Service Identifier (USID) for the service which is within the Request or Response PADP-element.  See Annex AQ1.2 for further information.

8.4.6.2.2 Response PADP-element

The Response PADP-element is used to provide service information between STAs using the GAS protocol in response to a Request PADP-element. The Response PADP-element is used in a GAS Query Response.

|  | Token | Service Descriptor List Count | SD#1 (optional) | … | SD#n (optional) |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |  | variable |

Figure 8-405aq – Response PADP-element format

The Service Descriptor List Count is a 1 octet field indicating the number of Service Descriptor sub-elements in the Response PADP-element.

The format of the Service Descriptor sub-field is shown in Figure 8-406aq

|  | Length | SIH | Additional Attributes Count | Additional Attribute#1 (optional) | … | Additional Attribute #n (optional) | ULP (optional) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octet: | 2 | 6 | 1 | 4 |  | 4 | 1 |

Figure 8-406aq – Service Descriptor sub-field format

The Length field is a 1 octet field indicating the length of the Service Descriptor sub-field.

The SIH sub-field is described in section 8.4.6.2.1.

The Additional Attributes Count is a 1 octet field indicating the number of Additional Attribute sub-fields.

The Additional Attribute sub-field is a 4 octet field each of which contain additional information about the service identified by the SIH, for example, a service name string.

The ULP sub-field is an optional 1 octet field that identifies an ULP associated with the SIH and is defined in section 8.4.2.122a.

8.4.6.3 PAD Request/Response using Service Identifier Number

The PADP Request/Response using a Service Identifier Number (SIN) is a transaction-oriented protocol for exchanging Service Discovery information, which identifies services by using a number.  The number is used both as a identifier within a service query request and also as a response identification of services that are available within the AP.

The PADP Protocol-Specfic Information fields are defined as follows:

8.4.6.2.1 Request PADP-element

The Request element is used to request service information between STAs using the PADP Request/Response protocol. The Request PADP-element is included in a GAS Query Request.

|  | Token | SIN | Service Type Mask |
| --- | --- | --- | --- |
| Octets: | 1 | 6 | 1 |

Figure 8-406aq – Request PADP-element format

The token identifies a specific instance of the PADP request.

The SIN sub-field is a 6 octet free form number of a service which is within the Request or Response PADP-element. See Annex AQ1.3 for further information. [this requires some more detail]

8.4.6.2.2 Response PADP-element

The Response PADP-element is used to provide service information between STAs using the GAS protocol in response to a Request PADP-element. The Response PADP-element is used in a GAS Query Response.

|  | Token | Service Descriptor List Count | SD#1 (optional) | … | SD#n (optional) |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |  | variable |

Figure 8-407aq – Response PADP-element format

The Service Descriptor List Count is a 1 octet field indicating the number of Service Descriptor sub-elements in the Response PADP-element.

The format of the Service Descriptor sub-field is shown in Figure 8-408aq

|  | Length | SIN | Additional Attributes Count | Additional Attribute#1 (optional) | … | Additional Attribute #n (optional) | ULP (optional) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octet: | 2 | 6 | 1 | 4 |  | 4 | 1 |

Figure 8-408aq – Service Descriptor sub-field format

The Length field is a 1 octet field indicating the length of the Service Descriptor sub-field.

The SIN sub-field is described in section 8.4.6.2.1

The Additional Attributes Count is a 1 octet field indicating the number of Additional Attribute sub-fields.

The Additional Attribute sub-field is a 4 octet field each of which contain additional information about the service identified by the SPI, for example, a service name string.

The ULP sub-field is a 1 octet field that identifies an ULP associated with the SPI and is defined in section 8.4.2.122a.

Change the text as the following after 8.4.6.3.2 Response PADP-element

8.4.6.4 PAD Request/Response using ULP Encapsulation

The Encapsulation element is used to exchange upper layer protocol (ULP) frames between STAs using the GAS protocol. The Encapsulation PADP-element is used as a transaction request included in a GAS Query Request or returned as a transaction response in the GAS Query Response.

|  | Length | Token | <TBD> |
| --- | --- | --- | --- |
| Octets: | 2 | 2 | variable |

Figure 8-409aq – Encapsulation PADP-element format

The Length and Token fields are defined in 8.4.6.1 (General).

The format of the <TBD> sub-field is  <TBD>.

Insert the following after 8.4.6.3.2 Response PADP-element

8.4.6.5 PAD Broadcasting of Service Information

The PAD Broadcasting of Service Information-element is used to broadcast service information. The PAD Broadcasting of Service Information-element is included in a GAS frame. The bit of Advertisement Broadcasting The field of PAD Identifiers mask in PADP-element format is on and only one of PAD request/response using a Service Identifier Hash and PAD request/response using  Service Identifier Number bit could be on. For the broadcasting of service information, the bit for ULP encapsulated can’t be set.

|  | Broadcasting Window | Service Descriptor List Count | SD#1 (optional) | … | SD#n (optional) |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |  | variable |

Figure 8-404aq – PAD Broadcasting of Service Information-element format

The Broadcasting Window specifies the window of the next broadcasting of service information for periodic broadcasting.

The PAD Identifier determine the next filed to be SIH or SIN. In the cast of the bit for PAD request/response using a Service Identifier Hash is on, it is SIH. In the case of the bit for PAD request/response using  Service Identifier Number is on,  it is SIN. The Service Identifier Hash (SIH) field is an 6 octet version of a Unique Service Identifier (USID) for the service which is within the Request or Response PADP-element.  See Annex AQ1.2 for further information.The SIN sub-field is described in section 8.4.6.2.1

The format of the Service Descriptor sub-field is shown in Figure 8-408aq and Figure 8-408aq.

10. MLME

10.24 WLAN interworking with external networks procedures

10.24.3 Interworking procedures: generic advertisement service (GAS)

Insert new subclause 10.24.3.4 after the end of 10.24.3.3 as follows:

10.24.3.4 Pre-Association Discovery Protocol (PADP) procedures

When dot11PADActivated is true, a STA shall include a PADP Capabilities element in Beacon and Probe Response frames.

<TBD> - text on how the PADP-elements are indicated by a STA

<TBD> - text on how the PADP-elements are interpreted by a non-AP STA.

When dot11PADActivated is true, a non-AP STA may use PADP to request information as defined in Table 8-175 (Advertisement protocol ID definitions) from a peer STA that has transmitted an Advertisement Protocol element indicating support for PADP (see 8.4.2.95 (Advertisement Protocol element)) in a Beacon or Probe Response frame. The receiving STA may respond to queries with or without proxying the query to a server in an external network. See Annex AQ2 for more information on use of a Proxy Entity

- <TBD> The PADP Capabilities element advertises service types as well as the PADP protocol supported by the STA.

- <TBD>The requesting STA shall use the protocol advertised  by the responding STA.

If information is not configured for a particular PADP-element, then a query for that element returns that element with no optional fields.

A STA may use PADP to exchange service information as defined in Table 8-190 from a peer STA.  PADP uses the Encapsulation PADP-element to transport a Service Identifier between STAs.

A STA that encounters an unknown or reserved PADP Info ID value in a GAS frame (see 8-210) received without error shall ignore that PADP Info ID and shall parse any remaining PADP Info IDs.

A STA that encounters an unknown vendor-specific PADP-element field or subfield in a GAS frame (see 8-210) received without error shall ignore that field or subfield respectively, and shall parse any remaining fields or subfields for additional information with recognizable field or subfield values.

10.24.3.4.1 PAD Request/Response  using Service Hash  procedures

<TBD> - Normative description of the protocol here.

10.24.3.4.2 PAD Request/Response  using Service Identifier Number  procedures

<TBD> - Normative description of the protocol here.

10.24.3.4.3 PAD Encapsulation Protcol  procedures

<TBD> - Normative description of the protocol here.

Insert the following after the end of 10.24.3.4.3 PAD Encapsulation Protocol procedure

10.24.3.4.4 PAD Broadcasting of Service Information

<TBD> - Normative description of the protocol here.

Annex C

(normative)

ASN.1 encoding of the MAC and PHY MIB

C.3 MIB Detail

Insert new MIB values as follows:

dot11PADImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable. Its value is determined by device capabilities.

This attribute when true, indicates the STA is capable of pre-association discovery (PAD)

with external networks. A STA setting this to true implements PAD. When this is false, the STA

does not implement PAD.

DEFVAL {false}

::= { dot11StationConfigEntry <tbd> }

dot11PADActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity or the SME. Changes take

effect as soon as practical in the implementation.

This attribute when true, indicates the capability of the STA to operate PAD

with external networks is enabled. The capability is disabled otherwise."

DEFVAL {false}

::= { dot11StationConfigEntry <tbd> }

Annex AQ1

(normative)

AQ1.1 Service Discovery

STA

MAC

PAD Proxy

AP

MAC

Access Network

Service Protocol Specific Communication

P802.11aq Signaling

PAD Proxy

AQ1.1 Unique Service Identifier (USPI)

<see https://mentor.ieee.org/802.11/dcn/13/11-13-0893-00-00aq-service-discovery-proposal.pptx>

- 128 bits long (16 octets) is large enough to be statistically unique (~3E+38)

- It is a type of “UUID”, a well defined construct in other standards.

AQ1.2. Service Identifier Hash (SPH)

Provides a convenient short identifier (e.g. 6 octets)

May not always be unique, there may be collisions. Collisions, however, can be very rare for well selected sizes and collision impact can be mitigated

Multiple SPIs can be created from the same Unique Service Identifier by taking different ranges for the truncation (e.g. First 6 octets, next 6 octets ...)

AQ1.3. Service Identifier Number (SPN)

Provides a convenient short identifier (e.g. 6 octets)

May not always be unique, there may be collisions. Collisions, however, can be very rare for well selected sizes and collision impact can be mitigated.

Annex AQ2

(normative)

AQ2.1 Proxy Entity

<placeholder> for description of a service directory in the STA. This is required to pass information up to higher layer applications (e.g. a service ID hash) and for the PADP to work.

It is assumed that there is proxy function in the network that maintains a list of services. The upper layer protocols are not exposed to un-associated STAs. The proxy is used to encapsulate the service identifiers and exchange that information to the STA.

Therefore PADP is opaque to the service definition and is handled by the proxy and the end STA itself.