IEEE P802.11
Wireless LANs

| REVmc - some MAC comment resolutions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2015-09-25 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Stephen McCann | BlackBerry Ltd | 200 Bath Road, Slough, Berkshire, SL1 3XE, UK | +44 1753 667099 | smccann@blackberry.com |

Abstract

This document contains some proposed resolutions to SB0 comments.

CID 5395, 6355

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Resolution | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5395 | 1063 | 8.4.5.1 |  | Presumably the ANQP-element has its unique hyphenated name because it has longer ID and Length fields. But there is no reason to limit this longer format to ANQP. At least future-proofing will be helped by having a longer element form available. To do this, create a more generic element name, maybe "longelement". Then "ANQP-element" becomes "ANQP longelement". | Replace "ANQP-element" with "ANQP longelement" throughout the draft. |  | MAC |

Proposed Resolution.

Reject.  The use of the term ANQP-element has nothing to do with a “longer ID”. See clause 8.4.2.1 for the definition of the Element ID Extension field.

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Resolution | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6355 | 3575 | V.3.3 |  | Table V-2---Example Enterprise DSCP to UP/AC mapping shows UP 2 being AC_BE, but shouldn't it be AC_BK per Table 9-1? | Change the two AC_BEs for UP 2 to AC_BKs |  | MAC |

Proposed Resolution.

Revised. Within Table V-2, change Per-hop behaviour (PHB) CS2 to User Priority 3 and Per-hop behaviour (PHB) AF1x to Access Category AC_BK.

Proposed Changes:

(The changes are shown in bold italic.)

Table V-2—Example Enterprise DSCP to UP/AC mapping

| Application Class | Per-hop behavior (PHB) | IEEE Std(#130) 802.1D(#3 128)) User Priority | Access Category |
| --- | --- | --- | --- |
| Network Control | CS6 | 7 | AC_VO |
| Telephony | EF | 6 | AC_VO |
| RT Interactive | CS4 | 6 | AC_VO |
| Multimedia Conference | AF4x | 5 | AC_VI |
| Signaling | CS5 | 5 | AC_VI |
| Broadcast Video | CS3 | 4 | AC_VI |
| Multimedia Stream | AF3x | 4 | AC_VI |
| Low Latency Data | AF2x | 3 | AC_BE |
| High Throughput Data | AF1x | 2 | AC_BEK |
| OAM | CS2 | 23 | AC_BE |