IEEE P802.11
Wireless LANs

| Specification Framework for TGay |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2016-10-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Carlos Cordeiro | Intel | 2111 NE 25th Ave, Hillsboro OR 97124, USA | +1-503-712-9356 | carlos.cordeiro@intel.com |

Abstract

This document provides the framework from which the draft TGay amendment will be developed. The document provides an outline of each the functional blocks that will be a part of the final amendment. The document is intended to reflect the working consensus of the group on the broad outline for the draft specification. As such it is expected to begin with minimal detail reflecting agreement on specific techniques and highlighting areas on which agreement is still required. It may also begin with an incomplete feature list with additional features added as they are justified. The document will evolve over time until it includes sufficient detail on all the functional blocks and their inter-dependencies so that work can begin on the draft amendment itself.

- Definitions and acronyms

  - Acronyms

| DL | Downlink |
| --- | --- |
| DMG | Directional Multi-Gigabit |
| EDMG | Enhanced DMG |

  - Definitions

enhanced directional multi-gigabit (EDMG) basic service set (BSS): A BSS in which a DMG Beacon frame transmitted by an EDMG station (STA) has the EDMG Supported field equal to one.

enhanced directional multi-gigabit (EDMG) station (STA): A directional multi-gigabit (DMG) STA whose radio transmitter is capable of transmitting and receiving EDMG physical layer (PHY) protocol data units (PPDUs).

- Frame formats

  - DMG Beacon

A EDMG Supported field shall be defined by using one reserved bit from the DMG Parameters field. An EDMG STA shall set the EDMG Supported field to 1.

  - Control frames

The 11ay specification shall define a generic MAC control frame that includes, but is not limited to, at least one of the following: SU/MU MIMO setup, channel aggregation and OFDMA. The generic MAC control frame will be signaled by the control frame extension.

    - Block Ack

The size of the Block Ack bitmap for EDMG STAs shall be one of 64, 128, 256, 512 or 1024 bits. The size of the Block Ack bitmap for a TID shall be the smallest size that is greater than the minimum between the value of the Buffer Size field in the ADDBA Request and Response frames that set up the block ack agreement for the TID.

- The EDMG Multi-TID BlockAck variant is indicated by setting the BA Type subfield of the BA Control field as shown in Table 1.

- —BlockAck frame variant encoding

| BA Type (B1 B2 B3 B4) | BlockAck frame variant |
| --- | --- |
| 0001 | EDMG Compressed BlockAck |
| 1110 | EDMG Multi-TID BlockAck |

      - EDMG Compressed BlockAck variant

The TID_INFO subfield of the BA Control field of the EDMG Compressed BlockAck frame contains the TID for which a BlockAck frame is requested.

The BA Information field of the EDMG Compressed BlockAck is formatted as indicated in Figure 1.

| Octets: | 2 | variable | 1 |
| --- | --- | --- | --- |
|  | BlockAck Starting Sequence Control | BlockAck Bitmap | RBUFCAP |

- —BA Information field (EDMG Compressed BlockAck)

The Starting Sequence Number subfield of the BlockAck Starting Sequence Control subfield contains the sequence number of the first MSDU or A-MSDU for which this BlockAck frame is sent. The value of this subfield is defined in 10.24.7.5 (Generation and transmission of BlockAck frames by an HT STA or DMG STA). The Fragment Number subfield of the Block Ack Starting Sequence Control subfield defines the size of the BlockAck Bitmap subfield within the BA Information field and is defined in Table 2.

- —Fragment Number subfield definition

| Value of Fragment Number subfield | Size of BlockAck Bitmap subfield (octets) |
| --- | --- |
| 0 | 8 |
| 1 | 16 |
| 2 | 32 |
| 3 | 64 |
| 4 | 128 |
| Others | Reserved |

The BlockAck Bitmap subfield of the BA Information field is used to indicate the received status of up to 8 times the size of the subfield, where each entry represents an MSDU or an A-MSDU. Each bit that is set to 1 in the BlockAck Bitmap subfield acknowledges the successful reception of a single MSDU or A-MSDU in the order of sequence number. The first bit of the BlockAck Bitmap subfield corresponds to the MSDU or A-MSDU with the sequence number that matches the value of the Starting Sequence Number subfield of the BlockAck Starting Sequence Control subfield.

The RBUFCAP field is defined in 9.3.1.9.5.

      - EDMG Multi-TID BlockAck variant

The EDMG Multi-TID BlockAck variant has the format shown in Figure 2.

| Octets: | 2 | 2 | 6 | 6 | 2 | 144 | 2 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Frame Control | Duration/ID | RA | TA | BA Control | BA Information | ACK Bitmap | FCS |

- —EDMG Multi-TID BlockAck variant definition

The BA Control field is formatted as indicated in Figure 3.

|  | B0 | B1 | B2 | B3 | B4 B11 | B12 | B13 B15 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BA ACK Policy | Multi-TID | Compressed Bitmap | GCR | Reserved | Management ACK | Reserved |
| Bits: | 1 | 1 | 1 | 1 | 8 | 1 | 3 |

- —BA Control field definition

The Management ACK field is set to one to indicate that the Management frame is acknowledged. Otherwise, it is set to zero.

The BA Information field is a concatenation of eight Per-TID BA Information subfields, one per TID. The Per-TID BA Information subfield is formatted as indicated in Figure 4.

| Octets: | 2 | 16 |
| --- | --- | --- |
|  | BlockAck Starting Sequence Control | BlockAck Bitmap |

- —Per-TID BA Information subfield definition

The BlockAck Starting Sequence Control subfield is defined in Figure 5.

|  | B0 B3 | B4 B15 |
| --- | --- | --- |
|  | TID | BA Starting Sequence Number |
| Bits: | 4 | 12 |

- —BlockAck Starting Sequence Control subfield definition

  - Information elements

    - EDMG Capabilities element

A non-AP or non-PCP EDMG STA declares that it is an EDMG STA by transmitting the EDMG Capabilities element.

The EDMG Capabilities element shall include the antenna polarization capability of the EDMG STA. The specific content of the antenna polarization capability is TBD.

    - Measurement Request element

The Directional Channel Quality Request and Report frames shall be modified to support:

- Request and report for measurements on multiple channels, and

- Measurements reports averaged over multiple channels

    - EDMG Extended Schedule element

The EDMG Extended Schedule element defines incremental signaling to the Extended Schedule element, including channel allocation indications if allocations are scheduled over more than one channel.

The format of the EDMG Extended Schedule element is shown in Figure 6.

|  | Element ID | Length | Element ID Extension | Number of Allocations | Channel Allocation |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 5×N |

- —EDMG Extended Schedule element format

The Number of Allocations field indicates the number, N, of Channel Allocation fields following it.

The Channel Allocation field is defined in Figure 7 and specifies the allocation and the bandwidth that the allocation occupies.

|  | B0-B23 | B24 | B25-B32 | B33-B39 |
| --- | --- | --- | --- | --- |
|  | Allocation Key | Channel Aggregation | BW | Reserved |
| Bits: | 24 | 1 | 8 | 7 |

- —Channel Allocation field format

The contents of the Allocation Key subfield are used to identify the allocation. This is done by correlating the contents of this subfield with the information obtained from the Extended Schedule element transmitted in the same frame. The Allocation Key subfield is formatted as shown in Figure 8.

|  | B0-B3 | B4-B11 | B12-19 | B20-23 |
| --- | --- | --- | --- | --- |
|  | Allocation ID | Source AID | Destination AID | Reserved |
| Bits: | 4 | 8 | 8 | 4 |

- —Allocation Key field format

The Allocation ID, Source AID and Destination AID subfields are collectively used to identify the allocation included as part of the Extended Schedule element.

The Channel Aggregation and BW subfields are defined in Table 6. These fields specify the properties of the allocation identified by the Allocation Key subfield.

  - MIMO

An EDMG STA may transmit a “MIMO Setup frame” (e.g., RTS or DMG CTS-to-self) prior to the transmission of a SU or MU MIMO PPDU. The “MIMO Setup frame” indicates the destination STA(s) addressed by the PPDU.

A “MIMO Setup frame” (e.g., RTS) transmission may trigger a response (e.g., DMG CTS or Ack) from destination STA(s).

- MAC sublayer

  - Channel access

To enable backward compatibility with non-EDMG STAs, in an EDMG BSS the BTI, A-BFT and ATI is always present on the primary channel of the BSS. The channel width of the primary channel of an EDMG BSS is 2.16 GHz.

    - Control trailer

A Control Trailer may be used in Control mode PPDUs to provide additional control signaling such as bandwidth information in RTS and DMG CTS frames. An EDMG STA shall set the TXVECTOR parameter CONTROL_TRAILER to Present to indicate the presence of the control trailer in a control mode PPDU. The control trailer is defined in 6.2.7.

    - Channel access over multiple channels

When using multiple channels, the following apply:

- Full carrier sense (physical and virtual) shall be maintained on a primary channel.

- An EDMG STA may transmit a frame to a peer EDMG STA to indicate intent to perform channel bonding transmission to the peer STA. This allows an EDMG STA to choose to operate over multiple channels only after receiving such a frame, thus saving power.

- At least CCA of energy detection shall be performed on each non-primary channel

The 11ay specification supports that, when using multiple channels, a PCP or an AP can simultaneously transmit to multiple STAs allocated to different channels individually.

The 11ay specification shall support allocation of SP(s) and scheduled CBAP(s) over more than one channel and over a bonded channel through DMG Beacon frame, Announce frame, and Grant frame. When allocations over different channels overlap in time, the source and destination of such allocations shall be different. Channels used for such allocations shall be limited to the operating channels of the BSS. An allocation and channel access within the allocation follow the following rules:

- If the allocation is a scheduled CBAP with either source AID or destination AID different than the broadcast AID or the allocation is an SP, then:

- The allocation does not have to include the primary channel

- Channel access during the allocation shall be limited to the channels describing the allocation

- If the allocation is a scheduled CBAP with both source AID and destination AID equal to the broadcast AID, then channel access during the allocation shall include the primary channel

- If the CBAP Only field is equal to one, then channel access within the CBAP shall include the primary channel.

A TXOP initiator shall not use a secondary channel if CCA of the channel is busy and may use the channel otherwise.

A TXOP responder shall be able to receive a frame sent on a channel that does not include all secondary channel(s).

If an EDMG Extended Schedule element is present in a transmitted frame, an Extended Schedule element shall also be present in the same frame. An allocation included in the EDMG Extended Schedule element shall be ignored if the contents of the Allocation Key subfield of this allocation do not match to an allocation present in the Extended Schedule element.

    - MIMO channel access

An EDMG STA may transmit a frame to one or more EDMG STAs to indicate a target time in which the transmitting STA intends to contend for the channel to send a MIMO PPDU to the STA(s).

  - DMG beamforming

The 11ay SLS beamforming protocol shall enable feedback of one or more sectors per TX and RX antenna.

The 11ay beamforming protocol shall enable TX and RX training using the same BRP frame.

The 11ay beamforming protocol shall enable simultaneous BF training of transmit DMG antennas.

The 11ay beamforming protocol shall support simultaneous RX antenna training.

The 11ay beamforming protocol shall provide the means to enable TX sector down selection as part of beamforming training.

The 11ay beamforming protocol shall support, for STAs with antenna pattern reciprocity, RX sector down selection as part of beamforming training.

The 11ay beamforming protocol shall support a mode of operation in which, in a single phase, the initiator trains its transmit antennas/sectors and multiple responders train their receive antennas/sectors.

The 11ay beamforming protocol supports multi-beamforming for multiple array antennas. Multi-beamforming means that a transmitter simultaneously sends SSW frames in multiple polarized directions. 11ay provides the means of multi-beamforming capability exchange in which the initiator and responders exchange the information on whether they have the capability of multi-beamforming.

An EDMG STA shall not transmit a Short SSW packet to another STA that is not also an EDMG STA.

For SLS using Short SSW frames, LBIFS is equal to 2×TXTIME(Short SSW) + 3×SBIFS.  An EDMG STA performing an SLS using Short SSW frames shall increase the value of the CDOWN field within the Short SSW frame by two for each LBIFS contained as part of a sector sweep.

    - A-BFT

The 11ay beamforming protocol should support simultaneous multi-STA RSS over multiple channels during the A-BFT.

The 11ay beamforming protocol shall support dynamic load balancing in A-BFT by providing means to adjust necessary parameters (e.g., RSSBackoff or/and RSSRetryLimit).

It shall be possible to insert additional SSW slots in the beacon interval for use by EDMG STAs only. The number of additional SSW slots in the beacon interval shall be equal to the value of A-BFT Length subfield multiplied by the value of the A-BFT Multiplier subfield, where the A-BFT Multiplier subfield is indicated using reserved bits from the Beacon Interval Control field of the DMG Beacon frame. If present in a beacon interval, the additional SSW slots shall be inserted immediately before the legacy A-BFT.

  - SU MIMO beamforming

BF for SU MIMO comprises the following phases:

- SLS:  Coarse BF training to allow TX sector down selection for Initiator and Responder including feedback with candidate sectors.

- TX-RX Sector/Antenna Mapping:  BF transining of RX sectors/antennas/RF chains for each of the candidate TX sectors to determine best TX-RX configuration for SU MIMO.

- Others:  TBD

  - MU MIMO beamforming

BF for MU-MIMO supports training of more STAs, including collecting their feedback, than the number of STAs transmitted to in each MU-MIMO transmission.

- MLME

  - Spatial reuse and interference mitigation

11ay shall provide means to extend the DMG spatial sharing mechanism for operation in a multi-channel configuration.

The EDMG spatial sharing mechanism shall enable an EDMG STA to perform concurrent measurements employing multiple RX antenna configurations as are used for receiving multiple streams from the target EDMG STA based on the same measurement configuration.

The EDMG spatial sharing mechanism shall enable an EDMG STA to report the average of the results of concurrent measurements using multiple RX antenna configurations in order to reduce the overhead of the measurement report.

- Security

An EDMG STA may include the Authentication IE within Management and Extension frames during discovery and association phase to parallelize the pre-shared key based authentication and key generation, and speed up the process of authentication and association.

- EDMG PHY

  - General

The 11ay specification shall support:

- SU-MIMO

- Downlink MU-MIMO

- Channel bonding of at least two 2.16 GHz

- Contiguous and non-contiguous channel aggregation of no more than two 2.16 GHz channels or two 4.32 GHz channels

- A 64-point non-uniform constellation

For MIMO transmissions, the 11ay specification shall enable transmit diversity schemes, including Alamouti code, for both SC and OFDM modulations.

11ay shall enable both SC and OFDM modulations for SU-MIMO and MU-MIMO data transmission.

The 11ay specification shall support SU-MIMO with predetermined and/or no precoding matrices.

11ay supports channel-wise DL OFDMA. This means that in a bonded channel, a PCP or AP can simultaneously transmit to multiple STAs allocated to different frequency resources in the unit of one channel bandwidth.

A 2.16 GHz duplicated mode shall be included in the 11ay specification.

  - Common parameters

    - Introduction

    - EDMG PPDU format

A single PPDU format is defined for all EDMG PHYs: the EDMG PPDU format. Figure 9 shows the EDMG PPDU format and all possible fields. Not all fields are transmitted in an EDMG PPDU. Fields are included depending on whether the PPDU is a SU PPDU or is a MU PPDU.

| L-STF | L-CEF | L-Header | EDMG-Header-A | EDMG-STF | EDMG-CEF | EDMG-Header-B | Data | AGC | TRN |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- —EDMG PPDU format

The fields of the EDMG PPDU format are summarized in Table 3.

- —Fields of the EDMG PPDU

| Field | Description |
| --- | --- |
| L-STF | Non-EDMG Short Training field |
| L-CEF | Non-EDMG Channel Estimation field |
| L-Header | Non-EDMG Header field |
| EDMG-Header-A | EDMG Header A field |
| EDMG-STF | EDMG Short Training field |
| EDMG-CEF | EDMG Channel Estimation field |
| EDMG-Header-B | EDMG Header B field |
| Data | The Data field carriers the PSDU(s) |
| AGC | Automatic Gain Control field |
| TRN | Training sequences field |

The EDMG-Header-A, EDMG-STF, EDMG-CEF and EDMG-Header-B fields exist only in EDMG PPDUs.

    - EDMG preamble

      - Introduction

      - Non-EDMG portion of EDMG format preamble

        - Cyclic shift for pre-EDMG modulated fields

For SC MIMO transmissions, the 11ay specification shall support a preamble structure with cyclic shift. A cyclic shift value TSCi is used for the L-STF, L-CEF, L-Header and EDMG-Header-A fields of the PPDU for stream i out of a total of N streams, where 1 ≤ i ≤ N. TSCi is defined in SC chip units at 1.76 GHz for single channel or multiple channels transmission. TSCi shall be inserted after EDMG-Header-A. The particular TSCi value for each configuration is TBD.

The parameter TSCi for the ith spatial stream is defined as TSCi = (i-1)×Nc×Tc, where Nc is the number of SC chips and Tc is a chip time duration.

Figure 10 shows the use of the cyclic prefix for the case of two streams and single channel transmission. Figure 11 shows the use of the cyclic prefix for the case of two streams and two channel bonding transmission. This approach is applicable to any number of streams and channels.

- —Preamble structure with cyclic shift for two streams and one 2.16 GHz channel

- — Preamble structure with cyclic shift for two streams and one 4.32 GHz channel

        - L-Header definition

The structure of the L-Header field is defined as follows:

- For a control mode PPDU, the L-Header field is the same as the DMG control mode header field (see Table 20-11) with the following change: the reserved bits 22 and 23 shall be both set to 1. In this case, the Scrambler Initialization field in the L-Header is defined as shown in Table 4.

- For a EDMG SC mode PPDU or an EDMG OFDM mode PPDU, the L-Header field is the same as the DMG SC mode PHY header (see Table 20-17) with the following change: the reserved bit 46 shall be set to 1 to indicate the presence of the EDMG-Header-A field. This implies that the PPDU is an EDMG SC or OFDM PPDU, respectively.

- —Definition of Scrambler Initialization field when transmitted using the control mode

| Bit field |  |  |  | Definition |
| --- | --- | --- | --- | --- |
| B0 | B1 | B2 | B3 |  |
| - 0 | - 0 | - Reserved | - Reserved | - Indicates the presence of the control trailer (see 6.2.7). |
| - 0 | - 1 | - Reserved | - Reserved | - Indicates the presence of the EDMG-Header-A field. This implies that the PPDU is an EDMG control mode PPDU. |
| - 1 | - Channel BW |  |  | - When the PPDU contains an RTS or DMG CTS frame, the Channel BW field indicates the bandwidth of the PPDU. Otherwise, the Channel BW field is reserved. The Channel BW field is defined in Table 5, where N is the value of the lowest operating channel number of the BSS indicated by the PCP or AP in the EDMG Operation element. |

- —Channel BW field definition

| Desired channel bandwidth | Channel(s) making up desired channelization |  |  |  | Channel BW field value |
| --- | --- | --- | --- | --- | --- |
|  | N | N+1 | N+2 | N+3 |  |
| 2.16 GHz | X | - | - | - | 0 |
|  | - | X | - | - |  |
|  | - | - | X | - |  |
|  | - | - | - | X |  |
| 4.32 GHz | X | X | - | - | 1 (N is even) |
|  | - | - | X | X |  |
| 4.32 GHz | X | X | - | - | 2 (N is odd) |
|  | - | - | X | X |  |
| 6.48 GHz | X | X | X | - | 3 |
| 6.48 GHz | - | X | X | X | 4 |
| 8.64 GHz | X | X | X | X | 5 |
| “X” indicates channel is used “-” indicates channel is not used |  |  |  |  |  |

      - EDMG portion of EDMG format preamble

        - General

An EDMG STA shall be able to determine the primary channel and occupied bandwidth from any EDMG PPDU it receives.

        - EDMG-Header-A definition

          - General

The EDMG-Header-A field carries information required to interpret EDMG PPDUs. The structure of the EDMG-Header-A field for an EDMG SU PPDU is shown in Table 6. The structure of the EDMG-Header-A field for an EDMG MU PPDU is shown in Table 7. In both cases, The the EDMG-Header-A field is two DMG SC symbols (see clause 20) long comprising of 112 data bits followed by a 16 bit CRC.

- —EDMG-Header-A field structure and definition for a SU PPDU

| Field | Number of bits | Start bit | Description |
| --- | --- | --- | --- |
| SU/MU Format | 1 | 0 | Indicates whether the PPDU is a SU PPDU or a MU PPDU. Set to 0 to indicate a SU PPDU and set to 1 otherwise. |
| Channel Aggregation | 1 | 1 | Set to 0 for a 2.16 GHz, 4.32 GHz, 6.48 GHz or 8.64 GHz channel PPDU. Set to 1 for a 2.16+2.16 GHz or 4.32+4.32 GHz PPDU. |
| BW | 8 | 2 | Indicates the bandwidth of the PPDU. If a bit is set to 1, it indicates that the corresponding channel is used for the PPDU transmission. Bit 0 corresponds to channel 1, bit 1 corresponds to channel 2, and so on. |
| Primary Channel Number | 3 | 10 | Contains the 3 LSBs of the primary channel number of the BSS minus one |
| GI/CP Length | 2 | 13 |  |
| Beamformed | 1 | 15 | Set to 1 to indicate that channel estimate smoothing is recommended. Set to 0 otherwise. |
| Short/Long LDPC | 1 | 16 | Indicates the LDPC codeword length used in the PSDU. Set to 0 for LDPC codeword of length 672. Set to 1 for LDPC codeword of length 1344. |
| STBC Applied | 1 | 17 | If set to 1, indicates that STBC was applied at the transmitter. Otherwise, set to 0. |
| EDMG-MCS |  |  |  |
| PSDU Length | 22 |  | Length of the PSDU field in octets. |
| Number of SS | 3 |  | The value of this field plus one indicates the number of SSs transmitted in the PPDU. |
| Reserved |  |  | Set to 0 by the transmitter and ignored by the receiver. |
| CRC | 16 | 112 | Header Check sequence. Calculation of the header check sequence is defined in 20.3.7. |

- —EDMG-Header-A field structure and definition for a MU PPDU

| Field | Number of bits | Start bit | Description |
| --- | --- | --- | --- |
| SU/MU Format | 1 | 0 | See Table 6 |
| Channel Aggregation | 1 | 1 | See Table 6 |
| BW | 8 | 2 | See Table 6 |
| Primary Channel Number | 3 | 10 | See Table 6 |
| GI/CP Length | 2 | 13 |  |
| Beamformed | 1 | 15 | See Table 6 |
| Short/Long LDPC | 1 | 16 | See Table 6 |
| STBC Applied | 1 | 17 | See Table 6 |
| SS Descriptor Set 0 |  |  | Describes the SS assignment to the first STA addressed within the MU PPDU. This field is formatted as described in Table 8. |
| SS Descriptor Set 1 |  |  | Describes the SS assignment to the second STA addressed within the MU PPDU. This field is formatted as described in Table 8. |
| SS Descriptor Set 2 |  |  | Describes the SS assignment to the third STA addressed within the MU PPDU. This field is formatted as described in Table 8. |
| SS Descriptor Set 3 |  |  | Describes the SS assignment to the fourth STA addressed within the MU PPDU. This field is formatted as described in Table 8. |
| SS Descriptor Set 4 |  |  | Describes the SS assignment to the fifth STA addressed within the MU PPDU. This field is formatted as described in Table 8. |
| SS Descriptor Set 5 |  |  | Describes the SS assignment to the sixth STA addressed within the MU PPDU. This field is formatted as described in Table 8. |
| SS Descriptor Set 6 |  |  | Describes the SS assignment to the seventh STA addressed within the MU PPDU. This field is formatted as described in Table 8. |
| SS Descriptor Set 7 |  |  | Describes the SS assignment to the eight STA addressed within the MU PPDU. This field is formatted as described in Table 8. |
| Reserved |  |  | Set to 0 by the transmitter and ignored by the receiver. |
| CRC | 16 | 112 | Header Check sequence. Calculation of the header check sequence is defined in 20.3.7. |

The SS Descriptor Set field is defined in Table 8.

- —SS Descriptor Set field

|  | AID |  |
| --- | --- | --- |
| Bits | 8 |  |

The AID subfield indicates the AID of a STA addressed by an MPDU contained within the MU PPDU.

          - Transmission

For an EDMG SC mode PPDU or an EDMG OFDM mode PPDU, the EDMG-Header-A field is transmitted as follows:

- The CRC is the same CRC as that of the L-Header field (see 6.2.3.2.2).

- The first 64 bits and the second 64 bits are each encoded and modulated as specified in steps 1-5 in 20.6.3.1.4. Restricting to steps 1-5 means that the symbol is not duplicated to the second symbol as in the DMG PHY.

- GI is added after each SC block as in the DMG SC mode.

The EDMG-Header-A generation process is illustrated in Figure 12.

- —EDMG-Header-A generation

        - EDMG-STF definition

For EDMG SC mode PPDU transmissions, the EDMG-STF field is defined in 6.4.2. For EDMG OFDM mode PPDU transmissions, the EDMG-STF field is defined in 6.5.3.

        - EDMG-CEF definition

For EDMG SC mode PPDU transmissions, the EDMG-CEF field is defined in 6.4.3. For EDMG OFDM mode PPDU transmissions, the EDMG-CEF field is defined in 6.5.4.

        - EDMG-Header-B definition

          - General

The EDMG-Header-B is transmitted only in EDMG MU PPDUs and on a per STA basis. The EDMG-Header-B field is defined in Table 9.

- —EDMG-Header-B field structure and definition

| Field | Number of bits | Start bit | Description |
| --- | --- | --- | --- |
| Scrambler Seed | 7 | 0 |  |
| PSDU Length | 22 | 7 | Length of the PSDU field in octets. |
| EDMG-MCS | TBD | 29 |  |
| Reserved | 64-(29+TBD) | 29+TBD |  |

          - Transmission

The encoding of EDMG-Header-B in an EDMG SC mode PPDU is specified in 6.4.4. The encoding of EDMG-Header-B in an EDMG OFDM mode PPDU is specified in 6.5.5.

    - Channelization

The channelization used by EDMG STAs is shown in Figure 13.

- —Channelization used by EDMG STAs

When using 2.16 GHz channels, an EDMG STA uses channels defined in 20.3.1. Therefore, an EDMG STA supports channel number 2.

When using 4.32 GHz channels, an EDMG STA uses channels that have the following relationship between center frequency and channel number:

Channel center frequency = Channel starting frequency + (Channel spacing/2) × (Channel number mod 8)

+ 1.08 (GHz)

where channel starting frequency, channel spacing and channel number are as defined in Annex E.

When using 6.48 GHz channels, an EDMG STA uses channels that have the following relationship between center frequency and channel number:

Channel center frequency = Channel starting frequency + (Channel spacing/3) × (Channel number mod 16) 				+ 2.16 (GHz)

where channel starting frequency, channel spacing and channel number are as defined in Annex E.

When using 8.64 GHz channels, an EDMG STA uses channels that have the following relationship between center frequency and channel number:

Channel center frequency = Channel starting frequency + (Channel spacing/4) × (Channel number mod 24)

+ 3.24 (GHz)

where channel starting frequency, channel spacing and channel number are as defined in Annex E.

The circumstances in which a channel can be used in a regulatory domain is determined by local regulatory rules and any additional rules prescribed by the 11ay specification.

    - Transmit mask

    - LDPC parity matrices

      - General

An LDPC codeword of size equal to 1344 bits is defined through the use of lifting matrices. A lifting matrix acts on the code matrix to generate a larger matrix as follows:

- A ‘0’ in the lifting matrix acts on the Z×Z cyclic-permutation matrix Pi in the code matrix (at the same location) to create the 2Z×2Z matrix:

| i | -1 |
| --- | --- |
| -1 | i |

- A ‘1’ in the lifting matrix acts on the Z×Z cyclic-permutation matrix Pi in the code matrix (at the same location) to create the 2Z×2Z matrix:

| -1 | i |
| --- | --- |
| i | -1 |

- A ‘-1’ in the lifting matrix acts on the Z×Z zero matrix in the code matrix to create the 2Z×2Z zero matrix.

NOTE—The cyclic-permutation matrix Pi is defined in 20.3.8.

      - Rate-1/2 LDPC code matrix for generating 1344 bits codeword

The lifting matrix in Table 10 is applied to the rate-1/2 LDPC code matrix specified in Table 20-6.

- —Rate-1/2 lifting matrix

| 0 | -1 | 1 | -1 | 0 | -1 | 1 | -1 | 0 | -1 | -1 | -1 | -1 | -1 | -1 | -1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | -1 | 0 | -1 | 1 | -1 | -1 | 1 | 0 | 0 | -1 | -1 | -1 | -1 | -1 | -1 |
| -1 | 0 | -1 | 1 | -1 | 0 | -1 | 1 | -1 | 1 | 0 | -1 | -1 | -1 | -1 | -1 |
| -1 | 1 | -1 | 1 | -1 | 1 | 0 | -1 | -1 | -1 | 0 | 0 | -1 | -1 | -1 | -1 |
| 0 | -1 | 1 | -1 | 1 | -1 | 0 | -1 | 0 | -1 | -1 | 1 | 0 | -1 | -1 | -1 |
| 1 | -1 | 1 | -1 | -1 | 1 | -1 | 0 | -1 | 1 | -1 | 1 | -1 | 0 | -1 | -1 |
| -1 | 0 | -1 | 0 | -1 | 1 | -1 | 0 | -1 | -1 | 0 | -1 | -1 | 1 | 0 | -1 |
| -1 | 0 | -1 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | -1 | -1 | 1 | -1 | 0 | 0 |

      - Rate-5/8 LDPC code matrix for generating 1344 bits codeword

The lifting matrix in Table 11 is applied to the rate-5/8 LDPC code matrix specified in Table 20-7.

- —Rate-5/8 lifting matrix

| 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | -1 | 1 | 0 | -1 | -1 | -1 | -1 | -1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | -1 | 1 | -1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | -1 | -1 | -1 | -1 |
| 0 | -1 | 1 | -1 | 1 | -1 | 0 | -1 | 0 | -1 | -1 | 1 | 0 | -1 | -1 | -1 |
| 1 | -1 | 1 | -1 | -1 | 1 | -1 | 0 | -1 | 1 | -1 | 1 | 1 | 0 | -1 | -1 |
| -1 | 0 | -1 | 0 | -1 | 1 | -1 | 0 | -1 | 0 | 0 | -1 | -1 | 1 | 0 | -1 |
| -1 | 0 | -1 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | -1 | -1 | -1 | -1 | 0 | 0 |

      - Rate-3/4 LDPC code matrix for generating 1344 bits codeword

The lifting matrix in Table 12 is applied to the rate-3/4 LDPC code matrix specified in Table 20-8.

- —Rate-3/4 lifting matrix

| 0 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | -1 | -1 | -1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | -1 | -1 |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | -1 | 1 | 0 | -1 |
| 1 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | -1 | 1 | 0 | 1 | 0 | 0 | 0 |

      - Rate-13/16 LDPC code matrix for generating 1344 bits codeword

The lifting matrix in Table 13 is applied to the rate-13/16 LDPC code matrix specified in Table 20-9.

- —Rate-13/16 lifting matrix

| 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | -1 | -1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | -1 |
| 1 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 0 |

    - Control trailer

A control trailer may be inserted in a control mode PPDU immediately following the Data field of the PPDU. The control trailer is one LDPC codeword with 18 data octets and 21 parity octets. The control trailer is added after the Data field of a control mode PPDU, using and continuing the control mode modulation and encoding.

To indicate the presence of a control trailer in a control mode PPDU, the Scrambler Initialization field in the L-Header shall be set as indicated in Table 4 and the value of the Training Length field in the L-Header shall be set to a value greater than or equal to 2. If the value of the Training Length field is equal to 2, the control trailer takes the place of the AGC and TRN fields following the Data field. If the value of the Training Length field is greater than 2, the AGC and TRN fields are appended after the control trailer. The format of the control trailer is indicated in Table 14.

- —Control trailer definition

| Field | Number of bits | Start bit | Description |
| --- | --- | --- | --- |
| Channel Aggregation | 1 | 1 | Set to 0 for a channel bonding or single 2.16 GHz channel PPDU. Set to 1 for a channel aggregation PPDU. |
| BW | 8 | 2 | Indicates the bandwidth of the PPDU. If a bit is set to 1, it indicates that the corresponding channel is used for the PPDU transmission. Bit 0 corresponds to channel 1, bit 1 corresponds to channel 2, and so on. |
| Primary Channel Number | 3 | 10 | Contains the 3 LSBs of the primary channel number of the BSS minus one |
| CTCS | 16 | 13 | Contains the CRC-16 computed over the content of the control trailer. This field is computed as defined in section 20.3.7 |
| Reserved | 116 | 29 | Set to 0 by the transmitter and ignored by the receiver. |

  - EDMG control PHY

  - EDMG SC PHY

    - Introduction

Transmission and reception of EDMG SC mode PPDUs by EDMG STAs is mandatory for selected MCSs.

    - EDMG-STF definition

      - General

The structure of the EDMG-STF field depends on the number of contiguous 2.16 GHz channels over which an EDMG PPDU is transmitted and the index i of the ith spatial stream.

For single spatial stream EDMG PPDU transmissions using the EDMG SC mode over a single 2.16 GHz channel, the EDMG-STF field is not present.

For EDMG SC transmissions, the EDMG-STF field shall be modulated using π/2-BPSK.

      - Definition

For EDMG SC transmissions using a single 2.16 GHz channel, the EDMG-STF field is comprised of multiple (TBD) repetitions of the sequence shown in Error: Reference source not found.

- —EDMG-STF sequence for single 2.16 GHz channel

For EDMG SC transmissions over a 4.32 GHz channel, the EDMG-STF field is comprised of multiple (TBD) repetitions of the sequence shown in Error: Reference source not found.

- —EDMG-STF sequence for a 4.32 GHz channel

For EDMG SC transmissions over a 8.64 GHz channel, the EDMG-STF field is comprised of multiple (TBD) repetitions of the sequence shown in Error: Reference source not found.

- —EDMG-STF sequence for a 8.64 GHz channel

    - EDMG-CEF definition

      - General

The structure of the EDMG-CEF field depends on the number of contiguous 2.16 GHz channels over which an EDMG PPDU is transmitted and the number, i, of spatial streams.

For single spatial stream EDMG PPDU transmissions using the EDMG SC mode over a single 2.16 GHz channel, the EDMG-CEF field is not present.

For EDMG SC transmissions, the EDMG-CEF field shall be modulated using π/2-BPSK.

The pairs of Golay complementary sequences (Gai128, Gbi128), (Gai256, Gbi256), (Gai384, Gbi384) and (Gai512, Gbi512) used in the definition of the EDMG-CEF field for different spatial streams are specified in section Error: Reference source not found.

      - Single spatial stream (i = 1)

For single spatial stream EDMG PPDU transmissions using the EDMG SC mode over a 4.32 GHz channel, the EDMG-CEF field is defined in Error: Reference source not found, where:

- Gui1024 = [-Gbi256 –Gai256 Gbi256 –Gai256], for i=1

- Gvi1024 = [-Gbi256 Gai256 –Gbi256 –Gai256], for i=1

- —EDMG-CEF field for a 4.32 GHz channel and single spatial stream

For single spatial stream EDMG PPDU transmissions using the EDMG SC mode over a 6.48 GHz channel, the EDMG-CEF field is defined in Error: Reference source not found, where:

- Gui1536 = [-Gbi384, -Gai384, +Gbi384, -Gai384], for i=1

- Gvi1536 = [-Gbi384, +Gai384, -Gbi384, -Gai384], for i=1

- —EDMG-CEF field for a 6.48 GHz channel and single spatial stream

For single spatial stream EDMG PPDU transmissions using the EDMG SC mode over a 8.64 GHz channel, the EDMG-CEF field is defined in Error: Reference source not found, where:

- Gui2048 = [-Gbi512 –Gai512 Gbi512 –Gai512], for i=1

- Gvi2048 = [-Gbi512 Gai512 –Gbi512 –Gai512], for i=1

- —EDMG-CEF field for a 8.64 GHz channel and single spatial stream

      - Two spatial streams (i = 2)

For two spatial streams EDMG PPDU transmissions using the EDMG SC mode over a single 2.16 GHz channel, the EDMG-CEF field is defined in Error: Reference source not found where:

- Gui512 = [-Gbi128 –Gai128 Gbi128 –Gai128], for i = 1, 2

- Gvi512 = [-Gbi128 Gai128 –Gbi128 –Gai128], for i = 1, 2

- —EDMG-CEF field for single 2.16 GHz channel and two spatial streams

For two spatial streams EDMG PPDU transmissions using the EDMG SC mode over a 4.32 GHz channel, the EDMG-CEF field is defined in Error: Reference source not found where:

- Gui1024 = [-Gbi256 –Gai256 Gbi256 –Gai256], for i = 1, 2

- Gvi1024 = [-Gbi256 Gai256 –Gbi256 –Gai256], for i = 1, 2

- —EDMG-CEF field for a 4.32 GHz channel and two spatial streams

For two spatial streams EDMG PPDU transmissions using the EDMG SC mode over a 6.48 GHz channel, the EDMG-CEF field is defined in Error: Reference source not found, where:

- Gui1536 = [-Gbi384, -Gai384, +Gbi384, -Gai384], for i=1, 2

- Gvi1536 = [-Gbi384, +Gai384, -Gbi384, -Gai384], for i=1, 2

- —EDMG-CEF field for 6.48 GHz channel and two spatial streams

For two spatial streams EDMG PPDU transmissions using the EDMG SC mode over a 8.64 GHz channel, the EDMG-CEF field is defined in Error: Reference source not found where:

- Gui2048 = [-Gbi512 –Gai512 Gbi512 –Gai512], for i = 1, 2

- Gvi2048 = [-Gbi512 Gai512 –Gbi512 –Gai512], for i = 1, 2

- —EDMG-CEF field for a 8.64 GHz channel and two spatial streams

      - Three and four spatial streams (i = 3, 4)

For three and four spatial streams EDMG PPDU transmissions using the EDMG SC mode over a single 2.16 GHz channel, the EDMG-CEF field is defined in Error: Reference source not found where:

- Gui512 = [-Gbi128 –Gai128 Gbi128 –Gai128], for i = 1, 2, 3, 4

- Gvi512 = [-Gbi128 Gai128 –Gbi128 –Gai128], for i = 1, 2, 3, 4

- —EDMG-CEF field for single 2.16 GHz channel, and three and four spatial streams

For three and four spatial streams EDMG PPDU transmissions using the EDMG SC mode over a 4.32 GHz channel, the EDMG-CEF field is defined in Error: Reference source not found where:

- Gui1024 = [-Gbi256 –Gai256 Gbi256 –Gai256], for i = 1, 2, 3, 4

- Gvi1024 = [-Gbi256 Gai256 –Gbi256 –Gai256], for i = 1, 2, 3, 4

- —EDMG-CEF field for a 4.32 GHz channel, and three and four spatial streams

For three and four spatial streams EDMG PPDU transmissions using the EDMG SC mode over a 6.48 GHz channel, the EDMG-CEF field is defined in Error: Reference source not found, where:

- Gui1536 = [-Gbi384, -Gai384, +Gbi384, -Gai384], for i=1, 2, 3, 4

- Gvi1536 = [-Gbi384, +Gai384, -Gbi384, -Gai384], for i=1, 2, 3, 4

- —EDMG-CEF field for 6.48 GHz channel, and three and four spatial streams

For three and four spatial streams EDMG PPDU transmissions using the EDMG SC mode over a 8.64 GHz channel, the EDMG-CEF field is defined in Error: Reference source not found where:

- Gui2048 = [-Gbi512 –Gai512 Gbi512 –Gai512], for i = 1, 2, 3, 4

- Gvi2048 = [-Gbi512 Gai512 –Gbi512 –Gai512], for i = 1, 2, 3, 4

- —EDMG-CEF field for a 8.64 GHz channel, and three and four spatial streams

In case of a three spatial stream EDMG PPDU transmission, the signal corresponding for the fourth (i = 4) stream shall not be transmitted.

      - Five and greater spatial streams (i = 5, 6, 7, 8)

For five, six, seven and eight spatial streams EDMG PPDU transmissions using the EDMG SC mode over a single 2.16 GHz channel, the EDMG-CEF field is defined in Error: Reference source not found where:

- Gui512 = [-Gbi128 –Gai128 Gbi128 –Gai128], for i = 1, 2, 3, 4, 5, 6, 7, 8

- Gvi512 = [-Gbi128 Gai128 –Gbi128 –Gai128], for i = 1, 2, 3, 4, 5, 6, 7, 8

- —EDMG-CEF field for single 2.16 GHz channel, and five, six, seven and eight spatial streams

For five, six, seven and eight spatial streams EDMG PPDU transmissions using the EDMG SC mode over a 4.32 GHz channel, the EDMG-CEF field is defined in Error: Reference source not found where:

- Gui1024 = [-Gbi256 –Gai256 Gbi256 –Gai256], for i = 1, 2, 3, 4, 5, 6, 7, 8

- Gvi1024 = [-Gbi256 Gai256 –Gbi256 –Gai256], for i = 1, 2, 3, 4, 5, 6, 7, 8

- —EDMG-CEF field for a 4.32 GHz channel, and five, six, seven and eight spatial streams

For five, six, seven and eight spatial streams EDMG PPDU transmissions using the EDMG SC mode over a 6.48 GHz channel, the EDMG-CEF field is defined in, where:

- Gui1536 = [-Gbi384, -Gai384, +Gbi384, -Gai384], for i=1, 2, 3, 4, 5, 6, 7, 8

- Gvi1536 = [-Gbi384, +Gai384, -Gbi384, -Gai384], for i=1, 2, 3, 4, 5, 6, 7, 8

- —EDMG-CEF field for a 6.48 GHz channel, and five, six, seven and eight spatial streams

For five, six, seven and eight spatial streams EDMG PPDU transmissions using the EDMG SC mode over a 8.64 GHz channel, the EDMG-CEF field is defined in Error: Reference source not found where:

- Gui2048 = [-Gbi512 –Gai512 Gbi512 –Gai512], for i = 1, 2, 3, 4, 5, 6, 7, 8

- Gvi2048 = [-Gbi512 Gai512 –Gbi512 –Gai512], for i = 1, 2, 3, 4, 5, 6, 7, 8

- —EDMG-CEF field for a 8.64 GHz channel, and five, six, seven and eight spatial streams

In case of a five, six, and seven spatial stream EDMG PPDU transmission, only the signals corresponding to the appropriate number of streams shall be transmitted and the remaining signals shall not be transmitted.

    - Encoding of EDMG-Header-B

The EDMG-Header-B shall be encoded as follows:

- Definitions

- Let b = (b1, b2, …, b64), where b1, b2, …, b64 represent the input bits from the EDMG-Header-B

- Let q = (01, 02, …, 07, s1, s2, …, s57), where s1, s2, …, s57 represent the first 57 bits at the output of the scrambler with initial seed value (b1, b2, …, b7)

- Let:

si=(01,02,...,0224,s1,s2,...,s224) for i=1(s(i−1)×448−224+1,s(i−1)×448−224+2,...,s(i−1)×448+224) for i>1

- where:

- s1, s2, …, sN represent the output of the scrambler with initial seed value (11, 12, …, 17)

- i represents the number of space time streams

- Steps

- XOR each corresponding bit of vector b and vector q to generate vector bq

- Given an LDPC codeword (bq1, bq2, …, bq64, 01, 02, …, 0440, p1, p2, …, p168) generated as defined in 20.3.8, generate c = (c1, c2) as:

- c1 = (bq1, bq2, …, bq64, p1, p2, …, p160)

- c2 = (bq1, bq2, …, bq64, p1, p2, …, p152,  p161, p162, …, p168)

- Generate one codeword, csi, for each space time stream i belonging to a STA addressed in the PPDU by XORing each corresponding bit of vector c and vector si to generate csi

Following encoding, the EDMG-Header-B shall be modulated as follows:

- Each codeword csi is modulated using π/2-BPSK modulation. This creates a SC symbol block, blki, of length 448 chips for the ith stream

- Each block blki is prepended with a guard interval which shall be a π/2 rotated Golay sequence. The length of the Golay sequence depends on the bandwidth of the PPDU. For a PPDU transmitted over a NCB × 2.16 GHz channel, where 1 ≤ NCB ≤ 4, the resulting SC symbol block for the ith space time stream is defined as:

- [Gai64, blki] for NCB = 1

- [Gai128, blki, blki] for NCB = 2

- [Gai192, blki, blki, blki] for NCB = 3

- [Gai256, blki, blki, blki, blki] for NCB = 4

  - EDMG OFDM PHY

    - Phase hopping

When using the EDMG OFDM mode, the 11ay specification shall define SU EDMG PPDU transmission with and without phase hopping. An EDMG STA may support transmission and reception of SU EDMG PPDU with phase hopping. In this mechanism, the following apply:

- Precoding may be used and

- The period of phase hopping is TBD

    - Modulation

For OFDM MIMO transmissions with two spatial streams, the 11ay specification shall enable space-frequency diversity scheme using SQPSK DCM modulation with subcarriers mapping shown in Figure 32.

- —Subcarrier mapping of SQPSK DCM

    - EDMG-STF definition

    - EDMG-CEF definition

    - Encoding of EDMG-Header-B

  - EDMG Beamforming

    - Short SSW packet

The format of the Short SSW packet is shown in Figure 33 and its fields are defined in Table 15.

|  | Packet Type | Addressing | CDOWN | RF Chain ID | Short SSW Feedback | Direction | Reserved | FCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bits | 2 | 16 | 11 | 2 | 11 | 1 | 1 | 4 |

- - Short SSW packet format

- - Short SSW field definition

| Field | Definition |
| --- | --- |
| Packet Type | Indicates the type of the packet. Possible values: 0: Short SSW 1-3: Reserved |
| Addressing | The format is TBD. |
| CDOWN | A down-counter indicating the number of remaining Short SSW packet transmissions and LBIFSs to the end of the TXSS/RXSS across all antennas. This field is set to 0 in the last Short SSW packet transmission. |
| RF Chain ID | Identifies the RF chain the transmitter is currently using for this transmission. |
| Short SSW Feedback | In a RSS, contains the value of the CDOWN field of the Short SSW packet that was received with best quality in the immediately preceding sector sweep. The determination of which packet was received with best quality is implementation dependent. This field is reserved when transmitted as part of an ISS. |
| Direction | Indicates the direction of the transmission. The Direction field is set to 0 to indicate that the frame is transmitted by the beamforming initiator and set to 1 to indicate that the frame is transmitted by the beamforming responder. |
| FCS | The four MSBs of the FCS |

  - Golay sequences

    - General

The EDMG PHY uses the pairs of complementary sequences defined in 20.11 and defines new sequences. The sequences defined in 20.11 are used in the L-STF, L-CEF, EDMG-STF, EDMG-CEF and in the guard intervals prior to the start of the EDMG-STF. The new sequences are used in the guard intervals of the Data field of an EDMG PPDU transmitted in the following modes or combinations thereof: SU, MU, one or more streams, and in 2.16 GHz, 4.32 GHz, 6.48 GHz and 8.64 GHz channels.

    - Sequences of length 128, 256 and 512

Golay sequences of length 128, 256 and 512 are generated using the procedure and notation specified in 20.11.

The value of the DK vector for each of the EDMG sequences are as follows:

- For Gai128 and Gbi128: DK = [1 8 2 4 16 32 64]

- For Gai256 and Gbi256: DK = [1 8 2 4 16 32 64 128]

- For Gai512 and Gbi512: DK = [1 8 2 4 16 32 64 128 256]

As opposed to the DK vector, the value of the WK vector depends on the spatial stream number used to define the Golay pair (Gai, Gbi) for ith stream. Table 16 shows the value of the WK vector defined for each spatial stream and corresponding sequence length.

- —WK vector value to generate Golay sequences

| Spatial stream number | WK vector for 128 sequence length | WK vector for 256 sequence length | WK vector for 512 sequence length |
| --- | --- | --- | --- |
| 1 | [-1,-1,-1,-1,+1,-1,-1] | [-1,-1,-1,-1,+1,-1,-1,+1] | [-1,-1,-1,-1,+1,-1,-1,+1,+1] |
| 2 | [+1,-1,-1,-1,+1,-1,-1] | [+1,-1,-1,-1,+1,-1,-1,+1] | [+1,-1,-1,-1,+1,-1,-1,+1,+1] |
| 3 | [-1,-1,-1,+1,-1,-1,+1] | [-1, -1, -1,+1,-1,-1,+1,-1] | [-1,-1,-1,-1,-1,-1,-1,-1,-1] |
| 4 | [+1,-1,-1,+1,-1,-1,+1] | [+1, -1, -1,+1,-1,-1,+1,-1] | [+1,-1,-1,-1,-1,-1,-1,-1,-1] |
| 5 | [-1,-1,-1,+1,-1,+1,+1] | [-1,-1,-1,+1,-1,+1,+1,-1] | [-1,-1,-1,-1,-1,+1,-1,-1,-1] |
| 6 | [+1,-1,-1,+1,-1,+1,+1] | [+1,-1,-1,+1,-1,+1,+1,-1] | [+1,-1,-1,-1,-1,+1,-1,-1,-1] |
| 7 | [-1,-1,-1,+1,+1,+1,-1] | [-1,-1,-1,+1,+1,+1,-1,-1] | [-1,-1,-1,-1,+1,-1,-1,-1,+1] |
| 8 | [+1,-1,-1,+1,+1,+1,-1] | [+1,-1,-1,+1,+1,+1,-1,-1] | [+1,-1,-1,-1,+1,-1,-1,-1,+1] |

    - Sequences of length 96, 192 and 384

The Golay sequences of length 96, 192, and 384 use a quadri-phase complex Golay complementary pair and are generated using the following recursive procedure, where i is the index of the stream number:

- Ga3 = [+1, +1, -1]

- Gb3 = [+1, +j, +1]

- If (i = 1, 3, 5 or 7) then (A0(n), B0(n)) = (+Ga3(2-n), +Gb3(2-n)). Otherwise if (i = 2, 4, 6 or 8) then (A0(n), B0(n)) = (+conj(Gb3(n)), -conj(Ga3(n)))

- Ak(n) = WkAk-1(n) + Bk-1(n-Dk)

- Bk(n) = WkAk-1(n) - Bk-1(n-Dk)

Starting with n = 3 and making 5, 6, and 7 iterations, corresponding sequences of length 96, 192, and 384 are obtained.

The value of the DK vector for each of the sequences are defined as follows, where i is the stream number and 1 ≤ i ≤ 8:

- For Gai96 and Gbi96: DK = [3 24 6 12 48]

- For Gai192 and Gbi192: DK = [3 24 6 12 48 96]

- For Gai384 and Gbi384: DK = [3 24 6 12 48 96 192]

As opposed to the DK vector, the value of the WK vector depends on the spatial stream number used to define the Golay pair (GaiN, GbiN) for ith stream. Table 17 shows the value of the WK vector defined for each spatial stream.

- —WK vector value to generate Golay sequences

| Spatial stream number | WK for Gai96 and Gbi96 | WK for Gai192 and Gbi192 | WK for Gai384 and Gbi384 |
| --- | --- | --- | --- |
| 1 | [-1,-1,-1,-1,+1] | [-1,-1,-1,-1,+1,+1] | [-1,-1,-1,-1,+1,-1,-1] |
| 2 |  |  |  |
| 3 | [-1,-1,-1,+1,-1] | [-1,-1,-1,+1,-1,+1] | [-1,-1,-1,+1,-1,-1,+1] |
| 4 |  |  |  |
| 5 | [-1,-1,+1,-1,-1] | [-1,-1,+1,-1,-1,+1] | [-1,-1,-1,+1,-1,+1,+1] |
| 6 |  |  |  |
| 7 | [-1,-1,+1,+1,-1] | [-1,-1,+1,+1,-1,+1] | [-1,-1,-1,+1,+1,+1,-1] |
| 8 |  |  |  |

The sequences are defined from Table 18 through Table 23. The sequences in the tables are normative, the description above is informative.

- —The sequence Gai96(n)

The Sequence Ga196(n), to be transmitted from left to right, up to down

+1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1

The Sequence Ga296(n), to be transmitted from left to right, up to down

+1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1

The Sequence Ga396(n), to be transmitted from left to right, up to down

+1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1

The Sequence Ga496(n), to be transmitted from left to right, up to down

+1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1

The Sequence Ga596(n), to be transmitted from left to right, up to down

+1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1

The Sequence Ga696(n), to be transmitted from left to right, up to down

+1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1

The Sequence Ga796(n), to be transmitted from left to right, up to down

+1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1

The Sequence Ga896(n), to be transmitted from left to right, up to down

+1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1

- —The sequence Gbi96(n)

The Sequence Gb196(n), to be transmitted from left to right, up to down

-1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1

The Sequence Gb296(n), to be transmitted from left to right, up to down

-1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1

The Sequence Gb396(n), to be transmitted from left to right, up to down

-1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1

The Sequence Gb496(n), to be transmitted from left to right, up to down

-1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1

The Sequence Gb596(n), to be transmitted from left to right, up to down

-1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1

The Sequence Gb696(n), to be transmitted from left to right, up to down

-1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1

The Sequence Gb796(n), to be transmitted from left to right, up to down

-1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1

The Sequence Gb896(n), to be transmitted from left to right, up to down

-1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1

- —The sequence Gai192(n)

The Sequence Ga1192(n), to be transmitted from left to right, up to down

-1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1

The Sequence Ga2192(n), to be transmitted from left to right, up to down

-1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1

The Sequence Ga3192(n), to be transmitted from left to right, up to down

-1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1

The Sequence Ga4192(n), to be transmitted from left to right, up to down

-1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1

The Sequence Ga5192(n), to be transmitted from left to right, up to down

-1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1

The Sequence Ga6192(n), to be transmitted from left to right, up to down

-1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1

The Sequence Ga7192(n), to be transmitted from left to right, up to down

-1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1

The Sequence Ga8192(n), to be transmitted from left to right, up to down

-1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1

- —The sequence Gbi192(n)

The Sequence Gb1192(n), to be transmitted from left to right, up to down

+1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1

The Sequence Gb2192(n), to be transmitted from left to right, up to down

+1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1

The Sequence Gb3192(n), to be transmitted from left to right, up to down

+1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1

The Sequence Gb4192(n), to be transmitted from left to right, up to down

+1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1

The Sequence Gb5192(n), to be transmitted from left to right, up to down

+1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1

The Sequence Gb6192(n), to be transmitted from left to right, up to down

+1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1

The Sequence Gb7192(n), to be transmitted from left to right, up to down

+1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1

The Sequence Gb8192(n), to be transmitted from left to right, up to down

+1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1

- —The sequence Gai384(n)

The Sequence Ga1384(n), to be transmitted from left to right, up to down

+1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1

The Sequence Ga2384(n), to be transmitted from left to right, up to down

+1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1

The Sequence Ga3384(n), to be transmitted from left to right, up to down

+1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1

The Sequence Ga4384(n), to be transmitted from left to right, up to down

+1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1

The Sequence Ga5384(n), to be transmitted from left to right, up to down

+1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1

The Sequence Ga6384(n), to be transmitted from left to right, up to down

+1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1

The Sequence Ga7384(n), to be transmitted from left to right, up to down

+1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1

The Sequence Ga8384(n), to be transmitted from left to right, up to down

+1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1

- —The sequence Gbi384(n)

The Sequence Gb1384(n), to be transmitted from left to right, up to down

-1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1

The Sequence Gb2384(n), to be transmitted from left to right, up to down

-1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1

The Sequence Gb3384(n), to be transmitted from left to right, up to down

-1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1

The Sequence Gb4384(n), to be transmitted from left to right, up to down

-1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1

The Sequence Gb5384(n), to be transmitted from left to right, up to down

-1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1

The Sequence Gb6384(n), to be transmitted from left to right, up to down

-1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1

The Sequence Gb7384(n), to be transmitted from left to right, up to down

-1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 -1 -1 +1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1 +1 -j +1 +1 +1 -1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 -1 -1 +1 -1 +j -1 +1 +1 -1 -1 +j -1 +1 +1 -1 +1 -j +1 -1 -1 +1 -1 +j -1 +1 +1 -1

The Sequence Gb8384(n), to be transmitted from left to right, up to down

-1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 -1 -j -1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1 +1 -1 -1 +1 +j +1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 -1 -j -1 -1 +1 +1 +1 +j +1 -1 +1 +1 +1 +j +1 +1 -1 -1 -1 -j -1 -1 +1 +1 +1 +j +1

# (normative)
Country elements and operating classes

| 3538-127 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |
| --- | --- | --- | --- | --- | --- | --- |

Change Table E-1 as follows

| 19 | 181 | 56.16 | 4320 | 9-11 | - | - |
| --- | --- | --- | --- | --- | --- | --- |
| 20 | 182 | 56.16 | 6480 | 17-18 | - | - |
| 21 | 183 | 56.16 | 8640 | 25 |  |  |
| 1922-127 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |

Change Table E-2 as follows

| 60 | 181 | 56.16 | 4320 | 9-11 | - | - |
| --- | --- | --- | --- | --- | --- | --- |
| 61 | 182 | 56.16 | 6480 | 17-18 | - | - |
| 62 | 183 | 56.16 | 8640 | 25 |  |  |
| 6063-127 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |

Change Table E-3 as follows

| 181 | E-1-35, E-2-19, E-3-60 | 56.16 | 4320 | 9-13 | - | - |
| --- | --- | --- | --- | --- | --- | --- |
| 182 | E-1-36, E-2-20, E-3-61 | 56.16 | 6480 | 17-20 | - | - |
| 183 | E-1-37, E-2-21, E-3-62 | 56.16 | 8640 | 25-27 |  |  |
| 181184-191 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |

Change Table E-4 as follows