IEEE P802.11
Wireless LANs

| Solicited PAD procedure update |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2017-01-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Stephen McCann | BlackBerry Ltd | The Pearce Building, West Street, Maidenhead, Berkshire, SL6 1RL, UK | +44 1753 667099 | smccann@blackberry.com |

Abstract

This document proposes text changes to update the P802.11aq D7.0 for the following SBO CIDs:

6114, 6146, 6184

9.4.2.27 Extended Capabilities element

Insert a new row at the end of the table as follows:

Table 9-135—Extended Capabilities field

| Bit | Information | Notes |
| --- | --- | --- |
| 75 | Solicited PAD (perhaps just PAD??) | Indicates support for solicited PAD procedure (see 11.25a.3 (Solicited PAD procedure)). |

## 11. MLME

#### 11.25 WLAN interworking with external networks procedures

##### 11.25.3 Interworking procedures: generic advertisement service (GAS)

11.25.3.2 ANQP procedures

Insert new subclauses as follows:

11.25a Pre-association discovery (PAD) procedures

11.25a.1 General

There are two types of PAD procedures: unsolicited and solicited. The unsolicited PAD procedure is described in 11.25a.2 (Unsolicited PAD procedure) and the solicited PAD procedure is described in 11.25a.3 (Solicited PAD procedure).

When dot11UnsolicitedPADActivated or dot11SolicitedPADActivated is true, a non-AP and non-PCP STA may use PAD procedures to discover the availability of services that the same non-AP and non-PCP STA may access when associated. While the specification of service specific information is outside the scope of this standard, the service specific information in the BSS is proxied by a SIR (see 4.5.9.1.3 (Service information registry)), which might be collocated with the AP or PCP.

##### 11.25a.2 Unsolicited PAD procedure

When dot11UnsolicitedPADActivated is true, an AP shall advertise services from an SIR, using a Service Hint element or Service Hash element or both in Beacon or Probe Response frames. Each service shall be advertised using either Service Hint element or Service Hash element or both in Beacon or Probe Response frames. When dot11UnsolicitedPADActivated is true, a PCP shall advertise services from an SIR, using a Service Hint element or Service Hash element or both in DMG Beacon or Announce frames. A Service Hint element is used to advertise the presence of one or more services, offered by a BSS or an external network reachable via that BSS, with a probability of matching a wrong service as indicated in the False

Positive Probability Range field of the Service Hint element. A Service Hash element is used to advertise the presence of one or more services, offered by a BSS or an external network reachable via that BSS, with a negligible probability of matching a wrong service. The use of a Service Hash or Service Hint element, by an AP or PCP, to advertise a particular service is beyond the scope of this standard.

When dot11UnsolicitedPADActivated is true, a non-AP and non-PCP STA searching for a service shall perform the following steps:

    - Determine if a Service Hash, Service Hint element or both are included in received Beacon or Probe Response frames. The presence of these elements indicates that services are offered by a BSS or an external network reachable via that BSS.

    - Construct a service hash value for each service being searched or determine the bit positions in the Bloom Filter Array field that will be set to 1 for the services being serached.

    - For each service being searched, determine if there is a matched service within the received elements, based on either of the following:

      - The service hash value in the Service Hashes field of the Service Hash element matches to the corresponding service hash value constructed in step 2.

      - The values of the bit positions of the Bloom Filter Bit Array field of the Service Hint ele- ment, as determined in step 2, are all equal to 1.

The non-AP and non-PCP STA may use the information on which services are available, to determine how to proceed. The non-AP and non-PCP STA might proceed with solicited PAD procedure (see 11.25a.3 (Solicited PAD procedure)) or authentication and association procedure (see 11.3 (STA authentication and association)) based on the perceived false positive probability and the nature of the service (see the examples in W.1 (Pre-association discovery usage scenarios)).

##### 11.25a.3 Solicited PAD procedure

When dot11SolicitedPADActivated is true, a non-AP and non-PCP STA may transmit to an AP or PCP either a Service Hash Request ANQP-element or a Service Information Request ANQP-element, to request information from the SIR about services reachable via the BSS or an external network reachable via that BSS. An AP or PCP advertises support for the Solicited PAD procedure by setting the Solicited PAD field of the Extended Capabilities element to 1 in its Beacon and Probe Response frames.

11.25a.3.1. Service Hash Request and Response

The Service Hash Request ANQP-element includes one or more service hashes generated from the service name(s) of the service(s) that the non-AP and non-PCP STA is searching, as well as valid combinations of services of interest.

When dot11SolicitedPADActivated is true, a non-AP and non-PCP STA may send a Service Hash Request ANQP-element (see 9.4.5.27 (Service Hash Request ANQP-element)) to an AP or PCP, to obtain information about a matching service from the SIR.

When dot11SolicitedPADActivated is true, an SIR shall use the information from the Service Hash Request ANQP-element (that it receives from a non-AP and non-PCP STA) to determine if the BSS or an external network reachable via that BSS, can provide the requested service(s) or combination of services.

Determination is based on the service hash values in the Service Hashes field of the received Service Hash Request ANQP-element, and valid service combinations specified through the Flags and Service Combination fields of the Service Hash Request ANQP-element. If the SIR determines that the BSS or an external network reachable via that BSS, can provide the requested service(s) or combination of services, it shall respond by requesting the AP or PCP to transmit a Service Hash Response ANQP-element that contains a Service Information Response Tuple subfield for each service that satisfies the request.

NOTE—For example, an AP or PCP that receives a Service Hash Request ANQP-element frame that includes hash val- ues for 4 services S1, S2, S3 and S4 (in that order) and a value of 0xFEEE in its Service Combination field, responds to the request if and only if it can provide service S1 or service S2 or both services S3 and S4. The Service Hash Response ANQP-element can contain Service Information Response Tuple subfields for any set of available services that satisfy the ANQP request, e.g., S1, S3 and S4.

The requesting non-AP and non-PCP STA shall process the Service Hash Response ANQP-element in the received ANQP response to select a service combination that satisfies the non-AP and non-PCP STA request.

If there is a matching service name, the non-AP and non-PCP STA might decide to proceed with a service information request (see 11.25a.3.2 (Service information request) or authentication and association procedure (see 11.3 (STA authentication and association)) depending on the nature of the service (see examples illustrated in W.1 (Pre-association discovery usage scenarios)).

11.25a.3.2. Service Information Request and Response

The Service Information Request ANQP-element is used by a non-AP and non-PCP STA to request more detailed information about services reachable through the BSS or an external network reachable via that BSS , to associated STAs. A service name or a service hash generated from the service name is placed within the ANQP request that is used to identify a service, see examples illustrated in W.1 (Pre-association discovery usage scenarios).

The Service Information Response ANQP-element contains detailed information about the services con- tained within the SIR.

When dot11SolicitedPADActivated is true, a non-AP and non-PCP STA may send a Service Information Request ANQP-element (see 9.4.5.28 (Service Information Request ANQP-element)) to obtain information about a matching service from the SIR.  The Service Information Request ANQP-element may include one or more Service Information Request Tuple subfields. Each Service Information Request Tuple subfield shall include either a service name or service hash within the Service Name subfield, and if applicable an Instance Name subfield, and may include a Service Information Query Request subfield that is service specific.

When dot11SolicitedPADActivated is true, an SIR shall use the information from the Service Information Request ANQP-element (that it receives from a non-AP and non-PCP STA) to determine if the BSS or an external network reachable via that BSS, can provide the requested service(s) or combination of services.

Determination is based on the Service Information Request Tuple subfields of the received Service Information Request ANQP-element. If the SIR determines that the BSS or an external network reachable via that BSS, can provide the requested service(s), it shall respond by requesting the AP or PCP to transmit a Service Information Response ANQP-element that contains a Service Information Response Tuple subfield for each service that satisfies the request.

Based on the content of the Service Information Response ANQP-element, the non-AP and non-PCP STA can determine whether the BSS, to which the request was sent, can support the required services it was searching for and then it might decide to proceed with the authentication and association procedure (see 11.3 (STA authentication and association)) (see examples illustrated in W.1 (Pre-association discovery usage scenarios)). The details of this decision are outside the scope of this standard.