IEEE P802.11
Wireless LANs

| Architecture update |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2016-12-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Stephen McCann | BlackBerry Ltd | The Pearce Building, West Street, Maidenhead, Berkshire, SL6 1RL, UK | +44 1753 667099 | smccann@blackberry.com |

Abstract

This document proposes text changes to update the P802.11aq D7.0 for the following SBO CIDs:

6070

## 4. General description

#### Overview of the services

##### Interworking with external networks

Change as follows:

An overview of the interworking functions addressed in this standard is provided below:

Network discovery and selection

Discovery of suitable networks through the advertisement of access network type, roaming consortium and venue information, via Management frames

Selection of a suitable IEEE Std 802.11 infrastructure using advertisement services (e.g., access network query protocol (ANQP) or an IEEE Std 802.21 Information Server) in the BSS or in an external network reachable via the BSS.

Selection of an SSPN or external network with its corresponding IEEE Std 802.11 Infrastructure

Pre-association discovery (PAD)

Discovery of services offered by a BSS or an external network reachable via that BSS

Emergency services

Emergency Call and Network Alert support at the link level

QoS mapping distribution

SSPN interface service between the AP and the SSPN

Insert the following new clauses:

      - Pre-association discovery (PAD)

PAD is an interworking function provided by a BSS (i.e., a network) to allow non-AP STAs, in a pre-associ- ation state, to discover information concerning services that are available to a STA when it is associated with the BSS.

3

##### Architecture

Figures 4-15a and 4-15b shows the functional flow of MAC messaging and service information between a service information client (SIC) and a service information registry (SIR). These entities are described below. The architecture differs within the infrastrucuture dependant upon whether unsolicted or solicited PAD is being used.

##### Figure 4-15a — Unsolicited PAD architecture

##### Figure 4-15b — Solicited PAD architecture

##### Figure 4-15c — PCP architecture

NOTE—An SIR can be either co-located with the AP or outside the AP. An SIR can only be co-located with an PCP (#6070).  The communications between the SIR and the AP, together with the SIR and the ANQP server are out of the scope of this document.

##### Service information entities

SIC and SIR are used to exchange information between higher layer entities above both the non-AP and non-PCP STA and the AP or PCP.  The PAD procedures operate between the SIC and SIR.

The SIC and SIR are used to transport service relevant information (e.g. a service name), and then exchange that information between the SIC and SIR. PAD signaling is opaque to the service relevant information because it is handled by these entities in the STA and AP or PCP.

##### Service information registry

The SIR contains information about services that might be reachable through BSS, for the STA to connect to after it associates with the AP. How the SIR obtains the information about services is outside the scope of this standard.

There is typically one SIR assigned to each ESS. It can be addressed by each AP and each service within that ESS. It includes supporting service discovery requests and responses from the BSS.

The SIR communicates with the ANQP server to respond to ANQP requests.

##### Service information client

The SIC initiates STA service discovery.

Within a STA this exchange includes supporting service discovery requests and responses to and from appli- cations. The SME determines whether to use unsolicited PAD or solicited PAD procedures. The SME also creates the contents of the probe requests and ANQP requests.