IEEE P802.11
Wireless LANs

| 802.11bc Functional Requirements Document |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2019-03-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Hitoshi Morioka | SRC Software | 2-14-38 Tenjin, Chuo-ku, Fukuoka 810-0001 Japan | +81-92-791-5766 | hmorioka@src-soft.com |
| Marc Emmelmann | Koden-TI | Berlin, Germany |  | emmelmann@ieee.org |
| Stephen McCann | BlackBerry | The Pearce Building, West Street, Maidenhead, SL6 1RL, UK |  | smccann@blackberry.com |
| Xiaofei Wang | InterDigital |  |  | Xiaofei.Wang@InterDigital.com |

### Abstract

This document describes the 802.11bc Functional Requirements.

R0: skeleton

R1: 11-19/171r1, 11-19/183r1 as redlined.

R2: 11-19/446r3 as redlined.

- Introduction

- Purpose

This document summarizes requirements, which agreed to by the Task Group, for solutions addressing functionality to be provided by the 802.11bc amendment, referred to as the TGbc Functional Requirements (FRs).

- Scope

The scope for deriving functional requirements is primarily set by the P802.11bc PAR Error: Reference source not found and CSD Error: Reference source not found, as well as by the TGbc usage models Error: Reference source not found. Additional requirements may be added if agreed by and considered essential by the task group.  The following figure illustrates the workflow on how to derive functional requirements and how to add them to the Functional Requirements Document owned by the task group following the steps of the 802.11bc Selection Procedure [Ref-4].

- Notation

Requirements are identified by a preceding unique number in the format of “TGbc Rn”, where n is an integer number representing the ID of the requirements.

- Definitions

TGbc D1:	“Consuming”:	By “consuming a broadcast service”, it is meant that STA receives a corresponding frame and forwards its contents to the upper layer.

- Functional Requirements

- Compliance to PAR

TGbc R1:	The 802.11bc amendment shall comply with the PAR Error: Reference source not found and the CSD Error: Reference source not found.

- Restrictions

TGbc R2:	The 802.11bc amendment shall not change the IEEE 802.11 PHY specifications. [Ref-1 5.2.b]

- Security

TGbc R3:	The 802.11bc amendment shall provide origin authenticity protection for broadcast data frames. [Ref-1 5.2.b]

TGbc R4:	The 802.11bc amendment shall provide a broadcast mechanism in which e2e security w/ confidentiality and integrity between STA and the server is assumed. [Ref-5]

TGbc R5:	The 802.11bc amendment shall provide a mechanism for the AP to verify the STA before forwarding the packets. [Ref-5]

- Association state

TGbc R6:	The 802.11bc amendment shall support modes of operation:

- in an infrastructure BSS where there is an association between the transmitter and the receiver(s)

- in cases where there is no association between transmitter(s) and receiver(s). [Ref-1 5.2.b]

TGbc R7:	802.11bc amendment shall have a mechanism to facilitate power-efficient broadcasting.

TGbc R8:	802.11bc amendment assumes that there is no association and/or handshake required between STA and the AP. [Ref-5]

  - Discovery

TGbc R9:	802.11bc amendment shall have a mechanism to facilitate discovery of 11bc-capable STAs.

TGbc R10:	802.11bc amendment shall have a mechanism to facilitate the discovery of STAs consuming a particular broadcast service.

  - Simultaneous broadcast service

TGbc R11:	STAs should be capable of consuming multiple broadcast services simultaneously.

  - Air time consumption

TGbc R12:	802.11bc amendment shall have a mechanism to facilitate the sharing of airtime between all broadcast services and all non-broadcast services communications.

- TGbc R13:	802.11bc amendment shall have a mechanism for starting a broadcast service.

- TGbc R14:	802.11bc amendment shall have a mechanism for stopping a broadcast service.

- References

- https://mentor.ieee.org/802.11/dcn/18/11-18-0825-09-0bcs-a-par-proposal-for-bcs.docx

- https://mentor.ieee.org/802.11/dcn/18/11-18-0826-09-0bcs-a-csd-proposal-for-bcs.docx

- https://mentor.ieee.org/802.11/dcn/19/11-19-0268-04-00bc-tgbc-use-case-document.pptx

- https://mentor.ieee.org/802.11/dcn/19/11-19-0135-00-00bc-tgbc-selection-procedure.docx

- https://mentor.ieee.org/802.11/dcn/19/11-19-0446-03-00bc-proposed-functional-requirements-for-802-11bc.doc