IEEE P802.11
Wireless LANs

| Proposed functional requirements for 802.11bc |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2019-03-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Bahar Sadeghi | Intel |  |  | Bahareh.sadeghi@intel.com |
| Stephen McCann | BlackBerry | The Pearce Building, West Street, Maidenhead, SL6 1RL, UK |  | smccann@blackberry.com |

### Abstract

This document proposes additional functional requirements based on [1] for inclusion in Sections 3.3 and 3.4 of TGbc functional requirements document [2].

References

[1] IEEE 802.11-19/0115r0 “Uplink BCS requirements.ppt”

[2] IEEE 802.11-19/0151r1 “802.11bc Functional Requirements Document.doc”

- Definitions

TGbc D1:	“Consuming”:	By “consuming a broadcast service”, it is meant that STA receives a corresponding frame and forwards its contents to the upper layer.

- Functional Requirements

- Compliance to PAR

TGbc R1:	The 802.11bc amendment shall comply with the PAR Error: Reference source not found and the CSD Error: Reference source not found.

- Restrictions

TGbc R2:	The 802.11bc amendment shall not change the IEEE 802.11 PHY specifications. [Ref-1 5.2.b]

- Security

TGbc R3:	The 802.11bc amendment shall provide origin authenticity protection for broadcast data frames. [Ref-1 5.2.b]

TGbc R4:	The 802.11bc amendment assumes E2E security with confidentiality and integrity between the STA and the server.

TGbc R5:	The 802.11bc amendment shall provide a mechanism for the AP to verify the STA before forwarding the packets.

- Association state

TGbc R46:	The 802.11bc amendment shall support modes of operation:

- in an infrastructure BSS where there is an association between the transmitter and the receiver(s)

- in cases where there is no association between transmitter(s) and receiver(s). [Ref-1 5.2.b]

TGbc R57:	802.11bc amendment shall have a mechanism to facilitate power-efficient broadcasting.

TGbc R8:	802.11bc amendment assumes that there is no association and/or handshake required between STA and the AP.