IEEE P802.11
Wireless LANs

| Specification Framework Document for 802.11bc |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2019-08-29 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Carol Ansley | CommScope | 3871 Lakefield Dr. Suwanee, GA 30024 US | +1 678 473 8630 | carol.ansley@commscope.com |
| Stephen McCann | BlackBerry Ltd | The Pearce Building, West Street, Maidenhead, SL6 1RL, UK | +44 1753 667099 | smccann@blackberry.com |

Abstract

This document is the Specification Framework Document for 802.11 TGbc.

R1: Update per Motion #38.

R2: Update from Motions 48, 49, 51, 52 and 53 as in 19/2123r8.

R3: Update per motions 61 and 62 as in 19/2123r9.

- Overview

1.3 Supplementary information on purpose

Insert the following to the end of the list:

Specifically, in the context of IEEE 802.11™-compliant devices, this standard

....

——Defines a mechanism to enable IEEE 802.11 stations to enable enhanced transmission and reception of broadcast data both in an infrastructure BSS where there is an association between the transmitter and the receiver(s) and in cases where there is no association between transmitter(s) and receiver(s).

- Normative references

The following referenced documents are indispensable for the application of this document (i.e., they must be understood and used, so each referenced document is cited in text and its relationship to this document is explained). For dated references, only the edition cited applies. For undated references, the latest edition of the referenced document (including any amendments or corrigenda) applies.

- Definitions

For the purposes of this document, the following terms and definitions apply. The IEEE Standards Dictionary Online should be consulted for terms not defined in this clause.

3.2 Definitions specific to IEEE Std 802.11 [Motion #52]

Enhanced Broadcast Service (eBCS) receiver: An STA that receives Enhanced Broadcast Services (eBCS) frames.

Enhanced Broadcast Service (eBCS) transmitter: An STA that transmits Enhanced Broadcast Services (eBCS) frames.

## 3.4 Abbreviations and acronyms

Insert the following acronym definitions (maintaining alphabetical order):

eBCS			Enhanced Broadcast Service

# 4 General Description

## 4.3 Components of the IEEE Std 802.11 architecture

Insert a new subclause after subclause 4.3.30 as follows:

### 4.3.31 Enhanced Broadcast Service

[Motion #62] Enhanced Broadcast Service (eBCS) enables efficient distribution of local information. eBCS provides enhanced transmission and reception of broadcast data both in an infrastructure BSS where there is an association between the transmitter and the receiver(s) and in cases where there is no association between transmitter(s) and receiver(s).

eBCS provides additional means for protecting broadcast traffic and the privacy of the stations receiving that traffic, including protection of origin authenticity between STAs that use a group temporal key security association (GTKSA) for broadcast transmissions.

The main features of eBCS are the following:

-TBD

4.5 Overview of the services

4.5.4 Access control and data confidentiality services

Change the nth paragraphs as follows:

<The current IEEE Std. 802.11 has a group temporal key security association (GTKSA) security framework for multicast that does not protect origin authenticity between devices having that GTKSA. Such protection is needed in some broadcast use cases.>

# 6 Layer Management

## MLME SAP interface

6.3.<ANA1> eBCS Info transmission [Motion #52]

Describe MLME SAP for eBCS Info frame transmission.

6.3.<ANA2> eBCS Info reception [Motion #52]

Describe MLME SAP for eBCS Info frame reception.

# 9 Frame Formats

- Beacon frame format  [Motion #49]

| - Beacon frame body |  |  |
| --- | --- | --- |
| Order | Information | Notes |
| <ANA> | E-BCS Capabilities | This element is optionally present if dot11eBCSSupportImplemented is true. |

- Probe Response frame format  [Motion #49]

| - Probe Response frame body |  |  |  |
| --- | --- | --- | --- |
| Order | Information | Notes |  |
| <ANA> | E-BCS Capabilities | This element is optionally present if dot11eBCSSupportImplemented is true. |  |

## 9.4.2 Elements

#### 9.4.2.1 General

Insert the following new rows into Table 9-77 (Element IDs) (header row shown for convenience):

Table 9-77 -- Element IDs

| Element | Element ID | Element ID Extension | Extensible |
| --- | --- | --- | --- |
| eBCS Parameters | 255 | <ANA> | Yes |
| E-BCS Capabilities | 255 | <ANA> | Yes [Motion #49] |
| E-BCS Container | 255 | <ANA> | Yes [Motion #49] |

#### 9.4.2.27 Extended Capabilities element

Table 9-134—Extended Capabilities element

| Bits | Information | Notes |
| --- | --- | --- |
| <ANA> | eBCS Support | A STA sets the eBCS Support field to 1 when dot11eBCSSupportImplemented is true. Otherwise, the STA sets the eBCS Support field to 0. |

Insert the following subclauses:

- 9.4.2.<ANA> E-BCS Capabilities [Motion #49]

An eBCS STA advertises its capabilities in this element

The element carries:

- A bit field indicating if the transmitting (AP) STA supports forwarding to a remote server

- A bit field indicating if the transmitting (AP) STA supports embedding information (such as location, date, time) when sending a frame to a remote server

- A bit field indicating if transmitting (non-AP) STA requires embedding information (such as location, date, time) when sending a frame to a remote server

- Set of bits indicating transmitting (AP) STA’s mode of operation when AP support forwarding service

- Set of bits indicating transmitting (non-AP) STA’s capabilities

- Other fields TBD

- 9.4.2.<ANA> E-BCS Container [Motion #49]

The element carries:

- A field for carrying higher layer data

  - Can carries the ID of the transmitting STA

- Other fields TBD

9.4.2.248 eBCS Parameters

The enhanced Broadcast Services (eBCS) Parameters element contains:

- A list of one or more eBCS services offered by the AP

- Indication whether one or more eBCS services requires association

- Indication of negotiation method for eBCS services: e.g., through eBCS Request/Response frames (e.g., for associated STAs), or through ANQP exchanges (e.g., for unassociated STAs) [Motion 48]

9.4.5 Access network query protocol (ANQP) elements: [Motion #38]

Add the following elements:

- Define a new “Broadcast Capability” ANQP-element for eBCS services advertisement

  - ANQP-element should indicate 0, 1, or multiple eBCS services

  - This element could contain broadcast channel and timing information (e.g. time of transmission)

  - Note: Since ANQP uses GAS, this provides both an unassociated and an association query/response mechanism (protected using Protected Dual of Public Action frames)

  - Note: GAS is bi-directional between two STAs and hence works on both the uplink and downlink.

  - The typical operation for a device would be to transmit a Broadcast Capability ANQP-element request to another device, and then the other device would respond with a Broadcast Capability ANQP-element response containing the required information, if the other device supports a broadcast capability.

- Define a new “Broadcast Switching” ANQP-element for requesting and reporting about switching broadcast channels on and off.

  - ANQP-element could request switching broadcast channels on and off.

  - Note: Since ANQP uses GAS, this provides both an unassociated and an association query/response mechanism (protected using Protected Dual of Public Action frames)

  - Note: GAS is bi-directional between two STAs and hence works on both the uplink and downlink.

  - The typical operation for a device would be to transmit a Broadcast Switching ANQP-element request to another device, with appropriate sub-fields set for the required action, and then the other device would respond with a Broadcast Switching ANQP-element response containing the required information, including error codes.

9.6 Action frame format details

9.6.7 Public Action details

9.6.7.1 Public Action frames

Add E-BCS UL Table 9-363.[Motion #49]

Add eBCS Info frame to Table 9-362. [Motion #52]

9.6.7.<ANA5> E-BCS UL frame format [Motion #49]

This GAS frame includes:

- Destination URI element (9.4.2.89)

  - Carries the URI to the destination server

- E-BCS Capabilities element (Optionally present)

  - Provides capabilities of the transmitting non-AP STA

- E-BCS Container

  - Carries the data to be forwarded to the remote server

9.6.7.<ANA5> eBCS Info frame format [Motion #52]

Describe eBCS Info frame format that contains:

- eBCS Info sequence number

- eBCS transmitter’s certificate signed by CA

- Timestamp

- Authentication algorithm identifier

- eBCS Info transmission interval

- Contents information

  - Human readable title

  - Higher layer protocol type

  - Destination IP address and UDP port (if UDP/IP)

  - Hash chain parameters (if hash chain is used) [11-19/1802r1]

    - Hash chain distances

    - Details are future work

- Signature for this eBCS Info frame signed by the eBCS transmitter’s private key

- Contents data (optional)

  - if public key authentication is used and uplink case, data can only be transmitted via this frame.

  - If public key authentication is used and downlink case, field is not present (piggy-backing not allowed).

9.6.31 eBCS Service Request Frame [Motion #48]

This frame is used by associated STAs to request eBCS services.

9.6.32 eBCS Service Response Frame [Motion #48]

This frame is used by an eBCS AP to respond to an eBCS services Request frame by an associated STA.

9.6.33 eBCS Service Advertisement Frame [Motion # 53]

This frame is used to periodically advertise the services provided by the eBCS AP.

10. MAC sublayer functional description

10.6 Multirate support

10.6.5 Rate selection for Data and Management frame

Insert the following subclauses:

10.6.5.3 Rate selection for eBCS frames [Motion # 51][Motion 61]

The transmission rate for eBCS Info frames and eBCS Data frames are determined from dot11EBCSInfoTxRate and dot11EBCSDataTxRate respectively, if FMS or GCR is not used, otherwise follow clause 10.***.

Dot11EBCSInfoTxRate and dot11EBCSDataTxRate are configurable item and the values are selected from any available rates other than DSSS.

Renumber 10.6.5.3 to 10.6.5.4:

10.6.5.4 Rate selection for other group addressed Data and Management frames

Change the 1st paragraph as follows:

This subclause describes the rate selection rules for group addressed Data and Management frames, excluding

the following:

— Non-STBC Beacon and non-STBC PSMP frames

— ER beacon and HE beacon

— STBC group addressed Data and Management frames

— Data frames located in an FMS stream (see 11.22.8 (FMS multicast rate processing))

— Group addressed frames transmitted to the GCR concealment address (see 11.22.16.3.5 (Concealment of GCR transmissions))

— Group addressed Data and Management frames transmitted in an HE ER SU PPDU (see 26.15.5 (Additional rules for ER beacons and group addressed frames))

— Group addressed Data and Management frames transmitted in an HE SU PPDU (see 26.15.6 (Additional rules for HE beacons and group addressed frames))

— Group addressed Data and Management frames transmitted in an HE MU PPDU (see 26.15.7 (Additional rules for group addressed frames in an HE MU PPDU))

— eBCS Info, Data and Service Discovery frames

# 11 MLME

### 11.22.6 Enhanced Broadcast Service (eBCS) procedures

Insert the following subclauses:

11.22.6.1 Overview

The eBCS procedures allow a STA to ….

- 11.22.6.2 eBCS Request and Response Procedure [Motion #48]

The eBCS procedures allow a STA and its associated AP to negotiate eBCS service provided.

- 11.22.6.3 eBCS Service Advertisement Procedure [Motion #53]

The eBCS Service Advertisement frame allows associated and unassociated stations to discover services provided by the eBCS AP.

- 11.<ANA>.<ANA> eBCS UL Service [Motion #49]

The eBCS UL Service procedure allows a non-AP STA to transmit an UL frame with the expectation that one or more eBCS AP(s) in the neighborhood forward the contents of the frame to a remote destination specified in the frame.

The eBCS spec shall provide a mode of operation for an eBCS AP that prevents unauthenticated unsigned packet to be forwarded to the remote destination.

The eBCS spec shall provide a mechanism for an eBCS AP to signal its mode of operation

A non-AP eBCS STA, based on its capabilities, shall conform to a mode of operation set by a neighboring eBCS AP. For example, an eBCS STA, based on its capabilities, monitors the WM before transmitting an UL frame directed to a remote server. A non-AP eBCS STA that doesn’t support a certain mode in which an eBCS AP operates is not required to confirm to the mode of operation set by a neighboring eBCS AP.

An eBCS AP may limit UL forwarding to prevent excessive ULs to a remote destination based on certain criteria TBD.

Describe the procedure in detail – including discussion on eBCS AP / non-AP STA capabilities and actions on eBCS AP if a STA requests embedding of information.

11.<ANA8>.<ANA9> eBCS Info frame generation and usage [Motion #52]

11.<ANA8>.<ANA9>.1 eBCS Info frame transmission

Describe eBCS Info frame transmission procedure.

11.<ANA8>.<ANA9>.2 eBCS Info frame reception

Describe eBCS Info frame reception procedure.

11.<ANA8>.<ANA10> eBCS Data frame generation and usage [Motion #52]

11.<ANA8>.<ANA10>.1 eBCS Data frame transmission

Describe eBCS Data frame transmission procedure.

11.<ANA8>.<ANA10>.2 eBCS Data frame reception

Describe eBCS Data frame reception procedure.

11.23.3.3 	ANQP procedures

Add behavior text about the 2 new ANQP-elements suggested in clause 9.4.5 [Motion #38]

# 12 Security

12.<ANA11> Frame authentication for eBCS [Motion #52]

12.<ANA11>.1 General

Describe abstract of the eBCS frame authentication mechanism that contains:

- eBCS public key frame authentication that uses only public key algorithm

- eBCS hash chain-public key frame authentication that uses combination of hash chain and public key algorithm

12.<ANA11>.2 eBCS public key frame authentication

Describe eBCS public key authentication procedure.

12.<ANA11>.3 eBCS hash chain-public key frame authentication

Describe eBCS hash chain-public key frame authentication procedure.

# (informative)
Bibliography

Bibliographical references are resources that provide additional or helpful material but do not need to be understood or used to implement this standard. Reference to these resources is made for informational use only.

Annex B

B.4 PICS proforma-IEEE Std 802.11-20xx

B.4.3 IUT configuration

Add description for eBCS transmitter and receiver support

B.4.<ANA12> eBCS features

Describe table that contains:

- eBCS Info frame transmission

- eBCS Info frame reception

- eBCS Data frame transmission

- eBCS Data frame reception

- eBCS public key frame authentication

- eBCS hash chain-public key frame authentication

# Annex C (normative)

ASN.1 encoding of the MAC and PHY MIB

C.3 MIB Detail

Insert the following entry in Annex C::

Dot11WirelessMgmtOptionsEntry ::=

SEQUENCE {

…

dot11FineTimingMsmtRespActivated TruthValue,

dot11FineTimingMsmtInitActivated TruthValue,

dot11LciCivicInNeighborReport TruthValue,

dot11RMFineTimingMsmtRangeRepImplemented TruthValue,

dot11RMFineTimingMsmtRangeRepActivated TruthValue,

dot11RMLCIConfigured TruthValue,

dot11RMCivicConfigured TruthValue,

dot11eBCSSupportImplemented TruthValue

}	…

dot11eBCSSupportImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This attribute, when true, indicates that a eBCS protocol (see 11.22.6  (eBCS Procedures)) is implemented. The capability is disabled otherwise."

DEFVAL { false }

::= { dot11WirelessMgmtOptionsEntry <ANA>}

Add the following line to “dot11smt”: [Motion #52]

-- dot11eBCSConfigTable ::= { dot11smt <ANA13> }

Add the following lines to appropriate place. [Motion #52]

-- ****************************************

-- * dot11eBCSConfigTable TABLE

-- ****************************************

dot11eBCSConfigTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11eBCSConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

“The table contains enhanced broadcast service configuration objects.”

::= { dot11smt <ANA13> }

Describe “Dot11eBCSConfigEntry” according to the amendment.

IEEE Standards Dictionary Online is available at: http://dictionary.ieee.org