IEEE P802.11
Wireless LANs

| CC36 CR on Annex Z EHT-SIG examples |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2021.07.02 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ross Jian Yu | Huawei Technologies | F3-6-A120, Huawei Base, Bantian, Longgang, Shenzhen, Guangdong, China, 518129 |  | ross.yujian@huawei.com |
| Ming Gan |  |  |  |  |

Abstract

This submission contains proposed comment resolutions to comments on P802.11be D1.0. The changes are based on P802.11be D1.01.

6 comments are resolved.

CID 4915, 4916, 5433, 6473, 6474, 6475

# Revision Notes

| R0 | Initial revision |
| --- | --- |

## CID 4915

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 620.36 | Z.6 | In the common field of EHT-SIG, LDPC Extra symbol segment is set to 1. and, this is commonly applied to484+242-tone MRU 2 and 242-tone RU 2 in table z-9. therefore, BCC for 242 tone RU 2 should be modified as LDPC. | As in comment | REJECTED If STA with STA-ID 1441 meets the condition of having an LDPC extra symbol segment, then LDPC Extra symbol segment is set to 1, even STA with STA-ID 1442, uses BCC. |

Discussion:

In P802.11be D1.0, page 482, it includes the description regarding when LDPC extrea symbol segment is set to 1:

## CID 4916

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 622.11 | Z.7 | In the common field of EHT-SIG, LDPC Extra symbol segment is set to 1. and, this is commonly applied to 484+242-tone MRU 2 and 242-tone RU2 in table z-13. therefore, BCC for 242 tone RU 2 should be modified as LDPC. | As in comment | REJECTED If STA with STA-ID 1441/1443/1445 meets the condition of having an LDPC extra symbol segment, then LDPC Extra symbol segment is set to 1, even STA with STA-ID 1442, uses BCC. |

## CID 5433

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 624.30 | Annex Z | The EHT-SIG contents shall be the same per 80MHz for non-OFDMA transmission. | Remove "and for non-OFDMA transmission" | ACCEPTED |

## CID 6473

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 620.41 | Z.6 | Change "a RU/MRU" to "an RU/MRU" | as in comment | REVISED Agreed in principle. Intstruction to the editor: Change “a RU Allocation subfield” to “an RU Allocation subfield”. in page 642, L41, page 644, L20, page 646, L1, and page 649, L1 of P802.11be D1.01. |

## CID 6474

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 624.01 | 624.01 | Change "a RU/MRU" to "an RU/MRU" | as in comment | REVISED Agreed in principle. Intstruction to the editor: Change “a RU Allocation subfield” to “an RU Allocation subfield”. in page 642, L41, page 644, L20, page 646, L1, and page 649, L1 of P802.11be D1.01. |

## CID 6475

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 627.01 | Z.23 | Change "a RU/MRU" to "an RU/MRU" | as in comment | REVISED Agreed in principle. Intstruction to the editor: Change “a RU Allocation subfield” to “an RU Allocation subfield”. in page 642, L41, page 644, L20, page 646, L1, and page 649, L1 of P802.11be D1.01. |