IEEE P802.11
Wireless LANs

| CR for Clause 36.3.2.2 Subcarriers and resource allocation for multiple RUs - part 2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2022.2.25 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Myeongjin Kim | Samsung |  |  | mj1108.kim@samsung.com |
| WookBong Lee | Samsung |  |  | wookbong.lee@samsung.com |

Abstract

This submission proposes resolutions for the comments from the CC36 on P802.11be D1.0

Baseline document: TGbe D1.4.

R0:

- The following 49 comments under “36.3.2.2 Subcarriers and resource allocation for multiple RUs” are resolved:

- CID 4681, 4682, 4683, 4684, 7148, 7150, 7153, 7154, 7155, 4791, 4792, 4793, 4794, 4795, 4796, 4797, 4798, 4799, 4800, 4801, 4903, 4986, 4987, 4988, 4989, 6790, 6789, 7318, 5466, 5812, 6787, 7151, 6428, 6429, 6430, 6785, 6786, 6788, 4674, 7144, 7145, 7146, 7147, 7149, 7152, 7156, 6129, 7142, 7756

R1:

- Updated resolutions for  CID 5466, 6129, 7142, 7756 and fixed the typos

# Revision Notes

| R0 | Initial revision |
| --- | --- |
| R1 | Updated resolutions |

## CID 4681, 4682, 4683, 4684, 7148, 7150, 7153, 7154, 7155

| CID | Clause Number | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 4681 | 36.3.2.2.3.2 | 358.45 | The allowed MRUs are exactly the same with non-OFDMA, so it is better to just cite it from the previous chapter in order to simplify the document | As in comment | Revised This comment is about 484+242-tone MRUs in an OFDMA 80 MHz EHT PPDU. Instructions to the editor: Please make the changes as shown in 11/22-0383r1 |
| 4682 | 36.3.2.2.3.2 | 359.12 | The allowed MRUs are exactly the same with non-OFDMA, so it is better to just cite it from the previous chapter in order to simplify the document | As in comment | Revised This comment is about 996+484-tone MRUs in an OFDMA 160 MHz EHT PPDU. Instructions to the editor: Please make the changes as shown in 11/22-0383r1 |
| 4683 | 36.3.2.2.3.2 | 360.01 | The allowed MRUs are exactly the same with non-OFDMA, so it is better to just cite it from the previous chapter in order to simplify the document | As in comment | Revised This comment is about 2×996+484-tone MRUs in an OFDMA 320 MHz EHT PPDU. Instructions to the editor: Please make the changes as shown in 11/22-0383r1 |
| 4684 | 36.3.2.2.3.2 | 360.43 | The allowed MRUs are exactly the same with non-OFDMA, so it is better to just cite it from the previous chapter in order to simplify the document | As in comment | Revised This comment is about 3×996-tone MRUs in an OFDMA 320 MHz EHT PPDU. Instructions to the editor: Please make the changes as shown in 11/22-0383r1 |
| 7148 | 36.3.2.2.3.2 | 358.60 | Figure 36-17 is identical to Figure 36-11. | Make reference to Figure 36-11 instead and delete duplicate figure. | Revised Already revised by CID 4681. Note to the editor: No further changes are needed. |
| 7150 | 36.3.2.2.3.2 | 359.27 | Figure 36-18 is identical to Figure 36-12. | Make reference to Figure 36-12 instead and delete duplicate figure. | Revised Already revised by CID 4682. Note to the editor: No further changes are needed. |
| 7153 | 36.3.2.2.3.2 | 360.34 | Figure 36-19 is identical to Figure 36-14. | Make reference to Figure 36-14 instead and delete duplicate figure. | Revised Already revised by CID 4683. Note to the editor: No further changes are needed. |
| 7154 | 36.3.2.2.3.2 | 360.55 | Figure 36-20 is identical to Figure 36-15. | Make reference to Figure 36-15 instead and delete duplicate figure. | Revised Already revised by CID 4684. Note to the editor: No further changes are needed. |
| 7355 | 36.3.2.2.3.2 | 361.25 | Figure 36-21 is identical to Figure 36-16. | Make reference to Figure 36-16 instead and delete duplicate figure. | Revised Instructions to the editor: Please make the changes as shown in 11/22-0383r1 |

Proposed Changes:

Modify P505 L47-L63 of D1.4 as follows: (The baseline for this text is 802.11be D1.4.)

For an OFDMA 80 MHz EHT PPDU, the four allowed 484+242-tone MRUs are same as for a non-OFDMA 80 MHz EHT PPDU as shown in Figure 36-11 (Allowed 484+242-tone MRUs in a non-OFDMA 80 MHz EHT PPDU).

Proposed Changes:

Modify P506 L11-L27 of D1.4 as follows: (The baseline for this text is 802.11be D1.4.)

For an OFDMA 160 MHz EHT PPDU, the four allowed 996+484-tone MRUs are same as for a non-OFDMA 160 MHz EHT PPDU as shown in Figure 36-12 (Allowed 996+484-tone MRUs in a non-OFDMA 160 MHz EHT PPDU).

Proposed Changes:

Modify P507 L1-L34 of D1.4 as follows: (The baseline for this text is 802.11be D1.4.)

The twelve allowed 2×996+484-tone MRUs in an OFDMA 320 MHz EHT PPDU are same as for a non-OFDMA 320 MHz EHT PPDU as shown in Figure 36-14 (Allowed 2×996+484-tone MRUs in a non-OFDMA 320 MHz EHT PPDU).

Proposed Changes:

Modify P507 L41-L57 of D1.4 as follows: (The baseline for this text is 802.11be D1.4.)

The four allowed 3×996-tone MRUs in an OFDMA 320 MHz EHT PPDU are are same as for a non-OFDMA 320 MHz EHT PPDU as shown in Figure 36-15 (Allowed 3×996-tone MRUs in a non-OFDMA 320 MHz EHT PPDU).

Proposed Changes:

Modify P508 L1-L26 of D1.4 as follows: (The baseline for this text is 802.11be D1.4.)

The eight allowed 3×996+484-tone MRUs in an OFDMA 320 MHz EHT PPDU are are same as for a non-OFDMA 320 MHz EHT PPDU as shown in Figure 36-16 (Allowed 3×996+484-tone MRUs in a non-OFDMA 320 MHz EHT PPDU).

## CID 4791, 4792, 4793, 4794, 4795, 4796, 4797, 4798, 4799, 4800, 4801

| CID | Clause Number | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 4791 | 36.3.2.2.3.1 | 355.38 | Figure 36-11 should be consistent with the layout of Figure 36-5 to Figure 36.10. | Show the DC locations and the regular 242 and 484 RU locations for an 80MHz EHT PPDU within Figure 36-11 | Revised Instructions to the editor: Please make the changes as shown in 11/22-0383r1 |
| 4792 | 36.3.2.2.3.1 | 356.05 | Figure 36-12 should be consistent with the layout of Figure 36-5 to Figure 36.10. | Show the DC locations and the regular 484 and 996 RU locations for an 160MHz EHT PPDU within Figure 36-12 | Revised Instructions to the editor: Please make the changes as shown in 11/22-0383r1 |
| 4793 | 36.3.2.2.3.1 | 356.31 | Figure 36-13 should be consistent with the layout of Figure 36-5 to Figure 36.10. | Show the DC locations and the regular 242, 484 and 996 RU locations for an 160MHz EHT PPDU within Figure 36-13 | Revised Instructions to the editor: Please make the changes as shown in 11/22-0383r1 |
| 4794 | 36.3.2.2.3.1 | 357.06 | Figure 36-14 should be consistent with the layout of Figure 36-5 to Figure 36.10. | Show the DC locations and the regular 484 and 996 RU locations for an 320MHz EHT PPDU within Figure 36-14 | Revised Instructions to the editor: Please make the changes as shown in 11/22-0383r1 |
| 4795 | 36.3.2.2.3.1 | 357.47 | Figure 36-15 should be consistent with the layout of Figure 36-5 to Figure 36.10. | Show the DC locations and the regular 996 RU locations for an 320MHz EHT PPDU within Figure 36-15 | Revised Instructions to the editor: Please make the changes as shown in 11/22-0383r1 |
| 4796 | 36.3.2.2.3.1 | 358.05 | Figure 36-16 should be consistent with the layout of Figure 36-5 to Figure 36.10. | Show the DC locations and the regular 484 and 996 RU locations for an 320MHz EHT PPDU within Figure 36-16 | Revised Instructions to the editor: Please make the changes as shown in 11/22-0383r1 |
| 4797 | 36.3.2.2.3.2 | 358.48 | Figure 36-17 should be consistent with the layout of Figure 36-5 to Figure 36.10. | Show the DC locations and the regular 242 and 484 RU locations for an 80MHz EHT PPDU within Figure 36-17 | Rejected Based on the resolutions for CID 4681, 4682, 4683, and, 4684, Figure 36-17 was deleted. |
| 4798 | 36.3.2.2.3.2 | 359.16 | Figure 36-18 should be consistent with the layout of Figure 36-5 to Figure 36.10. | Show the DC locations and the regular 484 and 996 RU locations for an 160MHz EHT PPDU within Figure 36-18 | Rejected Based on the resolutions for CID 4681, 4682, 4683, and, 4684, Figure 36-18 was deleted. |
| 4799 | 36.3.2.2.3.2 | 360.05 | Figure 36-19 should be consistent with the layout of Figure 36-5 to Figure 36.10. | Show the DC locations and the regular 484 and 996 RU locations for an 320MHz EHT PPDU within Figure 36-19 | Rejected Based on the resolutions for CID 4681, 4682, 4683, and, 4684, Figure 36-19 was deleted. |
| 4800 | 36.3.2.2.3.2 | 360.45 | Figure 36-20 should be consistent with the layout of Figure 36-5 to Figure 36.10. | Show the DC locations and the regular 996 RU locations for an 320MHz EHT PPDU within Figure 36-20 | Rejected Based on the resolutions for CID 4681, 4682, 4683, and, 4684, Figure 36-20 was deleted. |
| 4801 | 36.3.2.2.3.2 | 361.05 | Figure 36-21 should be consistent with the layout of Figure 36-5 to Figure 36.10. | Show the DC locations and the regular 484 and 996 RU locations for an 320MHz EHT PPDU within Figure 36-21 | Rejected Based on the resolutions for CID 4681, 4682, 4683, and, 4684, Figure 36-21 was deleted. |

Proposed Changes:

Modify Figure 36-11, Figure 36-12, Figure 36-13, Figure 36-14, Figure 36-15, and Figure 36-16 as follows: (The baseline for this text is 802.11be D1.4.)

Figure 36-11—Allowed 484+242-tone MRUs in a non-OFDMA 80 MHz EHT PPDU(#1296)

Figure 36-12—Allowed 996+484-tone MRUs in a non-OFDMA 160 MHz EHT PPDU(#1296)

Figure 36-13—Allowed 996+484+242-tone MRUs in a non-OFDMA 160 MHz EHT PPDU(#1296)

Figure 36-14—Allowed 2×996+484-tone MRUs in a non-OFDMA 320 MHz EHT PPDU(#1296)

Figure 36-15—Allowed 3×996-tone MRUs in a non-OFDMA 320 MHz EHT PPDU(#1296)

Figure 36-16—Allowed 3×996+484-tone MRUs in a non-OFDMA 320 MHz EHT PPDU(#1296)

## CID 4903, 4986, 4987, 4988

| CID | Clause Number | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 4903 | 36.3.2.2.3.3 | 361.37 | In the above clause, various MRUs for OFDMA and non-OFDMA are described separately. So, to use the consistent terminology, add the OFDMA in each table title as following 1. Table 36-13--Indices for large size MRUs in an OFDMA 80 MHz EHT PPDU and in a non-OFDMA 80 MHz EHT PPDU 2. Table 36-14--Indices for large size MRUs in a OFDMA 160 MHz EHT PPDU and in a non-OFDMA 160 MHz EHT PPDU 3. Table 36-15--Indices for large size MRUs in an OFDMA 320 MHz EHT PPDU and in a non-OFDMA 320 MHz EHT PPDU | As in comment | Accepted |
| 4986 | 36.3.2.2.3.3 | 361.50 | An 80 MHz EHT PPDU includes a non-OFDMA 80 MHz EHT PPDU. | Delete "and in a non-OFDMA 80 MHz EHT PPDU" in the title of Table 36-13. | Revised Already revised by CID 4903. Note to the editor: No further changes are needed. |
| 4987 | 36.3.2.2.3.3 | 362.07 | An 160 MHz EHT PPDU includes a non-OFDMA 160 MHz EHT PPDU. | Delete "and in a non-OFDMA 160 MHz EHT PPDU" in the title of Table 36-14. | Revised Already revised by CID 4903. Note to the editor: No further changes are needed. |
| 4988 | 36.3.2.2.3.3 | 363.07 | An 320 MHz EHT PPDU includes a non-OFDMA 320 MHz EHT PPDU. | Delete "in a non-OFDMA 320 MHz EHT PPDU" in the title of Table 36-15. | Revised Already revised by CID 4903. Note to the editor: No further changes are needed. |

## CID 4989, 6790, 6789, 7318, 5466

| CID | Clause Number | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 4989 | 36.3.2.2.3.3 | 364.21 | In 320MHz, 996+484+242-tone MRUs are not used. | Delete all rows for 996+484+242-tone MRU. | Revised Instructions to the editor: Please make the changes as shown in 11/22-0383r1 |
| 6790 | 36.3.2.2.3.3 | 364.21 | Table 36-15, 996+484+242 tone MRU is undefined for 320M EHT PPDU (only defined for 160M non-OFDMA EHT PPDU) | Remove the section of Table 36-15 corresponding to 996+484+242 tone MRU | Revised Already revised by CID 4989. Note to the editor: No further changes are needed. |
| 6789 | 36.3.2.2.3.3 | 364.07 | Table 36-15, typo for 996+484 MRU 4 combination: Should be "996-tone RU 1 + 484-tone RU 3" instead of "996-tone RU 2 + 484-tone RU 3" | As in comment | Revised Agree with the commenter. Instructions to the editor: Please make the changes as shown in 11/22-0383r1 |
| 7318 | 36.3.2.2.3.3 | 364.07 | MRU4 for 996+484 tone MRU should be 996-tone RU1 + 484-tone RU3. | Change "996-tone RU2 + 484-tone RU3" to "996-tone RU1 + 484-tone RU3" | Revised Already revised by CID 6789. Note to the editor: No further changes are needed. |
| 5466 | 36.3.2.2.3.3 | 361.50 | "Indices for large size MRUs in an 80 MHz EHT PPDU and in a non-OFDMA 80 MHz EHT PPDU", it is confusing. It includes both OFDMA and non-OFDMA. Moreover, using empty-RU242 is misleading for OFDMA case. The other RU242 can still be assigned to other uses if not punctured. Also there is no non-OFDMA EHT PPDU. | Change to "Indices for large size MRUs in an 80 MHz EHT PPDU". Remove the description in [] or think of better way to describe | Revised “Indices for large size MRUs in an 80 MHz EHT PPDU” was changed to “Indices for large size MRUs in an OFDMA 80 MHz EHT PPDU and in a non-OFDMA 80 MHz EHT PPDU” by CID 4903. Agree that empty-xxxRU can provie a confusion in OFDMA case. So, It looks better to change empty-xxxRU to (-) in Table 36-13, Table 36-14, and Table 36-15 and add descriptions as follows. NOTE 1—In non-OFDMA transmission, (-) indicates that the subchannel corresponding to (-) is punctured and is to help indicate the frequency order of the MRU in an 80/160/320 MHz PPDU. NOTE 2—In OFDMA transmission, (-) indicates that the subchannel corresponding to (-) is punctured or unassigned or used for data transmission by assigning one or multiple RU/MRU and is to help indicate the frequency order of the MRU within an 80/160/240/320 MHz subband. Instructions to the editor: Please make the changes as shown in 11/22-0383r1 |

Proposed Changes:

Modify Table 36-13, Table 36-14, and Table 36-15 as follows: (The baseline for this text is 802.11be D1.4.)

## Table 36-13—Indices for large size MRUs in an 80 MHz EHT PPDU and in a non-OFDMA 80 MHz EHT PPDU(#2398)

| MRU type | MRU index | Combinations |
| --- | --- | --- |
| 484+242-tone MRU | MRU 1 | 484-tone RU 2 + 242-tone RU 2; [(-) RU242 RU484] |
|  | MRU 2 | 484-tone RU 2 + 242-tone RU 1; [RU242 (-) RU484] |
|  | MRU 3 | 484-tone RU 1 + 242-tone RU 4; [RU484 (-) RU242] |
|  | MRU 4 | 484-tone RU 1 + 242-tone RU 3; [RU484 RU242 (-)] |
| NOTE 1—In non-OFDMA transmission, (-) indicates that the subchannel corresponding to (-) is punctured and is to help indicate the frequency order of the MRU in an 80/160/320 MHz PPDU. NOTE 2—In OFDMA transmission, (-) indicates that the subchannel corresponding to (-) is punctured or unassigned or used for data transmission by assigning one or multiple RU/MRU and is to help indicate the frequency order of the MRU within an 80/160/240/320 MHz subband. |  |  |

## Table 36-14—Indices for large size MRUs in a 160 MHz EHT PPDU and in a non-OFDMA 160 MHz EHT PPDU(#2398)

| MRU type | MRU index | Combinations |
| --- | --- | --- |
| 484+242-tone MRU | MRU 1 | 484-tone RU 2 + 242-tone RU 2; [(-) RU242 RU484] in lower 80 MHz channel |
|  | MRU 2 | 484-tone RU 2 + 242-tone RU 1; [RU242 (-) RU484] in lower 80 MHz channel |
|  | MRU 3 | 484-tone RU 1 + 242-tone RU 4; [RU484 (-) RU242] in lower 80 MHz channel |
|  | MRU 4 | 484-tone RU 1 + 242-tone RU 3; [RU484 RU242 (-)] in lower 80 MHz channel |
|  | MRU 5 | 484-tone RU 4 + 242-tone RU 6; [(-) RU242 RU484] in upper 80 MHz channel |
|  | MRU 6 | 484-tone RU 4 + 242-tone RU 5; [RU242 (-) RU484] in upper 80 MHz channel |
|  | MRU 7 | 484-tone RU 3 + 242-tone RU 8; [RU484 (-) RU242] in upper 80 MHz channel |
|  | MRU 8 | 484-tone RU 3 + 242-tone RU 7; [RU484 RU242 (-)] in upper 80 MHz channel |
| 996+484-tone MRU | MRU 1 | 996-tone RU 2 + 484-tone RU 2; [(-) RU484 RU996] |
|  | MRU 2 | 996-tone RU 2 + 484-tone RU 1; [RU484 (-) RU996] |
|  | MRU 3 | 996-tone RU 1 + 484-tone RU 4; [RU996 (-) RU484] |
|  | MRU 4 | 996-tone RU 1 + 484-tone RU 3; [RU996 RU484 (-)] |
| 996+484+242- tone MRU (only for non- OFDMA) | MRU 1 | 996-tone RU 2 + 484-tone RU 2 + 242-tone RU 2; [(-) RU242 RU484 RU996] |
|  | MRU 2 | 996-tone RU 2 + 484-tone RU 2 + 242-tone RU 1; [RU242 (-) RU484 RU996] |
|  | MRU 3 | 996-tone RU 2 + 484-tone RU 1 + 242-tone RU 4; [RU484 (-) RU242 RU996] |
|  | MRU 4 | 996-tone RU 2 + 484-tone RU 1 + 242-tone RU 3; [RU484 (-) RU996] |
|  | MRU 5 | 996-tone RU 1 + 484-tone RU 4 + 242-tone RU 6; [RU996 (-) RU242 RU484] |
|  | MRU 6 | 996-tone RU 1 + 484-tone RU 4 + 242-tone RU 5; [RU996 RU242 (-)RU484] |
|  | MRU 7 | 996-tone RU 1 + 484-tone RU 3 + 242-tone RU 8; [RU996 RU484 (-) RU242] |
|  | MRU 8 | 996-tone RU 1 + 484-tone RU 3 + 242-tone RU 7; [RU996 RU484 RU242 (-)] |
| NOTE 1—In non-OFDMA transmission, (-) indicates that the subchannel corresponding to (-) is punctured and is to help indicate the frequency order of the MRU in an 80/160/320 MHz PPDU. NOTE 2—In OFDMA transmission, (-) indicates that the subchannel corresponding to (-) is punctured or unassigned or used for data transmission by assigning one or multiple RU/MRU and is to help indicate the frequency order of the MRU within an 80/160/240/320 MHz subband. |  |  |

Table 36-15—Indices for large size MRUs in an 320 MHz EHT PPDU and in a non-OFDMA 320 MHz EHT PPDU(#2398)

| MRU type | MRU index | Combinations |
| --- | --- | --- |
| 484+242-tone MRU | MRU 1 | 484-tone RU 2 + 242-tone RU 2; [(-) RU242 RU484] in lower 80 MHz channel in lower 160 MHz |
|  | MRU 2 | 484-tone RU 2 + 242-tone RU 1; [RU242 (-) RU484] in lower 80 MHz channel in lower 160 MHz |
|  | MRU 3 | 484-tone RU 1 + 242-tone RU 4; [RU484 (-) RU242] in lower 80 MHz channel in lower 160 MHz |
|  | MRU 4 | 484-tone RU 1 + 242-tone RU 3; [RU484 RU242 (-)] in lower 80 MHz channel in lower 160 MHz |
|  | MRU 5 | 484-tone RU 4 + 242-tone RU 6; [(-) RU242 RU484] in upper 80 MHz channel in lower 160 MHz |
|  | MRU 6 | 484-tone RU 4 + 242-tone RU 5; [RU242 (-) RU484] in upper 80 MHz channel in lower 160 MHz |
|  | MRU 7 | 484-tone RU 3 + 242-tone RU 8; [RU484 (-) RU242] in upper 80 MHz channel in lower 160 MHz |
|  | MRU 8 | 484-tone RU 3 + 242-tone RU 7; [RU484 RU242 (-)] in upper 80 MHz channel in lower 160 MHz |
|  | MRU 9 | 484-tone RU 6 + 242-tone RU 10; [(-) RU242 RU484] in lower 80 MHz channel in upper 160 MHz |
|  | MRU 10 | 484-tone RU 6 + 242-tone RU 9; [RU242 (-) RU484] in lower 80 MHz channel in upper 160 MHz |
|  | MRU 11 | 484-tone RU 5 + 242-tone RU 12; [RU484 (-) RU242] in lower 80 MHz channel in upper 160 MHz |
|  | MRU 12 | 484-tone RU 5 + 242-tone RU 11; [RU484 RU242 (-)] in lower 80 MHz channel in upper 160 MHz |
|  | MRU 13 | 484-tone RU 8 + 242-tone RU 14; [(-) RU242 RU484] in upper 80 MHz channel in upper 160 MHz |
|  | MRU 14 | 484-tone RU 8 + 242-tone RU 13; [RU242 (-) RU484] in upper 80 MHz channel in upper 160 MHz |
|  | MRU 15 | 484-tone RU 7 + 242-tone RU 16; [RU484 (-) RU242] in upper 80 MHz channel in upper 160 MHz |
|  | MRU 16 | 484-tone RU 7 + 242-tone RU 15; [RU484 RU242 (-)] in upper 80 MHz channel in upper 160 MHz |
| 996+484-tone MRU | MRU 1 | 996-tone RU 2 + 484-tone RU 2; [(-) RU484 RU996] in lower 160 MHz |
|  | MRU 2 | 996-tone RU 2 + 484-tone RU 1; [RU484 (-) RU996] in lower 160 MHz |
|  | MRU 3 | 996-tone RU 1 + 484-tone RU 4; [RU996 (-) RU484] in lower 160 MHz |

Table 36-15—Indices for large size MRUs in an 320 MHz EHT PPDU and in a non-OFDMA 320 MHz EHT PPDU(#2398) (continued)

| MRU type | MRU index | Combinations |
| --- | --- | --- |
|  | MRU 4 | 996-tone RU 1 + 484-tone RU 3; [RU996 RU484 (-)] in lower 160 MHz |
|  | MRU 5 | 996-tone RU 4 + 484-tone RU 6; [(-) RU484 RU996] in upper 160 MHz |
|  | MRU 6 | 996-tone RU 4 + 484-tone RU 5; [RU484 (-) RU996] in upper 160 MHz |
|  | MRU 7 | 996-tone RU 3 + 484-tone RU 8; [RU996 (-) RU484] in upper 160 MHz |
|  | MRU 8 | 996-tone RU 3 + 484-tone RU 7; [RU996 RU484 (-)] in upper 160 MHz |

Table 36-15—Indices for large size MRUs in an 320 MHz EHT PPDU and in a non-OFDMA 320 MHz EHT PPDU(#2398) (continued)

| MRU type | MRU index | Combinations |
| --- | --- | --- |
| 2×996+484-tone MRU | MRU 1 | 996-tone RU 2 + 996-tone RU 3 + 484-tone RU 2; [(-) RU484 RU996 RU996 (-)] |
|  | MRU 2 | 996-tone RU 2 + 996-tone RU 3 + 484-tone RU 1; [RU484 (-) RU996 RU996 (-)] |
|  | MRU 3 | 996-tone RU 1 + 996-tone RU 3 + 484-tone RU 4; [RU996 (-) RU484 RU996 (-)] |
|  | MRU 4 | 996-tone RU 1 + 996-tone RU 3 + 484-tone RU 3; [RU996 RU484 (-)RU996 (-)] |
|  | MRU 5 | 996-tone RU 1 + 996-tone RU 2 + 484-tone RU 6; [RU996 RU996 (-) RU484 (-)] |
|  | MRU 6 | 996-tone RU 1 + 996-tone RU 2 + 484-tone RU 5; [RU996 RU996 RU484 (-) (-)] |
|  | MRU 7 | 996-tone RU 3 + 996-tone RU 4 + 484-tone RU 4; [(-) (-) RU484 RU996 RU996] |
|  | MRU 8 | 996-tone RU 3 + 996-tone RU 4 + 484-tone RU 3; [(-) RU484 (-) RU996 RU996] |
|  | MRU 9 | 996-tone RU 2 + 996-tone RU 4 + 484-tone RU 6; [(-) RU996 (-) RU484 RU996] |
|  | MRU 10 | 996-tone RU 2 + 996-tone RU 4 + 484-tone RU 5; [(-) RU996 RU484 (-) RU996] |
|  | MRU 11 | 996-tone RU 2 + 996-tone RU 3 + 484-tone RU 8; [(-) RU996 RU996 (-) RU484] |
|  | MRU 12 | 996-tone RU 2 + 996-tone RU 3 + 484-tone RU 7; [(-) RU996 RU996 RU484 (-)] |
| 3×996-tone MRU | MRU 1 | 996-tone RU 2 + 996-tone RU 3 + 996-tone RU 4; [(-) RU996 RU996 RU996] |
|  | MRU 2 | 996-tone RU 1 + 996-tone RU 3 + 996-tone RU 4; [RU996 (-) RU996 RU996] |
|  | MRU 3 | 996-tone RU 1 + 996-tone RU 2 + 996-tone RU 4; [RU996 RU996 (-) RU996] |
|  | MRU 4 | 996-tone RU 1 + 996-tone RU 2 + 996-tone RU 3; [RU996 RU996 RU996 (-)] |

Table 36-15—Indices for large size MRUs in an 320 MHz EHT PPDU and in a non-OFDMA 320 MHz EHT PPDU(#2398) (continued)

| MRU type | MRU index | Combinations |
| --- | --- | --- |
| 3×996+484-tone MRU | MRU 1 | 996-tone RU 2 + 996-tone RU 3 + 996-tone RU 4 + 484-tone RU 2; [(-) RU484 RU996 RU996 RU996] |
|  | MRU 2 | 996-tone RU 2 + 996-tone RU 3 + 996-tone RU 4 + 484-tone RU 1; [RU484 (-) RU996 RU996 RU996] |
|  | MRU 3 | 996-tone RU 1 + 996-tone RU 3 + 996-tone RU 4 + 484-tone RU 4; [RU996 (-) RU484 RU996 RU996] |
|  | MRU 4 | 996-tone RU 1 + 996-tone RU 3 + 996-tone RU 4 + 484-tone RU 3; [RU996 RU484 (-) RU996 RU996] |
|  | MRU 5 | 996-tone RU 1 + 996-tone RU 2 + 996-tone RU 4 + 484-tone RU 6; [RU996 RU996 (-) RU484 RU996] |
|  | MRU 6 | 996-tone RU 1 + 996-tone RU 2 + 996-tone RU 4 + 484-tone RU 5; [RU996 RU996 RU484 (-) RU996] |
|  | MRU 7 | 996-tone RU 1 + 996-tone RU 2 + 996-tone RU 3 + 484-tone RU 8; [RU996 RU996 RU996 (-) RU484] |
|  | MRU 8 | 996-tone RU 1 + 996-tone RU 2 + 996-tone RU 3 + 484-tone RU 7; [RU996 RU996 RU996 RU484 (-)] |
| NOTE 1—In non-OFDMA transmission, (-) indicates that the subchannel corresponding to (-) is punctured and is to help indicate the frequency order of the MRU in an 80/160/320 MHz PPDU. NOTE 2—In OFDMA transmission, (-) indicates that the subchannel corresponding to (-) is punctured or unassigned or used for data transmission by assigning one or multiple RU/MRU and is to help indicate the frequency order of the MRU within an 80/160/240/320 MHz subband.. |  |  |

## CID 5812, 6787, 7151

| CID | Clause Number | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5812 | 36.3.2.2.3.2 | 359.31 | For easy of understanding, it is better to delete "only within each of the two 160 MHz channels occupied by the 320 MHz transmission" from "For OFDMA transmission in 320 MHz, the allowed combinations for a 996+484-tone MRU in OFDMA 160 MHz EHT PPDU are allowed only within each of the two 160 MHz channels occupied by the 320 MHz transmission only within the primary 160 MHz channel or secondary 160 MHz channel" | as in the comment | Revised Instructions to the editor: Please make the changes as shown in 11/22-0383r1 |
| 6787 | 36.3.2.2.3.2 | 359.32 | Fix typo/redundant information: "For OFDMA transmission in 320 MHz, the allowed combinations for a 996+484-tone MRU in OFDMA 160 MHz EHT PPDU are allowed only within each of the two 160 MHz channels occupied by the 320 MHz transmission only within the primary 160 MHz channel or secondary 160 MHz channel". | As in comment | Revised Already revised by CID 5812. Note to the editor: No further changes are needed. |
| 7151 | 36.3.2.2.3.2 | 359.30 | Delete redundant wording "only within each of the two 160 MHz channels occupied by the 320 MHz transmission" | Change "only within each of the two 160 MHz channels occupied by the 320 MHz transmission only within the primary 160 MHz channel or secondary 160 MHz channel" to "only within the primary 160 MHz channel or secondary 160 MHz channel" | Revised Already revised by CID 5812. Note to the editor: No further changes are needed. |

Proposed Changes:

Modify P506 L29-L32 of D1.4 as follows: (The baseline for this text is 802.11be D1.4.)

(#2605)For OFDMA transmission in 320 MHz, the allowed combinations for a 996+484-tone MRU in OFDMA 160 MHz EHT PPDU are allowed only within the primary 160 MHz channel or secondary 160 MHz channel(#1300).

## CID 6428, 6429, 6430

| CID | Clause Number | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 6428 | 36.3.2.2 | 355.21 | For consistency, change "multiple RUs" to "MRUs" | As in comment | Accepted Instructions to the editor: Corresponding page and line numbers in 802.11be D1.4 are P502 L21. |
| 6429 | 36.3.2.2 | 358.33 | For consistency, change "multiple RUs" to "MRUs" | As in comment | Accepted Instructions to the editor: Corresponding page and line numbers in 802.11be D1.4 are P505 L35. |
| 6430 | 36.3.2.2 | 361.35 | For consistency, change "multiple RUs" to "MRUs" | As in comment | Accepted Instructions to the editor: Corresponding page and line numbers in 802.11be D1.4 are P508 L36. |

## CID 6785, 6786

| CID | Clause Number | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 6785 | 36.3.2.2.3.2 | 358.40 | It's good to clarify up-front that 484+242 MRU is allowed only within an 80MHz subblock. In the current text, one has to read the description of OFDMA transmission in 160MHz and 320MHz to infer this fact. | Edit as: "The 484+242-tone MRU is obtained by combining a 484-tone RU and a 242-tone RU within an 80 MHz frequency subblock". | Accepted Instructions to the editor: Corresponding page and line numbers in 802.11be D1.4 are P505 L43. |
| 6786 | 36.3.2.2.3.2 | 359.07 | It's good to clarify up-front that 996+484 MRU is allowed only within a 160MHz channel. In the current text, one has to read the description of OFDMA transmission in 320MHz to infer this fact. | Edit as: "The 996+484-tone MRU is obtained by combining a 996-tone RU and a 484-tone RU in adjacent 80 MHz frequency subblocks of a 160 MHz channel." | Accepted Instructions to the editor: Corresponding page and line numbers in 802.11be D1.4 are P506 L7. |

## CID 6788

| CID | Clause Number | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 6788 | 36.3.2.2.3.2 | 361.28 | The mandatory support for L-MRU in OFDMA for non-AP STA is also dependent on the MRU location relative to the STA's operating bandwidth, and not just the MRU size | Edit as: "It is mandatory for a non-AP STA to support the transmission and reception of 484+242-tone MRU in each 80 MHz subblock, 996+484-tone MRU in the primary 160 MHz channel and the secondary 160 MHz channel, 2×996+484-tone MRU, 3×996-tone MRU, and 3×996+484-tone MRU in 320 MHz channel for an OFDMA 80/160/320 MHz EHT PPDU unless the MRU size is larger than its supported bandwidth provided the entire MRU is located within the non-AP STA's operating bandwidth." | Accepted Agree with the commenter for clarity. Instructions to the editor: Corresponding page and line numbers in 802.11be D1.4 are P508 L29-L34. |

## CID 4674, 7144, 7145, 7146, 7147, 7149, 7152

| CID | Clause Number | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 4675 | 36.3.2.2.3.3 | 361.56 | MRU names are non-unique and then relatively opaque (e,g, see P439L19, which refers to MRU-2, but which MRU-2?) | For each of small and large size RUs, add a convention like MRU_{80MHz}^{484+242}-1 so that we have uniique names when we need them. Use these unique names in Table 36-35 etc. | Rejected Each of MRUs is discriminated by the size of MRUs in spec like “484+242-tone” MRU-2 and “996+484-tone” MRU-2. So, each of MRUs already has a unique name. |
| 7144 | 36.3.2.2.3.1 | 355.28 | Move sentence "The 484+242-tone MRU is allowed in a non-OFDMA 80 MHz EHT PPDU." to end of paragraph (i.e. definition first, description of usage next). Make similar change for other MRU sizes. | See comment | Rejected I don't think there is any problem in understanding the large size MRUs for OFDMA with the current paragraph. |
| 7145 | 36.3.2.2.3.1 | 355.28 | MRU is defined here as "The 484+242-tone MRU is obtained by puncturing any one of the four 242-tone RUs in the 80 MHz EHT PPDU.". Note that in 36.3.2.2.3.2 the same MRU is defined as "The 484+242-tone MRU is obtained by combining a 484-tone RU and a 242-tone RU.". The latter is actually a better definiton given the fundamental definiton of an MRU as a combination of RUs. No need to bring the loosely defined term "puncturing" into this. | Make definition consistent and replace "The 484+242-tone MRU is obtained by puncturing any one of the four 242-tone RUs in the 80 MHz EHT PPDU." with "The 484+242-tone MRU is obtained by combining a 484-tone RU and a 242-tone RU". Similar changes needed in multiple places for MRUs of other sizes as well. | Rejected In non-OFDMA, MRU is used to transmit EHT PPDU when one or more 20MHz subchannels are puncured. So, keeping the sentence looks better in order to emphasize this. |
| 7146 | 36.3.2.2.3.2 | 358.40 | Change "combining a 484-tone RU and a 242-tone RU" with "combining any non-overlapping 484-tone RU and 242-tone RU" | See comment | Rejected From Figure 36-17 (Allowed 484+242-tone MRUs in an OFDMA 80 MHz EHT PPDU), we can understand that the 484+242-tone MRU is obtained by any non-overlapping 484-tone RU and 242-tone RU. So, I think that we don’t need to change the sentence. |
| 7147 | 36.3.2.2.3.2 | 358.39 | Move sentence "The 484+242-tone MRU is allowed in a OFDMA 80 MHz, 160 MHz and 320 MHZ EHT PPDU." to end of paragraph (i.e. definition first, description of usage next). Make similar change for other MRU sizes. | See comment | Rejected I don't think there is any problem in understanding the large size MRUs for OFDMA with the current paragraph. |
| 7149 | 36.3.2.2.3.2 | 358.60 | OFDMA allows 484+242 also in 160 MHz and 320 MHz. There are no figures for this. | Either include figures or add sentence stating that Figure 36-11 is repeated in each 80 MHz subblock. | Rejected The sectence of “For OFDMA transmission in 160 MHz and 320 MHz, the allowed combinations for a 484+242-tone MRU in an OFDMA 80 MHz EHT PPDU are allowed in each 80 MHz frequency subblock of OFDMA 160 MHz and 320 MHz EHT PPDU.” in spe describes this. |
| 7152 | 36.3.2.2.3.2 | 359.34 | Change "The 2×996+484-tone MRU is obtained by combining two 996-tone RUs and 484-tone RU." to "The 2×996+484-tone MRU is obtained by combining two 996-tone RUs and 484-tone RU that fall within the same 240 MHz frequency span" | See comment | Rejected From Figure 36-19 (Allowed 2×996+484-tone MRUs in an OFDMA 320 MHz EHT PPDU), we can understand that the 2×996+484-tone MRU is obtained by combining two 996-tone RUs and 484-tone RU that fall within the same 240 MHz frequency span. So, I think that we don’t need to change the sentence. |

## CID 7156

| CID | Clause Number | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 7156 | 36.3.2.2.3.3 | 361.42 | After "Table 36-15", add "for 80, 160 and 320 MHz respectively". (Note that Table captions in references are not included in the published version) | See comment | Accepted Instructions to the editor: Corresponding page and line numbers in 802.11be D1.4 are P508 L42. |

## CID 6129, 7142, 7756

| CID | Clause Number | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 6129 | 36.3.2.2 | 0.00 | What is the difference between puncturing and MRU? For example, if S20 is busy in an 80M channel, what is the difference between doing a transmission where S20 is punctured out, and doing an MRU transmission consisting of an RU on the P20 and an RU on the S40? | Clarify | Rejected MRU is a resource unit to handle a puncturing case as well as other resource allocation cases. There is no transmission using puncturing. |
| 7142 | 36.3.2.2.2 | 351.07 | How are The "Indices for small size MRUs" related to the PHY MRU index in Table 9-29j1? | Clarify | Rejected RU and MRU indices in the Table 9-29j1 to the PHY MRU indices are already specified in the P137 L1-22 of D1.4. |
| 7756 | 36.3.2.2.2 | 345.61 | The subclause only describes the Small size MRU to be used in OFDMA PPDU, and the OFDMA PPDU is defined as for more than one user. It should also be benefitial to allow SU transmission with Small size MRU | Please explicitly clarify the case that Small size RU can be used in non-OFDMA PPDU | Rejected SU transmission using small size MRU will result in spectral efficiency loss. No benefit from it. |