IEEE P802.11
Wireless LANs

| CC36 CR on CID 5497 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2022.03.04 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ross Jian Yu | Huawei Technologies | H3, Huawei Base, Bantian, Longgang, Shenzhen, Guangdong, China, 518129 |  | ross.yujian@huawei.com |
| Ming Gan |  |  |  |  |
| Stephen McCann |  |  |  |  |
| Edward Au |  |  |  |  |

Abstract

This submission contains proposed comment resolutions to comments on P802.11be D1.0. The changes are based on P802.11be D1.4.

1 CID is resolved: CID 5497

# Revision Notes

| R0 | Initial revision |
| --- | --- |

## CID 5497

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 0.00 | 35.x | OBSS PD SR has similar issues as PSR based SR. Need to resolve related isssues | Will bring a detailed proposal | REVISED Agreed in principle. The issue has been resolved in 11-22/0083r2. Note to the editor: no further action is needed. |

Discussion:

In 11ax, the received signal strength is defined to be measured within the whole bandwidth, which is not applicable for many cases. For example, the Rx’s RF capability (L-preamble reception capability) is smaller than the received PPDU’s bandwidth, or the Rx doesn’t have complete knowledge of the puncture info of the received PPDU. On the other hand, OBSS_DP needs to be normalized because the received signal strength is defined within the whole bandwidth.

To resolve the above issues, the intention is to define everything in dBm/20MHz. The receiver can choose either one or multiple of 20MHz subchannels. On which one or ones are implementation specific.

Spec text from 11ax: