IEEE P802.11
Wireless LANs

| Proposed 802.11bp Functional Requirements |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2024-07-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Bin Qian | Huawei Technologies | F3, Huawei Base, Bantian, Longgang District, Shenzhen, China | +8618476397450 | qianbin14@huawei.com |
| Lei Huang |  |  |  |  |

### Abstract

This submission proposes the 802.11bp Functional Requirements as derived from the 802.11bp PAR Error: Reference source not found and CSD [Ref-1].

-Rev 0: initial draft

- Introduction

  - Purpose

This document proposes requirements for solutions addressing functionality to be provided by the 802.11bp amendment, referred to as the TGbp Functional Requirements (FRs).

  - Scope

The 802.11bp amendment shall comply with the PAR Error: Reference source not found and the CSD [Ref-2]. The functional requirements as stated in this document cover the following aspects of 802.11bp:

- Data Communication

- Wireless Power Transfer

- Coexistence

  - Notation

Requirements are identified by a preceding unique number in the format of “TGbp Rn”, where n is an integer number representing the ID of the requirements.

- Functional Requirements

  - Data Communication

- The 802.11bp amendment shall define at least one mode of data communication in the sub-1 GHz and at least one mode of data communication in the 2.4 GHz band with the AMP communication access category (AC) being set to AC-BK (background).

  - Wireless Power Transfer

- The 802.11bp amendment shall define at least one mode of wireless power transfer in the sub-1 GHz band.

  - Coexistence

- The 802.11bp amendment shall define mechanisms for the coexistence of an AMP STA and deployed STAs compliant with IEEE Std 802.11™-2020 that operate in the same radio frequency band as the AMP STA

Note: The TGbp R3 will revisit the coexistence with published and in development 802.11 amendments

- References

- 11-24-0575-01-0amp-p802-11bp-par

- 11-23-1212-04-0amp-ieee-802-11-amp-sg-proposed-csd