IEEE P802.11
Wireless LANs

| Follow-up on proposed IEEE 802.11 Automotive TIG Technical Report Text for the Regional HD map updates use case |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2025-05-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Jing Ma | Toyota Motor Corporation | 1-6-1 Otemachi, Chiyoda-ku, Tokyo, 100-0004, Japan |  | jing.ma@toyota.global |

Abstract

This document contains the proposed technical report text of the IEEE 802.11 Automotive TIG, especially for the regional HD map updates use case.

Revision history:

r0: the initial version

r1: Modified the use-case description in Section 2.1 and the requirements in Section 3.1

# Table of Contents

- Introduction

  - Terminologies

  - Background information

- Automotive Use cases for IEEE 802.11

Note: use cases potentially can identify KPIs

  - Use case 1: Regional HD MAP updates using IEEE802.11 WLAN

    - Use case description

Modern vehicles rely on High Definition (HD) map data for safe and autonomous operation. In this use case, it is clarified that the vehicle functions as a non-AP station (non-AP STA) to opportunistically connect to external IEEE802.11 WLAN access points (APs) for sharing sensor data, receiving map updates, reporting discrepancies, and downloading region-specific map data. This enhances map accuracy, utilizes local infrastructure for efficient data exchange, reduces cellular data costs, and creates a collaborative ecosystem where each vehicle contributes to overall map improvement. In addition, edge servers connected to APs can locally cache and serve HD map tiles directly over the WLAN, ensure low-latency delivery, and handle update processing at the network edge. The use case comprises two primary scenarios:

Scenario 1: Regional HD Map Element Downloads and Sensor Data Upload in stationary or near-stationary

In this scenario, non-AP STA (vehicle) download the HD map data element data or upload the collect sensor data by opportunistically connecting to APs, typically deployed at parking areas, service stations, or charging station, dealer shop along the vehicles’ planned route.

Substantial data volumes (~dozens of GB) transfer with high reliability needs but flexible timing, for example[1-5]:

- Full-region map refresh: download ~8GB over a 15-minute dwell

- On-demand segment pull: download ~300MB for the next 20km roadway when stop

- time is limited to 3-5 minutes

- Extensive sensor data sharing: upload ~5GB sensor data including LiDAR point clouds,

camera frames, IMU logs, and so on, when stop time is limited to several minutes.

Data transfers should support resumable, integrity-checked sessions which may be covered by upper layer such as HTTP byte-range request or application-layer checkpointing, so that if a connection drops, the non-AP STA (vehicle) can resume exactly where is left off.

Upon non-AP STA (vehicle) arrival and during stationary or near-stationary (≤ 10 km/h) conditions, the non-AP STA (vehicle) obtains an opportunistic WLAN connection window, typically lasting several to dozens of minutes, during which it can download regional HD-map segments and upload its collected sensor dataset. However, RF impairments (mobility-induced fading, transient link drops), dense-deployment interference and congestion, and contention during network discovery and association significantly erode usable transfer time. Moreover, in this scenario, multiple non-AP STAs (vehicle) may download common map elements. Delivering identical map elements via individual unicast sessions exacerbates redundancy and congestion in high-density scenarios (e.g. busy parking facilities). To maximize offload efficiency, the system must minimize discovery and connect-setup latency, support robust session resumption for large transfers, and enable group-oriented delivery mechanisms that reduce redundant traffic.

Senario 2: Local Dynamic Map Updates when drive-thru

In this senario, Non-AP STA(vehicle) travelling at moderate speed (e.g. ≤ 60km/h) passes through roadside APs’ coverage cells, yielding one or successive short “drive-thru” connection windows typically on the order of 10-20 seconds per AP. During each brief interval, the Non-AP STA(vehicle) need to opportunistically exchange low-latency map and telemetry data with the cloud or edge server via the WLAN connectivity

Dynamic map updates in this scenario involve [2-5]

- Downloading moderate-sized data objects (e.g. 50 MB)—for example, real-time traffic

- overlays, temporary road-condition warnings or signaling information—with latency requirements in the sub-second to several-second range.

- Uploading compact event reports (e.g. 10 KB each) reflecting detected dynamic objects

(e.g. unexpected obstacles, lane-closure alerts), which similarly demand end-to-end latencies of a few seconds at most to remain actionable.

Effective drive-thru data offload faces several key challenges: mobility-induced rapid signal fading and transient drops create highly variable link quality and force frequent retransmissions; each roadside-AP encounter lasts only 10~20s, imposing a stringent cap on transferable data volumes; network scanning, channel switching, and 802.11 association/authentication procedures introduce overhead relative to these brief windows; full or partial re-authentication and roaming between intra- and inter-ESS cells further erode usable time; and dense urban deployments with multiple APs and co-channel users produce contention and interference that degrade effective throughput.

  - Use case 2

  - Use case N

- Requirements and Potential features analysis (high level)

  - Requirements

The automotive use cases described in section 2, such as regional HD Map updates, highlights WLAN’s critical role in supporting automotive applications. This section defines the key requirements for vehicles leverage opportunistic WLAN to offload vecular data, whether stationary/near-stationary or drive-thru urban:

    - Fast network discovery & connection:

Rapid identification and establishment of WLAN connections are crucial to optimize data transfer opportunities, whether in extended dwell times when vehicle parked or brief connection windows when vehicle drive-thru. This requires mechanisms to prioritize scanning and conduct parallel searches across multiple channels, reducing discovery overhead in dense deployments with numerous vehicles.

Off-channel neighbor detection can accelerate the identification of available access points, while streamlined authentication processes that minimize message exchanges, enabling simultaneous association and authentication within a single round-trip, ensure connection setup completes within stringent timeframes (e.g., under 500ms in congested settings or under 100ms in high-mobility scenarios). Moreover, predictive selection algorithms further adapt to rapidly changing signal conditions, improving connection reliability.

These capabilities are primarily driven by lower-layer optimizations at the MAC/PHY levels to minimize channel-switching delays and authentication overhead, though upper-layer coordination may be necessary to standardize behavior across diverse devices and ensure consistent performance in varied environments.

    - Fast and

    - seamless roaming

Reliable transitions between APs or across networks (e.g. inter/intra-ESS and WLAN-to-cellular) are essential for continuous connectivity across different mobility conditions.

Fast re-authentication and credential pre-loading enable seamless switches with minimal interruption (e.g., under 200ms). Group key management avoids rekey delays during handoffs, while routing optimizations like tunneling maintain IP continuity, particularly during inter-ESS transitions where delays may be reduced to under 2 seconds.

Enhanced rate adjustments at the PHY layer adapt to signal fluctuations and doppler effects, ensuring stable connectivity. Recovery from failed handoffs may rely on upper-layer fallback mechanisms to maintain robustness, complementing lower-layer techniques that drive efficient roaming.

    - Efficent data transfer

Efficient data transfer over opportunistic WLAN connectivity should satisfy stringent requirements to handle both extended bulk data offloads and short update data exchanges within variable connectivity windows.

Support for parallel aggregation of multiple channels via multi-link operation maximizes air-time capacity, while frame aggregation with EDCA-based QoS ensures that sub-second map updates or event reports can preempt lower-priority bulk transfers.

In dense urban deployments, spatial reuse and BSS coloring enable concurrent transmissions on underutilized resource units, and where available, mmWave links may be leveraged as ephemeral, ultra-high-rate paths for short-range data bursts.

To avoid redundant traffic when multiple vehicles request identical geographic data, broadcast or multicast delivery mechanisms are required.

- Furthermore, dynamic transmit-power control, adaptive channel assignment, and coordinated AP scheduling stabilize link quality under mobility and contention, and client-side airtime management guarantees timely delivery of small telemetry packets amidst competing bulk flows.

- GAP Analysis

While IEEE 802.11 standards continue to evolve, several key gaps remain for automotive applications, particularly for vehicles in motion[12, 13]:

Optimized Service Discovery for Automotive Services: IEEE 802.11u [6] provides GAS/ANQP discovery procedures, but lacks faster network discovery and selection optimized for brief vehicular connection windows.

Fast Connection Establishment for Automotive Use Cases: IEEE 802.11ai [7] introduced FILS, but links remain unstable on the move, causing delays in setup and failures during handover for vehicular scenarios

Mobility Support at Automotive Speeds: Existing handover standards are designed for pedestrian mobility, not vehicles at city speeds (30-60 km/h). Enhanced procedures are needed to handle rapid AP transitions and changing RF conditions experienced during vehicular movement.

Inter-ESS Roaming Support: While IEEE 802.11r (Fast BSS Transition) [8], IEEE 802.11k (Radio Resource Measurement) [9], and IEEE 802.11bn (seamless roaming) [10] standards supports intra-ESS roaming with smooth handover mechanisms, inter-ESS roaming between different operator networks remains challenging. Addressing this requires both protocol enhancements and industry-wide coordination agreements.

Efficient multi-vehicle support: While IEEE 802.11bc (Enhanced Broadcast Service) [11] offers improvements for group communication, additional enhancements to broadcast/multicast capabilities are needed to efficiently support numerous vehicles that are both in high density and in motion, particularly in dynamic traffic scenarios like congested urban areas where many vehicles simultaneously require connectivity.

- Summary and Recommendations

- References

- AFDC, “EV Charging Stations,”https://afdc.energy.gov/fuels/electricity-stations

- AECC, Operational Behavior of a High-Definition Map Application White Papers, May 26, 2020

- ETSI TR 102 863, Intelligent Transport Systems; Vehicular Communications; Basic Set of

Applications; Local Dynamic Map, 2011

- 5AGG, TR, C-V2X Use Cases and Service Level Requirements volume 1, 2023

- 5AGG, TR, C-V2X Use Cases and Service Level Requirements volume 2, 2023

- 5AGG, TR, C-V2X Use Cases and Service Level Requirements volume 3, 2023

- IEEE 802.11u-2011, Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer

(PHY) Specifications - Interworking with External Networks.

- IEEE 802.11ai-2016, Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer

(PHY) Specifications - Fast Initial Link Setup (FILS).

- IEEE 802.11r-2008, Part 11: Wireless LAN Medium Access Control (MAC) and Physical

Layer (PHY) Specifications - Fast BSS Transition.

- IEEE 802.11k-2008, Part 11: Wireless LAN Medium Access Control (MAC) and Physical

Layer (PHY) Specifications - Radio Resource Measurement.

- IEEE 802.11bn (Draft), Seamless Roaming Enhancements.

- IEEE 802.11bc-2024, Enhanced Broadcast Service.

- 802.11-24/1526/r1, Automotive WLAN use case study

- 802.11-25/69/r1, Follow-up of Automotive WLAN use case study