| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Supplementary Channel Configuration in talk-around direct communication |  |
| Date Submitted | 2012-05-04 |  |
| Source(s) | Sungcheol Chang, Seokki Kim, Miyoung Yun, Won-Ik Kim, Hyun Lee, Kim Eunkyung Kim, Sungkyung Kim, Chulsik Yoon ETRI | E-mail: scchang@etri.re.kr |
| Re: | Letter Ballot #38a on P802.16.1a/D2 |  |
| Abstract | This provides text proposals for supplementary channel configuration and distributed resource allocation procedures using RTS/CTS messages in talk-around direct communications |  |
| Purpose | To be discussed and adopted by 802.16 GRIDMAN TG |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

# Supplementary Channel Configuration in talk-around direct communication

Sungcheol Chang, Seokki Kim, Miyoung Yun, Won-Ik Kim, Hyun Lee, Kim Eunkyung Kim, Sungkyung Kim, Chulsik Yoon

ETRI

# Introductions

This contribution provides configuration of supplementary channel and how to update supplementary channel as the followings:

- Configuring ranging channel, CQI channel, and feedback channel on supplementary channel during reserving dedicated channel

- Update the configuration of three channels on supplementary channel.

# Proposed Text for P802.16.1a/D2

Note:

The text in BLACK color: the existing text in the 802.16.1a Draft Standard Text

The text in RED color: the removal of existing 802.16.1a Draft Standard Text

The text in BLUE color: the new text added to the 802.16.1a Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

## [Remedy1: Modify the following text in Section 6.2.3.65.37]

6.2.3.65.37 AAI-DC-RTS

An HR-MS transmits an AAI-DC-RTS message to reserve a dedicated channel in a distributed way.

Table 106a – AAI-DC-RTS message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Source DCTID | 24 | Indicates a source HR-MS address |  |
| Destination DCTID or DCGID | 24 | Indicates a destination HR-MS (Group) address. |  |
| Maximum Index of Burst Size | 8 | Indicates a maximum index of burst size that the sending HR-MS suggests the receiving HR-MS to recommend. The receiving HR-MS selects burst size that is less than |  |
| Ranging Period | 4 | Indicate the period that ranging feedback sequences are sent on supplementary channel periodically. In units of DC frames. |  |
| Ranging Offset | 4 | Indicates the number of DC frames from the DC frame at which first data packet is sent to the DC frame at which first ranging feedback sequence is sent |  |
| CQI Period | 4 | Indicate the period that CQI feedback sequences are sent on supplementary channel periodically. In units of DC frames. |  |
| CQI Offset | 4 | Indicates the number of DC frames from the DC frame at which first data packet is sent to the DC frame at which first CQI feedback sequence is sent |  |
| Maximum Number of HARQ Retransmission | 2 | Indicates maximum number of PHY burst retransmission for HARQ operation. 0: HARQ retransmission is disabled 1~3: HARQ retransmission is enabled |  |
| Destination Address Type | 1 | Indicates type of destination address. 0: DCTID 1: DCGID |  |
| Piggyback Message Indicator | 1 | Indicates whether a control message is piggybacked or not 0: no piggyback 1: MAC control message |  |
| Reserved | 4 |  |  |
| MAC Control Message | variable | MAC control messages in Table 1216 except AAI-DC-RTS and AAI-DC-CTS messages. | Present if Piggyback message indicator is set to 1 |

6.2.3.65.38 AAI-DC-CTS

An HR-MS transmits an AAI-DC-CTS message in response to AAI-DC-RTS message.

Table 106b – AAI-DC-CTS message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Source DCTID | 24 | Indicates a source HR-MS address |  |
| Destination DCTID or DCGID | 24 | Indicates a destination HR-MS (Group) address. |  |
| Recommended Index of Burst Size | 8 | Indicates a recommended index of burst size for a dedicated channel |  |
| Ranging Period | 4 | Indicate the period that ranging feedback sequences are sent on supplementary channel periodically. In units of DC frames. |  |
| Ranging Offset | 4 | Indicates the number of DC frames from the DC frame at which first data packet is sent to the DC frame at which first ranging feedback sequence is sent |  |
| CQI Period | 4 | Indicate the period that CQI feedback sequences are sent on supplementary channel periodically. In units of DC frames. |  |
| CQI Offset | 4 | Indicates the number of DC frames from the DC frame at which first data packet is sent to the DC frame at which first CQI feedback sequence is sent |  |
| Maximum Number of HARQ Retransmission | 2 | Indicates maximum number of PHY burst retransmission for HARQ operation |  |
| Piggyback Message Indicator | 1 | Indicates whether a control message is piggybacked or not 0: no piggyback 1: MAC control message |  |
| Reserved | 5 |  |  |
| MAC Control Message | variable | MAC control messages in Table 1216 except AAI-DC-RTS and AAI-DC-CTS messages. | Present if Piggyback message indicator is set to 1 |

## [Remedy2: Insert the following text in the end of Section 6.12.2.3.2.4]

Ranging sequences and CQI sequences are sent periodically so that corresponding resources of supplementary channel are called ranging channel and CQI channel, respectively. The ranging channel is defined with two fields, Ranging Period and Ranging Offset. The CQI channel is defined with two fields, CQI Period and CQI Offset. When resources of supplementary channel are shared by Ranging channel and CQI channel, they are used to carry Ranging sequences. The other resources of supplementary channel except Ranging channel and CQI channel are used to carry feedback sequences and are called feedback channel.

## [Remedy3: Modify the following text in Section 6.2.3]

6.2.3 MAC control messages

Change Table 26 as indicated:

Table 26—MAC control messages

| No. | Functional Areas | Message names | Message description | Security | Connection |
| --- | --- | --- | --- | --- | --- |
| …… | …… | …… | …… | …… | …… |
| TBD | Resource allocation | AAI-DC-CTS | Clear To Send |  | Unicast |
| TBD | MCS ChangeDirect Communication | AAI-DC-MCHGCFG-CMD | MCS Change Command Configuration Command |  | Unicast or multicast |
| …… | …… | …… | …… | …… | …… |

## [Remedy4: Modify the following text in Section 6.2.3.65.39]

        - AAI-DC- MCHGCFG-CMD

An HR-MS transmits an AAI-DC- MCHGCFG-CMD (MCS Change Configuration Command) message to change MCS level of data signals on the dedicated channel and configure supplementary channel.

Table 106c – AAI-DC- MCHGCFG-CMD message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Maximum Recommended index of burst size | 8 | Indicates a maximum recommended index of burst size for a dedicated channel |  |
| Ranging Period | 4 | Indicate the period that ranging feedback sequences are sent on supplementary channel periodically. In units of DC frames. |  |
| Ranging Offset | 4 | Indicates the number of DC frames from the DC frame at which first data packet is sent after this DC frame to the DC frame at which first ranging feedback sequence is sent |  |
| CQI Period | 4 | Indicate the period that CQI feedback sequences are sent on supplementary channel periodically. In units of DC frames. |  |
| CQI Offset | 4 | Indicates the number of DC frames from the DC frame at which first data packet is sent after this DC frame to the DC frame at which first CQI feedback sequence is sent |  |

## [Remedy4: Insert the following text after Section 6.12.2.3.1.10]

6.12.2.3.1.11 Configuration

When an HR-MS needs to change either MCS level on dedicated channel or configuration of supplementary channel, it sends AAI-DC-CFG-CMD message on dedicated channel to one or more HR-MSs.

Figure 242a Configuration procedures for unicast (left) and multicast (right) links

## [Remedy5: Update the texts with the following proposals]

- Replace “6.12.2.3.1.11.1” with “6.12.2.3.1.12.1” in IEEE P802.16.1a/D2.

- Replace “6.12.2.3.1.11.2” with “6.12.2.3.1.12.2” in IEEE P802.16.1a/D2.

- Replace “6.12.2.3.1.11.3” with “6.12.2.3.1.12.3” in IEEE P802.16.1a/D2.

- Replace “6.12.2.3.1.11” with “6.12.2.3.1.12” in IEEE P802.16.1a/D2.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

[1] IEEE P802.16.1a/D2, “WirelessMAN-Advanced Air Interface for Broadband Access Systems -  Draft Amendment: Higher Reliability Networks,” April 2012.

# Title of Contribution

[Authors of Contribution]

[Affiliation of authors, per <http://standards.ieee.org/faqs/affiliationFAQ.html>]

# Template Instructions

This page provides instructions on using the template and submitting a contribution. Before submitting, delete these instructions or replace them with your text to be submitted.

# Document Format

Edit the cover page. Replace the square-bracketed sections with the correct text for your document. Eliminate the brackets. Leave the non-bracketed words on the cover page. Limit the cover page information to one page, shrinking the font if necessary.

Begin the main document with a page break after the cover page. Restate the document title and author name. You should use heading styles that support an outline view of the document.

# Header

The document number <http://ieee802.org/16/submit.html> goes in the upper right of the header. It takes the form “IEEE 802.16-YY-#####-RR-FFFF”, where:

• Every IEEE 802.16 document number begins with “IEEE 802.16-”.

• Following “IEEE 802.”, the remainder of the document number (“16-YY-#####-RR-FFFF”) is the Document Control Number (DCN), used by IEEE Mentor as the basis of the file name.

• “YY” represents the final two digits of the calendar year of original submission, as assigned by the IEEE Mentor documentation system.

• “####” represents the document serial number, as assigned by the IEEE Mentor documentation system. The serial number is uniquely assigned for each calendar year.

• “RR” represents the revision number, as assigned by the IEEE Mentor documentation system. The initial version of the document uses the revision number “00”.

• “FFFF” represents the Mentor “file code” <http://ieee802.org/16/submit.html>. It is selected by the submitter as the most appropriate identifier among the list of  IEEE 802.16 file codes in the IEEE Mentor documentation system.

# Fonts

Use only the fonts “Times” (or “Times New Roman”), “Arial” (or “Helvetica”),  and “Symbol,” since these fonts need not be embedded in PDF files. You may use other fonts only if necessary.

# Document Submission

To contribute a document to the IEEE 802.16 Working Group or a subgroup, create it using this template or another template at <http://ieee802.org/16/submit.html>. (If it cannot be put into one of the available formats, then create at least a cover page using an available template.) Submit the document following the instructions in the Call for Contributions or the IEEE 802.16 Document Submission Instructions <http://ieee802.org/16/submit.html>.