| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarifications on Maintenance of Alternative Path to forwarding HR-MS in 16n Networks |  |
| Date Submitted | 2012-09-19 |  |
| Source(s) | Anseok Lee, Eunkyung Kim, Jaesun Cha, Wooram Shin, Kwangjae Lim, Dongseung Kwon ETRI | E-mail: alee@etri.re.kr |
| Re: | In response to SB000n on P802.16n/D5 |  |
| Abstract | This provides AWD text proposals of clarification on Alternative Path in IEEE 802.16n Network. |  |
| Purpose | To be discussed and adopted by 802.16 GRIDMAN TG |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

Clarifications on Maintenance of Alternative Path to forwarding HR-MS in 16n Networks

Anseok Lee, Eunkyung Kim, Jaesun Cha, Wooram Shin, Kwangjae Lim, Dongseung Kwon

ETRI

# Introductions

This contribution provides clarification on Alternative Path Management in 16n Network.

# Proposed Text for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy 1: Accept the following modifying to Section 16.6.1.5]

16.6.1.5 Alternative path maintenance

An alternative path to a target HR-infrastructure station may be updated or changed with a new MOB_BSHO-REQ message. When a mode-change or mode-release occurs as described in 16.1, new alternative path may be selected.

An alternative path to a target forwarding HR-MS may be updated or changed by either HR-MS or HR-BS according to network conditions such as the forwarding HR-MS is moving out of reach etc.

The serving HR-BS shall schedule periodic ranging code transmission and reception for the HR-MS and target forwarding HR-MS respectively by using HR-RNG-CMD message for alternative path maintenance. The target forwarding HR-MS shall periodically report the measurement result of ranging code from the HR-MS to the serving HR-BS by the HR-RNG-REP message.

The serving HR-BS shall maintain the connectivity between the HR-MS and the target forwarding HR-MS by the received measurement result of ranging code. When the serving HR-BS detects loss of link between the HR-MS and target forwarding HR-MS, the serving HR-BS shall recover the alternative path for the HR-MS by finding another target forwarding HR-MS by the neighbor discovery procedure as in 16.6.1.2.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

[1] IEEE P802.16n/D5, “Air Interface for Broadband Wireless Access Systems Draft Amendment: Higher Reliability Networks”