Features in support of Emergency Services in 802.16

IEEE 802.16-2009

Page 89

The following TLV parameter shall be included in the RNG-REQ message when the MS is attempting to perform reentry, HO, or location update:

Ranging Purpose Indication

The presence of this item in the message indicates the following MS action:

If Bit 0 is set to 1, in combination with a serving BSID, it indicates that the MS is currently attempting to HO or reentry; or, in combination with a Paging Controller ID, indicates that the MS is attempting network reentry from idle mode to the BS.

If Bit 1 is set to 1, it indicates that the MS is initiating the idle mode location update process.

Bit 2: Seamless HO indication. When this bit is set to 1 in combination with other included information elements, it indicates the MS is initiating ranging as part of seamless HO procedure.

Bit 3: Ranging Request for Emergency Call Setup. When this bit is set to 1, it indicates MS action of Emergency Call Process.

Bit 4: MBS update. When this bit is set to 1, the MS is currently attempting to perform location update due to a need to update service flow management encodings for MBS flows.

Bits 5–7: Reserved

Page 489

6.3.27 Emergency Service

Emergency Service is defined as a service that would provide the public with alerts on imminent emergency events, such as earthquake, storm, tidal wave, etc. The alerts would target subscribers in a specific geographical location. The BS shall transmit the Emergency Service compound TLV via the DCD message (see Table 575).

The Emergency Service compound TLV shall contain the CIDs for Emergency Service TLV encoding required for an MS to identify a MAC PDU containing Emergency Service Message (ESM) (see Table 575). The Emergency Service compound TLV may include a “CS type for Emergency Service TLV” (see Table 578). If an MS supports the CS type used for Emergency Service, the MS shall receive and decode the Emergency Service message when there is one. CIDs specified by the “CIDs for Emergency Service TLV” encoding (see Table 578) shall not be assigned to the MS’s connection which is established via DSA transaction. The BS may also broadcast emergency-related helpful alert information (e.g., commercial advertisement and announcements that may be of interest to the public) via ESM. The Emergency Service connection shall use neither header compression nor PHS.

The BS may broadcast ESM(s) either through an MBS permutation zone or through a normal DL zone (e.g., PUSC, FUSC and so on). If the BS decides to broadcast the ESM(s) through the MBS permutation zone, the BS shall transmit MBS_MAP_IE with indication of an ESM existence in the MBS permutation zone (see 8.4.5.3.12). Even if an MS is not monitoring the MBS channel, the MS shall check for and decode at least these two parameters in an MBS_MAP IE (the Macro diversity enhanced and the Existence of Emergency Service Message). If the MS detects the existence of ESM(s) sent through the MBS permutation zone, the MS shall decode the MBS_MAP message in order to identify the MBS data burst in which the MAC PDU containing Emergency Service Message(s) will be transmitted.

Instead of Broadcast Control Pointer IE (see 8.4.5.3.25), an Extended Broadcast Control Pointer IE (see 8.4.5.3.31) may be used to indicate the frame in which ESM(s) are going to be transmitted. When an MS in Idle Mode or Sleep Mode receives the Extended Broadcast Control Pointer IE with Type = 0x0 during its own interval (i.e., Paging Listening Interval in Idle Mode or Listening Window in Sleep Mode), the MS shall wake up in the frame specified by the Extended Broadcast Control Pointer IE and stay awake during the Transmission Duration indicated by the Extended Broadcast Control Pointer IE.

Page 745

When a BS needs to transmit Emergency Service Message in an MBS region, the BS shall transmit an

MBS_MAP_IE() with MBS permutation zone defined = 1 and Existence of Emergency Service Message =

1. If there is MBS_MAP IE in a DL-MAP message, MS shall decode it and check whether Emergency

Service Message(s) will be transmitted or not through an MBS permutation zone. If the MS supporting the

CS type used for ES detects the existence of Emergency Service Message(s) in the MBS region, the MS

shall decode the MBS-MAP message in order to identify the MBS data burst on which MAC PDU

containing Emergency Service Message(s) will be transmitted.

In Table 339 MBS MAP IE

| Existence of Emergency Service Message | 1 | 0: Indicates that there is no Emergency Service Message(s) in MBS region 1: Indicates that there is MBS_DATA_IE for Emergency Service Message in an MBSMAP message |
| --- | --- | --- |

In Table 362—Broadcast Control Pointer IE format

| Broadcast_System_Update_Type | 1 | Shows the type of Broadcast_System_Update 0: For MOB_NBR-ADV Update 1: For Emergency Services Message |
| --- | --- | --- |

In Table 375—Extended Broadcast Control Pointer IE format

| Type | 4 | Indicates the type of message. 0x0 : Emergency Service Message 0x1~0xF : Reserved |
| --- | --- | --- |
| Transmission Frame Offset | 16 | A relative value from the current frame number in which a BS will start to transmit Emergency Service Message. '0' means the current frame in which this MAP IE is transmitted. |
| Transmission Duration | 8 | Indicates the period during which an MS in Idle Mode or Sleep Mode shall keep awake to receive emergency service message(s). The value '0' indicates that the MS keeps awake during only Transmission Frame. Its unit is frame. |

In Table 575—DCD channel encodings

| Emergency Service | 63 | 1 | The Emergency Service is a compound TLV that defines the parameters required for Emergency Service (see Table 578) |
| --- | --- | --- | --- |

In Table 578—Emergency service description

| CIDs for Emergency Service | 63.1 | Length is defined as: (Number of Multicast CIDS) × 2 | One or more Multicast CIDs (see Table 558) used for DL Emergency Services. Emergency Service message shall be transmitted on these connections |
| --- | --- | --- | --- |
| CS type for Emergency Service | 63.2 | 1 | This TLV indicates CS type which is used for Emergency Service. If this field is omitted, MS shall regard the CS type as default CS type. 0. GPCS 1. Packet, IPv4 (default) 2. Packet, IPv6 3. Packet, IEEE802.3/Ethernet 4. Packet, IPv4 over IEEE 802.3/Ethernet 5. Packet, IPv6 over IEEE 802.3/Ethernet 6. ATM 7–255: Reserved |

In Table 582—RNG-REQ message encodings

| Ranging Purpose Indication | 6 | 1 | Bit 0: HO indication (when this bit is set to 1 in combination with other included information elements indicates the MS is currently attempting to HO or network reentry from idle mode to the BS) Bit 1: Location update request (when this bit is set to 1, it indicates MS action of idle mode location update process) Bit 2: Seamless HO indication (when this bit is set to 1 in combination with other included information elements indicates the MS is currently initiating ranging as part of the seamless HO procedure) Bit 3: Ranging Request for Emergency Call Setup (when this bit is set to 1, it indicates MS action of Emergency Call Process) Bit 4: MBS update. When this bit is set to 1, the MS is currently attempting to perform location update due to a need to update service flow management encodings for MBS flows. Bits 5–7: Reserved |
| --- | --- | --- | --- |

In Table 606—Service flow encodings

| 52 | Emergency Indication |
| --- | --- |
| 53 | Regional Emergency Indication |

Page 1314

11.13.41 Priority indication

The value of this TLV, if present, indicates the associated flow is used for emergency purposes defined by local regulations. The exact conditions for when this TLV shall be included and the value that it shall convey are outside the scope of this standard. The behavior of the MS and BS when receiving this TLV is also outside the scope of this standard.

Page 1314

11.13.42 Regional Emergency Indication parameter

The value of this parameter, if present, indicates the associated flow is used for emergency purposes. The emergency indication parameter shall take precedence over any conflicting service flow QoS parameter.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [145/146].52 | 1 | Bit 0: Emergency indication Bits 1–7: Reserved | DSx-REQ, DSx-RSP |

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [145/146].53 | 63 | Bit 0–3: indicates the class of un-scheduled alert. When the bit is set to 1, it indicates the type of alert to be transmitted in this service flow. Bit 0: Priority 0 Bit 1: Priority 1 Bit 2: Priority 2 Bit 3: Priority 3 Bits 4–7: Reserved | The Emergency Service is a compound TLV that defines the parameters required for Emergency Service (see Table 578) |